# 3GPP TSG-RAN WG4 Meeting #112	R4-2411455

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.15.3

Source:	Apple

Title:	On fast SCell activation with EMR

Document for:	Discussion

# Introduction

From this meeting, RAN4 will start the work on R19 fast SCell activation for UE supporting Rel-18 EMR, and the latest WID[1] approved in last RANP#103 meeting includes following scope:

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In this contribution, we will discuss the RRM requirement for fast SCell activation for UE supporting Rel-18 EMR, and RAN4’s plan is to complete this work by end of Dec’ 2024.

# Discussion

EMR in RRC_IDLE/RRC_Inactive mode was introduced in Rel-16 to facilitate UE and network for CA configuration/activation after entering RRC_Connected mode. However, the existing known/unknown SCell condition during SCell configuration/activation does not consider whether UE has valid EMR for the target SCell or not, which may cause unnecessary long delay requirement for the target SCell activation.

The enhancement of EMR in the R18 has been specified as following, UE will only report the valid measurement result back to network when conditions are met.

Both R16 EMR measurement side condition in clause 4.4 and the re-selection measurement side condition in clause 4.2.2 are SSB Ês/Iot>=-4dB. However, in some cases of SCell activation, it was specified SNR condition as >=-2dB for SCell activation. The mismatch of the SNR condition will result into one issue: the measurement ifor EMR during IDLE/Inactive can support fast SCell activation like legacy known case or not?

For example, UE performs multiple cells measurement on different frequencies or RATs, some of them may be performed under good SINR condition but some of them may be performed under poor SINR condition, like cell 1 and cell 2 in the figure 1. However, based on current EMR reporting or existing measurement result report rule, network cannot differentiate which one is more reliable measurement result to support fast Scell activation as known SCell case compared with the others. In previous release, the validity of measurement is mainly limited to check the freshness of the measurement (how long time ago the measurement is performed), that is, valid measurement result may not be the correct indication of known SCell activation.

Figure 1. example for SINR condition mismatch between measurement and SCell activation

One more justification is: different from legacy measurement and activation procedure, the measurement speed in IDLE/Inactive mode might be quite slow and the X second configured by network might be also quite long, and therefore it’s very likely the SINR condition will change in this new scenario (measurement in IDLE/Inactive but known SCell activation in connected). Thus, we are wondering if it’s necessary to define a consistent SINR condition during the whole procedure for known SCell activation requirement (including EMR measurement in IDLE/Inactive and known SCell activation in RRC connected mode), e.g., SINR>=-2dB.

Proposal 1: RAN4 to discuss whether it’s necessary to define a consistent SINR condition during the whole procedure for known SCell activation requirement (including EMR measurement in IDLE/Inactive and known SCell activation in RRC connected mode), e.g., SINR>=-2dB.

Another potential issue to use R18 EMR report as a condition for known SCell activation is about the measurement period in FR1 known SCell activation. In the legacy FR1 SCell activation, it was defined that,

In previous release, such threshold has been discussed based on deactivated SCell measurement period based on proper measurement resource sharing factor in RRC connected mode, however, in IDLE/Inactive mode, the measurement period can be significantly different from the deactivated SCell measurement in RRC connected mode, e.g., DRX cycle is much longer and no CSSF counting for deactivated SCell measurement. We recommend RAN4 to also discuss whether and how to change the measurement period condition of FR1 known SCell activation (i.e., 2400ms in current requirement) for fast SCell activation with EMR.

Proposal 2: RAN4 to discuss whether and how to change the measurement period condition of FR1 known SCell activation (i.e., 2400ms in current requirement) for fast SCell activation with EMR.

When the above issues can be successfully addressed, we think it’s natural to use EMR reporting as a condition to decide known SCell activation after UE entering RRC connected mode. The only thing to clarify is the condition to decide known or unknown, especially for FR1, and in the current requirement it’s defined that,

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown. The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

In principle, we think the current valid measurement report in the known/unknown condition can cover the EMR report besides the legacy L3 report, based on the definition of EMR report case and validation in R18.

Observation:  “a valid measurement report” in the current known/unknown condition of SCell activation can cover the EMR report case.

For time condition to decide FR2 known SCell (i.e., 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4), we think it can be still reused for fast SCell activation with EMR without any change.

Proposal 3: the current side condition and delay requirement for FR2 known SCell activation can cover the fast SCell activation with EMR. Potential clarification can only focus on the issue in proposal 1.

For time condition to decide FR1 known SCell (max(5*measCycleSCell,  5*DRX cycles)), since this is a time relation condition between “the measurement reporting” and “the SCell activation command”, and both of them happen in the RRC connected mode, we didn’t see the strong need to change this condition in the current spec for fast SCell activation with EMR. Moreover, such time condition is mainly to define how long time can hold the T/F and AGC information from the reporting time point.

However, another condition to decide FR1 known SCell is: the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay. For fast SCell activation with EMR, the SSB measurement happens during the IDLE/Inactive period and therefore here the measurement period shall be changed to inter-frequency NR cells measurement period in IDLE/Inactive mode, i.e.,

- measurement period in section 4.4.2.2, if a UE supporting measValidationReportEMR-r18 and configured with measIdleCarrierListNR-r16 by higher layers.

- measurement period in section 4.2.2.4, if UE supporting measValidationReportReselectionMeasurements-r18 and idleInactiveNR-MeasReport-r16.

Proposal 4: the current side condition and delay requirement for FR1 known SCell activation shall be updated to cover the fast SCell activation with EMR, as following:

- The side condition that “the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay” shall be changed to “the SSB measured during the period equal to measurement period in IDLE/Inactive mode for EMR report also remains detectable during the SCell activation delay”, and the “measurement period in IDLE/Inactive mode” refers to:

  - measurement period in section 4.4.2.2, if a UE supporting measValidationReportEMR-r18 and configured with measIdleCarrierListNR-r16 by higher layers.

  - measurement period in section 4.2.2.4, if UE supporting measValidationReportReselectionMeasurements-r18 and idleInactiveNR-MeasReport-r16.

- The update of FR1 known SCell activation delay requirement shall be based on the solution for issue in proposal 2.

# Conclusion

In this contribution, we discuss the RRM requirement for fast SCell activation for UE supporting Rel-18 EMR, and RAN4’s plan is to complete this work by end of Dec’ 2024.

Proposal 1: RAN4 to discuss whether it’s necessary to define a consistent SINR condition during the whole procedure for known SCell activation requirement (including EMR measurement in IDLE/Inactive and known SCell activation in RRC connected mode), e.g., SINR>=-2dB.

Proposal 2: RAN4 to discuss whether and how to change the measurement period condition of FR1 known SCell activation (i.e., 2400ms in current requirement) for fast SCell activation with EMR.

Observation:  “a valid measurement report” in the current known/unknown condition of SCell activation can cover the EMR report case.

Proposal 3: the current side condition and delay requirement for FR2 known SCell activation can cover the fast SCell activation with EMR. Potential clarification can only focus on the issue in proposal 1.

Proposal 4: the current side condition and delay requirement for FR1 known SCell activation shall be updated to cover the fast SCell activation with EMR, as following:

- The side condition that “the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay” shall be changed to “the SSB measured during the period equal to measurement period in IDLE/Inactive mode for EMR report also remains detectable during the SCell activation delay”, and the “measurement period in IDLE/Inactive mode” refers to:

  - measurement period in section 4.4.2.2, if a UE supporting measValidationReportEMR-r18 and configured with measIdleCarrierListNR-r16 by higher layers.

  - measurement period in section 4.2.2.4, if UE supporting measValidationReportReselectionMeasurements-r18 and idleInactiveNR-MeasReport-r16.

- The update of FR1 known SCell activation delay requirement shall be based on the solution for issue in proposal 2.

# References

[1] RP-240830, NR Radio Resource Management (RRM) Phase 5, Apple, CATT, RANP#103