3GPP TSG-RAN WG4 Meeting#112		R4-2411456

Maastricht, the Netherlands, Aug 19th -23rd, 2024

Agenda item:	8.1.1.3.1

Source: 	Spreadtrum Communications

Title: 	Discussion on reference sensitivity requirements for 6RX UE

Document for:	Discussion

Introduction

In RAN4#111 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

General considerations for specifying ΔRIB,6R value

Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA Way Forward: Further discuss the following options. - Option 1: RAN4 needs to determine whether to define different ΔRIB,6R value for handheld UE and FWA separately - Option 2: Same value for handheld UE and FWA - Option 3: Different value for handheld UE and FWA

Handheld UE has more severe impact of coupling and antenna correlation due to limited size compared with form factor for FWA. Unless it is a foldable smart phone, the implantation of 6RX is a challenge for handheld UE. On the whole, we should distinguish handheld UE and FWA for ΔRIB,6R value.

Proposal 1: We support option 3. Different value for handheld UE and FWA.

ΔRIB,6R value for handheld UE and FWA

Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA Way Forward: Further discuss the following options for the ΔRIB,6R values. Ultimately, the decision to specify separate or same requirements for handheld UE and FWA will be dependent on outcome of Issue 1-1-2. - Option 1: Consider range of ΔRIB,6R in brackets for each case below for further discussion. Operating bands; ΔRIB,6R for FWA; ΔRIB,6R for Handheld. n77, n78, n79, [n104]; [-3.0 ~ -3.3] dB; [-3.0 ~ -3.2] dB. n41; [-3.0 ~ -4.0] dB; [-3.0 ~ -4.0] dB. - Option 2: Consider average value of ΔRIB,6R from company proposals for further discussion. - Option 3: Other options are not precluded.

The number of antenna for 6RX is three times that of 2RX, thus the diversity gain of 6RX is 10log3=4.77 dB higher than for 2RX. In theory, the diversity gain of 6RX is 10log1.5=1.76dB for 4RX and the diversity gain of 8RX is 10log1.33=1.25dB for 6RX. But practically, we should consider the impact of coupling and antenna correlation, which will be more severe than 4RX but less severe than 8RX. Based on the current specification in TS 38.101-1 table 7.3.2-2 and Table 7.3.2-2a, in our understanding, ΔRIB,6R  of FWA/CPE/vehicle/industrial devices is -3.2dB for n77/n78/n79/n104.But n41 has lower frequencies than n77/n78/n79, we could separately consider this question. ΔRIB,6R is -3.6dB for n41. In addition, we think that handheld UE could be lower 0.3dB compared with CPE/FWA/vehicle/industrial devices for n41 and could be lower 0.2dB compared with CPE/FWA/vehicle/industrial devices for n77/n78/n79/n104. In view of the above, we could take the table 1 into consideration.

Proposal 2: Adopt the value of ΔRIB,6R  in Table1 for n41/n77/n78/n79/n104 for handheld UE and FWA.

Table1: Six antenna port reference sensitivity allowance ΔRIB,6R

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.61,-3.32 |
| n77, n78, n79,n104 | -3.21,-3.02 |
| NOTE 1: When 6 Rx operation is supported by FWA form factor. NOTE 2: When 6 Rx operation is supported by handheld UE. |  |

Proposal 3: We accept option2 that consider the average value of ΔRIB,6R from company proposals for further discussion.

Conclusions

In this paper, we provide our views on 6RX UE RF requirements, we have made the following proposals.

Proposal 1: n104 should be included in the high band (n77, n78, n79) category for 6RX case.

Proposal 2: Different values are used for handheld UE and FWA.

Proposal 3: Adopt the value of ΔRIB,6R  in Table1 for n41/n77/n78/n79/n104 for handheld UE and FWA.

Table1: Six antenna port reference sensitivity allowance ΔRIB,6R

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.61,-3.32 |
| n77, n78, n79,n104 | -3.21,-3.02 |
| NOTE 1: When 6 Rx operation is supported by FWA form factor. NOTE 2: When 6 Rx operation is supported by handheld UE. |  |

- Reference

- R4-2410751, WF on requirements for 6RX, AT&T.TSG-RAN4#111 meeting.