3GPP TSG-RAN WG4 Meeting#112		R4-2411457

Maastricht, the Netherlands, Aug 19th -23rd, 2024

Agenda item:	8.1.1.3.3

Source: 	Spreadtrum Communications

Title: 	Discussion on SRS antenna switching requirements for 6RX UE

Document for:	Discussion

Introduction

In RAN4#111 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

SRS antenna switching configurations

Issue 2-1-2: Whether to use different ∆TRxSRS based on operating frequency Way forward: RAN4 defines different ∆TRxSRS values based on operating frequency. Further discuss whether to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104

As we all known, if the frequency is higher, the insertion loss of front-end devices and PCB traces will be higher. However, according to our practical experience, ∆TRxSRS value of n104 is higher than n79 by 0.5dB. In our understanding, there is no obvious difference between n79 and n104 for ∆TRxSRS value. Hence, there is no need to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

Proposal 1: There is no need to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

ΔTRXSRS value

Issue 2-2-1: Proposed ∆TRxSRS values Way forward: Further discuss the following options for the ∆TRxSRS requirements. - Option 1: Consider average value of ∆TRxSRS for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; 4.0 dB; 3.4 dB; 4.4 dB. Band n79, [n104]; 5.3 dB; 4.5 dB; 5.7 dB. - Option 2: Consider range of ∆TRxSRS in brackets for each case below for further discussion. Operating bands; ΔTRxSRS requirement for t1r6; ΔTRxSRS requirement for t2r6; ΔTRxSRS requirement for t1r6-t2r6. Band n41, n77, n78; [2.8 ~ 5.0] dB; [2.3 ~ 4.0] dB; [3.3 ~ 5.5] dB. Band n79, [n104]; [3.6 ~ 6.0] dB; [3.0 ~ 5.5] dB; [4.2 ~ 6.5] dB. Note: Maximum level for t1r6 corrected for OPPO proposals per OPPO. - Option 3: Other options are not precluded.

A conclusion was reached in RAN-P #104 meeting that RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. SRS antenna switching to support 4T6R is precluded. According to the above conclusion, we could first consider ΔTRXSRS for 1T6R, 2T6R for PC3. It is observed that 2T6R can be divided into 1T4R and 1T2R. Thus we can get the value of ΔTRXSRS for 2T6R is 3 dB. For 1T6R, we can see the difference with 8RX (in Figure1 and Figure2 in Annex) is the length of PCB (0.5dB), thus we can get ΔTRXSRS of 1T6R is 3.5dB. Similarly, we did not see the difference between the 1T6R-2T6R architecture and1T6R. And so on, we can get the following table.

Proposal 2: Adopt the value of ΔTRXSRS in Table 1 for 1T6R, 2T6R for PC3.

Table 1: ΔTRXSRS for 1T6R, 2T6R for PC3

| UE capability | Bands whose FUL_high is lower than the FUL_low of n79 (dB） | Bands whose FUL_high is higher than the FUL_low of n79 (dB） |
| --- | --- | --- |
| 1T6R/1T6R-2T6R | 3.5 | 5 |
| 2T6R | 3 | 4.5 |

Proposal 3: We accept option1 that consider average value of ∆TRxSRS for further discussion.

Conclusions

In this paper, we provide our views on 6RX UE RF requirements, we have made the following proposals.

Proposal 1: There is no need to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

Proposal 2: Adopt the value of ΔTRXSRS in Table 1 for 1T6R, 2T6R for PC3.

Table 1: ΔTRXSRS for 1T6R, 2T6R for PC3

| UE capability | Bands whose FUL_high is lower than the FUL_low of n79 (dB） | Bands whose FUL_high is higher than the FUL_low of n79 (dB） |
| --- | --- | --- |
| 1T6R/1T6R-2T6R | 3.5 | 5 |
| 2T6R | 3 | 4.5 |

Proposal 3: We accept option1 that consider average value of ∆TRxSRS for further discussion.

- Reference

- R4-2410751, WF on requirements for 6RX, AT&T.TSG-RAN4#111 meeting.

- Annex

Figure 1: an example of 1T6R architecture                         Figure 2: an example of 1T8R architecture