3GPP TSG-RAN WG4 Meeting#112		R4-2411458

Maastricht, the Netherlands, Aug 19th -23rd, 2024

Agenda item:	8.1.1.3.4

Source: 	Spreadtrum Communications

Title: 	Discussion on SRS IL imbalance for 6RX UE

Document for:	Discussion

Introduction

In RAN4#111, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

General considerations for SRS IL imbalance issue

Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behaviour for SRS transmissions in case of SRS IL. - Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL. - Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation.

According to TS 38.213, the transmit power of SRS is described as follows. If a UE transmits SRS based on a configuration by SRS-ResourceSet on active UL BWP b of carrier f of serving cell c using SRS power control adjustment state with index l, the UE determines the SRS transmission power PSRS,b,f,c(i,qs,l) in SRS transmission occasions i as

This formula takes SRS-ResourceSet as granularity, it means that the SRS power of all ports is the same or it is expected that the UE can achieve that no matter what way.

According to TS 38.101, with regard to different ports pass through different devices and PCB loss (SRS insertion loss), TS 38.101 specifies that UE is allowed to The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot with ∆TRxSRS. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

Which means that among different ports of SRS power deviation is allowed. However, according to practical experience, if SRS power is unbalanced among SRS ports, it will affect the channel estimation, which will lead to the deterioration of DL performance. Thus, in order to meet the SRS power balance of all ports and achieve better performance, UE actually made power compensation. However, when the power deviation exceeds a certain value (e.g., 5dB) due to the limited power of PA devices, it cannot be completely compensated. This leads to the final failure to achieve SRS port power balance. In our understanding, UE’s self-compensation is up to UE implementation, but when UE’s self-compensation cannot meet the power balance, it needs to rely on the compensation of gNB.

Observation 1: When the power deviation exceeds a certain value (e.g., 5dB) due to the limited power of the PA device, it cannot be completely compensated by the UE.

Proposal 1: Whether the UE perform power imbalance self-compensation depends on UE implementation.

Proposal 2: The compensation of gNB is necessary when UE cannot keep SRS power balanced.

SRS IL imbalance issue solutions

Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: After determining the set of initial considerations for the study, discuss candidate solutions considering the UE behaviour in terms of the IL imbalance compensation and whether the UE can compensate the IL imbalance for AS-SRS for all power levels and whether NW and UE have consistent understanding for the possible compensation.

Many companies have provided simulation results and proved that the gain to report SRS insertion loss imbalance across SRS ports for 8RX in Rel-18, but some companies have doubts about the reported gain, and finally no agreement has been reached about this. In our understanding, it is necessary to report SRS IL imbalance across SRS ports to the NW because the UE cannot always satisfy the balance between the main branch and the diversity branch SRS port by UE self-compensation. The solution is to enable UE report on the actual SRS IL imbalance to the NW. In our understanding, reporting statically or dynamically is based on UE implantation.

When UE supports to report statically, UE needs to indicate the behaviour of compensation to NW. If the UE does not perform self-compensation, UE could report a two-dimensional table which contains the fixed insertion loss mapping to each diversity branch and main branch according to the different capabilities of SRS antenna switching to NW. If the UE performs self-compensation but still does not keep the power balanced, UE could configure a power threshold that SRS power of main branch below this value there is no need to report, in case that beyond this value, UE could report Q (Q is equal to max power –power threshold) two-dimensional tables which contains the fixed insertion loss mapping to each diversity branch and main branch according to the different capabilities of SRS antenna switching to NW.

When the UE supports to report dynamically, there is no need to distinguish the behaviour whether performs self-compensation. The UE obtains the output power of each SRS port (e.g. by PD detection) and reports the output power to NW, then NW calculates the difference value (main branch output power and each diversity branch output power) or UE calculates the difference value of each diversity branch output power then report to NW according to the SRS period (including periodic, semi-persistent and aperiodic) in real time.

Proposal 3: If UE reports statically, UE needs to indicate compensation behaviour to NW.

- If the UE does not perform self-compensation, the UE should report a two-dimensional table which contains the fixed insertion loss mapping to NW according to the different SRS antenna switching capabilities.

- If the UE does perform self-compensation but still cannot keep the power balanced, UE could configure a power threshold in case that beyond this value, UE could report Q (Q is equal to max power –power threshold) two-dimensional tables which contains the fixed insertion loss mapping according to the different SRS antenna switching capabilities.

Proposal 4: If UE reports dynamically, UE report the difference value of each diversity branch output power to NW according to the SRS period (including periodic, semi-persistent and aperiodic) in real time.

Conclusions

In this paper, we provide our views on SRS IL imbalance, we have made the following observation and proposals.

Observation 1: When the power deviation exceeds a certain value (e.g., 5dB) due to the limited power of the PA device, it cannot be completely compensated by the UE.

Proposal 1: Whether the UE perform power imbalance self-compensation depends on UE implementation.

Proposal 2: The compensation of gNB is necessary when UE cannot keep SRS power balanced.

Proposal 3: If UE reports statically, UE needs to indicate compensation behaviour to NW.

- If the UE does not perform self-compensation, the UE should report a two-dimensional table which contains the fixed insertion loss mapping to NW according to the different SRS antenna switching capabilities.

- If the UE does perform self-compensation but still cannot keep the power balanced, UE could configure a power threshold in case that beyond this value, UE could report Q (Q is equal to max power –power threshold) two-dimensional tables which contains the fixed insertion loss mapping according to the different SRS antenna switching capabilities.

Proposal 4: If UE reports dynamically, UE report the difference value of each diversity branch output power to NW according to the SRS period (including periodic, semi-persistent and aperiodic) in real time.

- Reference

- R4-2410751, WF on requirements for 6Rx, AT&T,TSG-RAN4#111 meeting