3GPP TSG-RAN WG4 Meeting #112	R4-2411462

Maastricht, NL, 19th - 23th Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4676 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_solutions-Core) CR on update field name of NTN features for NGSO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | 2024-8-19 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | R2 LS is agreed (R2-2403858) to change the field name as below. <1> Rename enhancedMeasurementLEO to enhancedMeasurementNGSO. <2> Rename maxNumber-LEO-SatellitesPerCarrier-r17 to maxNumber-NGSO-SatellitesPerCarrier-r17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | update field name of NTN features for NGSO |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect name of NTN features for NGSO |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2C.2.3, 4.2C.2.4, 9.2C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Change>

#### 4.2C.2.3	Measurements of intra-frequency NR cells

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

If Srxlev > SIntraSearchP and Squal > SIntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if the distanceThresh is configured  (see TS 38.304[1]) and UE has location information, then the UE is not required to perform measurement of intra-frequency.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 [1] within Kmulti_SMTC * Tdetect,NR_Intra when that Treselection= 0 if the UE does not support the feature for enhanced RRM requirements defined in TS38.306 [14] or if the enhancedMeasurementNGSO-r17 is not enabled, or within Kmulti_SMTC *  Tdetect,NR_Intra_enh if the UE supports the feature for enhanced RRM requirements defined in TS38.306 [14] and the enhancedMeasurementNGSO-r17 is enabled. An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.1.6 for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Kmulti_SMTC * Tmeasure,NR_Intra (see table 4.2C.2.3-1) if the UE does not support the feature for enhanced RRM requirements defined in TS38.306 [14] or if the enhancedMeasurementNGSO-r17 is not enabled, or every Kmulti_SMTC * Tmeasure,NR_Intra_enh (see table 4.2C.2.3-2) if the UE supports the feature for enhanced RRM requirements defined in TS38.306 [14]  and the enhancedMeasurementNGSO-r17 is enabled, for intra-frequency cells that are identified and measured according to the measurement rules.

The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2.

For UE in FR1-NTN:

If smtcs do not overlap with each other,

-	Kmulti_SMTC=1, if GEO satellites are measured on the carrier;

-	Kmulti_SMTC=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier;

-	If smtcs partially overlap with each other,

-	Kmulti_SMTC=NSMTC,overlap, if only GEO satellites are measured on the carrier;

-	Kmulti_SMTC=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier;

Where

-	NLEO,i Is the number of LEO satellites to be measured within i-th SMTC,

-	NLEO,simul Is the number of LEO satellites that UE can measure in parallel within an SMTC,

-	NSMTC,overlap Is the number of smtcs that partially overlap with each other.

Note: for deriving Kmulti_SMTC for Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tdetect,NR_Intra, Tmeasure,NR_Intra or Tevaluate,NR_Intra.

For UE in FR2-NTN, Kmulti_SMTC = 1.

The parameter Kmulti_SMTC is the scaling factor for measurements of multiple SMTCs which correspond to different satellites.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Kmulti_SMTC * Tevaluate,NR_Intra if the UE does not support the feature for enhanced RRM requirements defined in TS38.306 [14]  or if the enhancedMeasurementNGSO-r17 is not enabled, or within Kmulti_SMTC * Tevaluate,NR_Intra_enh if the UE supports the feature for enhanced RRM requirements defined in TS38.306 [14]  and the enhancedMeasurementNGSO-r17 is enabled, when Treselection = 0 as specified in table 4.2C.2.3-1 or table 4.2C.2.3-2 provided that:

-	when rangeToBestCell is not configured:

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

-	when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

-	the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2 if the current serving cell is among them.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a nonzero value and the intra-frequency cell is satisfied with the reselection criteria which are defined in TS38.304 [1], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

Table 4.2C.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra

| DRX cycle length [s] | Scaling Factor (N1) | Tdetect,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tevaluate,NR_Intra [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
|  | FR1 |  |  |  |
| 0.32 | 1 | 11.52 x N1 x M2 (36 x N1 x M2) | 1.28 x N1 x M2 (4 x N1 x M2) | 5.12 x N1 x M2 (16 x N1 x M2) |
| 0.64 |  | 17.92 x N1 (28 x N1) | 1.28 x N1 (2 x N1) | 5.12 x N1 (8 x N1) |
| 1.28 |  | 32 x N1 (25 x N1) | 1.28 x N1 (1 x N1) | 6.4 x N1 (5 x N1) |
| 2.56 |  | 58.88 x N1 (23 x N1) | 2.56 x N1 (1 x N1) | 7.68 x N1 (3 x N1) |
| Note 1: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 4 upon more than 1 SMTC configured at the UE; M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; otherwise M2=1. Where, NSMTC is the number of SMTCs configured by SAN If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. Note 2: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment. |  |  |  |  |

Table 4.2C.2.3-2: Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh

| DRX cycle length [s] | Tdetect,NR_Intra_enh [s] (number of DRX cycles) | Tmeasure,NR_Intra_enh [s] (number of DRX cycles) | Tevaluate,NR_Intra_enh [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | 2.56 x M2 (8 x M2)Note 1 | 0.32 x M3 (1 x M3) Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | 5.12 (8) | 0.64 (1) | 1.92 (3) |
| 1.28 | 8.96 (7) | 1.28 (1) | 3.84 (3) |
| 2.56 | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| Note 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 2, M3 = M4 = 2.5 |  |  |  |

If ‘t-Service’ is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before the serving cell stops serving the area regardless of whether the distance condition based on serving cell reference location is met or the legacy Srxlev/Squal condition are met, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-Service is acquired by the UE for the first time to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger.

Ttrigger = max(Kmulti_SMTC*Tdetect,NR_Intra, Kmulti_SMTC*Kcarrier* Tdetect,NR_Inter),

where

-	Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell,

-	Tdetect,NR_Intra refers to intra-frequency cell detection delay in IDLE/INACTIVE mode defined Table Table 4.2C.2.3-2,

-	Tdetect,NR_Inter refers to inter-frequency cell detection delay in IDLE/INACTIVE mode defined Table 4.2C.2.4-2.

The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in Table 4.2C.2.3-1 and Table 4.2C.2.3-2 is expected.

#### 4.2C.2.4	Measurements of inter-frequency NR cells

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2C.2.9.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. The requirements apply provided that the distance exceeds the distanceThresh by a margin of 50 m. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS38.304 [1] within i=1KcarrierKmulti_SMTC,i*Tdetect,NR_Inter if the UE does not support the feature for enhanced RRM requirements defined in TS38.306 [14]  or if the enhancedMeasurementNGSO-r17 is not enabled, or within i=1KcarrierKmulti_SMTC,i*Tdetect,NR_Inter_enh if the UE supports the feature for enhanced RRM requirements defined in TS38.306 [14]  and the enhancedMeasurementNGSO-r17 is enabled, if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met by a margin of at least [5]dB in FR1 for reselections based on ranking or [6]dB in FR1 for SS-RSRP reselections based on absolute priorities or [4]dB in FR1 for SS-RSRQ reselections based on absolute priorities. The parameter Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell.

The parameter Kmulti_SMTC,i is the scaling factor for measurement of multiple SMTCs or multiple satellites.

For FR2-NTN, Kmulti_SMTC,i = 1.

For UE in FR1-NTN:

-	If SMTCs do not overlap with each other,

-	KmultiSMTC,i=1, if GEO satellites are measured on the carrier;

-	KmultiSMTC,i=NLEO,iNLEO,simul, if LEO satellites are measured on the carrier;

-	If SMTCs partially overlap with each other,

-	KmultiSMTC,i=NSMTC,overlap, if only GEO satellites are measured on the carrier;

-	KmultiSMTC,i=i=1NSMTC,overlapNLEO,iNLEO,simul, if only LEO satellites are measured on the carrier;

where

NLEO,i is the number of LEO satellites to be measured within i-th SMTC,

NLEO,simul is the number of LEO satellites that UE can measure in parallel within an SMTC,NSMTC,overlap is the number of SMTCs that partially overlap with each other.

Note: 	for deriving Kmulti_SMTC,i for Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter of frequency layer i, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tdetect,NR_Inter, Tmeasure,NR_Inter or Tevaluate,NR_Inter.

An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.1.7 for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every i=1KcarrierKmulti_SMTC,i*Tmeasure,NR_Inter (see table 4.2C.2.4-1) if the UE does not support the feature for enhanced RRM requirements defined in TS38.306 [14]  or if the enhancedMeasurementNGSO-r17 is not enabled, or every i=1KcarrierKmulti_SMTC,i*Tmeasure,NR_Inter_enh (see table 4.2C.2.4-2) if the UE supports the feature for enhanced RRM requirements defined in TS38.306 [14]  and the enhancedMeasurementNGSO-r17is enabled, for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [1] within i=1KcarrierKmulti_SMTC,i*Tevaluate,NR_Inter if the UE does not support [capability for enhanced requirements] or if the [NW configuration for enhanced requirements] is not enabled, or within i=1KcarrierKmulti_SMTC,i*Tevaluate,NR_Inter_enh if the UE supports the feature for enhanced RRM requirements defined in TS38.306 [14]  and the enhancedMeasurementNGSO-r17 is enabled, when Treselection = 0 as specified in table 4.2C.2.4-1 provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and

when rangeToBestCell is not configured:

-	the cell is at least [5]dB better ranked in FR1 or.

when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them

-	the cell is at least [5]dB better ranked in FR1 if the current serving cell is among them. or

-	[6]dB in FR1 for SS-RSRP reselections based on absolute priorities or

-	4]dB in FR1 for SS-RSRQ reselections based on absolute priorities.

### <Next Change>

### 9.2C.3	Number of cells and number of SSB

#### 9.2C.3.1	Requirements for FR1

For each intra-frequency layer, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least:

-	8 identified cells, and

-	8 SSBs with different SSB index and/or PCI on the intra-frequency layer, where the number of SSBs in the serving cell (except for the SCell) is not smaller than the number of configured RLM-RS SSB resources.

-	4 SSBs with different SSB index and/or PCI from neighbour cells in GEO deployment.

-	In NGSO deployments: cells from 2 satellites including the satellite serving the PCell if UE does not support the capability  maxNumber-NGSO-SatellitesPerCarrier-r17; or cells from 3 or 4 satellites satellites including the satellite serving the PCell, depending on the value indicated in maxNumber-NGSO-SatellitesPerCarrier-r17.

### <End of Change>