3GPP TSG-RAN WG4 Meeting #112	R4-2411464

Maastricht, NL, 19th - 23th Aug 2024

Agenda Item:	5.29.1

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for inter-band SSB-less SCell operation

WID/SID:        Netw_Energy_NR-Core

Document for:	Discussion

# 1  Introduction

Remaining issue on inter-band SSB-less SCell operation is dicussed.

# 2  Multiple SSB-less SCells activation

The delay requirement for multiple SSB-less SCells activation was discussed in [1].

Issue 1-1-3: Multiple SSB-less SCells activation

Agreements:

  - Also define requirements for following scenario:

    - When all to-be-activated SCells are SSBless on the same band and SCells are contiguous.

    - All to-be-activated SCells have same QCL source cell.

    - P-TRS is used.

      - Option 1: The multiple SCell activation delay requirements are based on TRS with the shortest periodicity. The detailed requirements can be discussed in the CR.

      - Option 2: UE activate each to-be-activated SCell based on the TRS on the SCell, and the requirements to be defined accordingly (i.e. single CC delay)

    - FFS A-TRS based requirements.

It can be observed that Option 1 implies that the UE shall determine the fine timing of one SSB-less cell using the TRS from another SSB-less cell, with the shortest periodicity. This approach represents an enhancement and will require a new UE capability. To establish the minimum requirement, we recommend adopting Option 2.

Proposal 1: Regarding the minimum requirement of multiple SSB-less SCells activation, the UE should activate each to-be-activated SCell based on the TRS on the SCell, and the requirements to be defined accordingly (Option 2).

# Conclusion

Proposal 1: Regarding the minimum requirement of multiple SSB-less SCells activation, the UE should activate each to-be-activated SCell based on the TRS on the SCell, and the requirements to be defined accordingly (Option 2).

# References

- RP-241650, Revised WID: Enhancements of network energy savings for NR

- Chair notes RAN1_117 eom1