3GPP TSG-RAN WG4 Meeting #112	R4-2411467

Maastricht, NL, 19th - 23th Aug 2024

Agenda Item:	8.8

Source:	MediaTek Inc.

Title:	Discussion on Enhanced RRM requirements and conductive test methodology for NTN

WID/SID:        NR_IoT_NTN_req_test_enh

Document for:	Discussion

# 1  Introduction

In RAN#104 meeting, the “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” has been approved in [1], and the main impact on RRM requirement is discussed in this contribution as following:

- Core: Less than 5MHz for NR NTN

- Perf: NTN testing for NGSO on RRM performance requirements and test cases

# 2  Impact from Less than 5MHz for NR NTN on RRM core requirements

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

The RRM requirements specified in R18 TN NR_FR1_lessthan_5MHz_BW can be leveraged to update the requirements for FR1 NR NTN. The potential impact on RRM requirement is analyzed below, with considerations that it introduces smaller BW for <5MHz (e.g. 12/15/20 PRBs PDCCH transmission parameters), and it only impacts on SSB/PBCH-related procedures while CSI-RS has not impacted.

| Requirements | The potential impact from “less than 5MHz” |
| --- | --- |
| IDLE/INACTIVE | No impact foreseen |
| CONN Mobility (Handover) & CONN Control | Relax HO delay requirements for SSB BW of 12PRBs. |
| Timing | No impact foreseen |
| Signaling characteristics (RLM) (CONN), BFD/CBD | - SSB RLM/BFD: Update PDCCH transmission parameters - CSI-RS RLM/BFD: No impact foreseen |
| Meas. Procedure in CONN | Update Time period for time index detection |

Proposal 1: RAN4 to revisit the following NR NTN RRM requirements, for the impact from “less than 5MHz”:

- HO delay requirements for SSB BW of 12PRBs

- SSB RLM/BFD: PDCCH transmission parameters

- Measurement Procedure: Time period for time index detection

# 2  Impact from NTN testing for NGSO on RRM performance requirements and test cases

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

For the objective of NTN testing as shown above [1], RAN4 is requested to specify the TE-emulated channel model with varying Doppler and delay shifts for NTN bands. The main impact on RRM is the uplink timing test cases for NGSO. As performance part TU has been provided, we would suggest to discuss the TE-emulated channel model in demodulation session, in order to leverage the discussion of HST (where varying Doppler has been specified) and to avoid duplicated discussion in both RRM and Demod. Once the TE-emulated channel model has been defined, RRM UL timing test cases can be updated accordingly.

Proposal 2: Discuss the TE-emulated channel model with varying Doppler and delay shifts for NTN bands in demod session. Update RRM uplink timing test cases for NGSO when the TE-emulated channel model has been specified.

# 3  Conclusion

Proposal 1: RAN4 to revisit the following NR NTN RRM requirements, for the impact from “less than 5MHz”:

- HO delay requirements for SSB BW of 12PRBs

- SSB RLM/BFD: PDCCH transmission parameters

- Measurement Procedure: Time period for time index detection

Proposal 2: Discuss the TE-emulated channel model with varying Doppler and delay shifts for NTN bands in demod session. Update RRM uplink timing test cases for NGSO when the TE-emulated channel model has been specified.

# References

- RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN