3GPP TSG-RAN WG4 Meeting #112	R4-2411468

Maastricht, NL, 19th - 23th Aug 2024

Agenda Item:	8.21.2

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for network energy saving

WID/SID:        Netw_Energy_NR_enh-Core

Document for:	Discussion

# 1  Introduction

In RAN#104 meeting, the “Revised WID: Enhancements of network energy savings for NR” has been approved in [1],. the objective is listed below. The 1st objective and the 3rd objective marked with R4 impact are discussed in this contribution.

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

# 2  On-Demand SSB SCell operation (Objective#1 )

On-Demand SSB SCell operation will have clear RRM requirement impact on the SCell activation requirements, as specified in 8.3.2 in 38.133.

Agreement

- For a cell supporting on-demand SSB SCell operation,

  - Support RRC based signaling to indicate on-demand SSB transmission on the cell.

  - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

In RAN1#117, it agreed that both RRC and MAC-CE triggering method are supported.

Besides, depending whether there is always on legacy SSB or not, there are 2 cases

- Case 1: always-on legacy SSB + OD-SSB

  - Case 1a: always-on legacy SSB + RRC based OD-SSB

  - Case 1b: always-on legacy SSB + MAC-CE based OD-SSB

- Case 2: standalone OD-SSB, i.e. a SSB-less SCell

  - Case 1a: standalone RRC based OD-SSB

  - Case 1b: standalone MAC-CE based OD-SSB

Proposal 1: RAN4 to discuss SCell activation requirements for R19 NES for the following scenarios:

- Case 1: RRC based / MAC-CE based OD-SSB operation for a SSB SCell

- Case 2: RRC based / MAC-CE based OD-SSB operation for a SSB-less SCell

# 3 Time domain adaptation of SSB/PRACH/Paging occasion (Objective#3)

Regarding objective#3, there is a need to justify the introduction of new RRM requirements for the time domain adaptation of SSB/PRACH/Paging occasion. For instance, consider the scenario where the SSB transitions from long periodicity to short periodicity. However, the existing RRM requirements are applicable to both long periodicity and short periodicities, suggesting that defining new requirements for the transition behavior may not be essential.

Proposal 2: RAN4 to further discuss the necessity of introducing RRM requirements on the transitions behavior for the time domain adaptation of SSB/PRACH/Paging occasion.

# Conclusion

Proposal 1: RAN4 to discuss SCell activation requirements for R19 NES for the following scenarios:

- Case 1: RRC based / MAC-CE based OD-SSB operation for a SSB SCell

- Case 2: RRC based / MAC-CE based OD-SSB operation for a SSB-less SCell

Proposal 2: RAN4 to further discuss the necessity of introducing RRM requirements on the transitions behavior for the time domain adaptation of SSB/PRACH/Paging occasion.

# References

- RP-241650, Revised WID: Enhancements of network energy savings for NR

- Chair notes RAN1_117 eom1