3GPP TSG-RAN WG4 Meeting #112	R4-2411469

Maastricht, NL, 19th - 23th Aug 2024

Agenda Item:	8.25.4

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for NR NTN phase 3

WID/SID:        NR_NTN_Ph3-Core

Document for:	Discussion

# 1  Introduction

In RAN#104 meeting, the “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3” has been approved in [1], and the main impact on RRM requirement would be the objective #5, support of (e)RedCap. The objective is listed below.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

# 2  Impact on RRM Requirement for (e)RedCap with NR NTN

The specified TN Rel-17 RedCap and Rel-18 eRedCap RRM requirements can be leveraged to update the requirements for FR1 NTN Redcap UE. The potential impact from (e)Redcap is analyzed below, with considerations such as eDRX, NCD-SSB, 1 RX and HD-FDD.

| Requirements | The potential impact from (e)Redcap |
| --- | --- |
| IDLE/INACTIVE, including SDT | - Update for eDRX - Update for 1 Rx, including UE measurement capability - Update for HD-FDD, for paging reception |
| CONN Mobility (Handover) & CONN Control | - Update for NCD-SSB - Update for 1 Rx - Update for HD-FDD |
| Timing | - No major update foreseen |
| Signaling characteristics (RLM) (CONN), BFD/CBD | - Update for 1 Rx - Update for HD-FDD |
| Meas. Procedure in CONN | - Update for NCD-SSB - Update for 1 Rx - Update for HD-FDD |

Proposal 1: RAN4 to revisit the following NR NTN RRM requirements, for the impact from (e)Redcap:

- IDLE/INACTIVE mode, including SDT

- CONNECTED Mobility (Handover) and Control

- Signaling characteristics, including RLM and Link Recovery

- Measurement Procedure

# 3  Conclusion

Proposal 1: RAN4 to revisit the following NR NTN RRM requirements, for the impact from (e)Redcap:

- IDLE/INACTIVE mode, including SDT

- CONNECTED Mobility (Handover) and Control

- Signaling characteristics, including RLM and Link Recovery

- Measurement Procedure

# References

- RP-241667, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3