3GPP TSG-RAN WG4 Meeting #112	R4-2411471

Maastricht, NL, 19th - 23th Aug 2024

Agenda Item:	8.26.2

Source:	MediaTek Inc.

Title:	Discussion on RF requirement impact for IoT NTN phase 3

WID/SID:        IoT_NTN_Ph3-Core

Document for:	Discussion

# 1  Introduction

In RAN#104 meeting, the “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” has been approved in [1], and the objectives of this WID are listed below. In this contribution, we provide our view on the RF requirement impacts.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4] ### 4.2 Objective of Performance part WI - Specify, if needed, RRM performance requirements [RAN4] - Specify, if needed, SAN demodulation performance requirements [RAN4]

# 2  Requirement impact on 36.102 and 36.108 for the support of capacity enhancements for uplink

According to the objectives, RAN4 will discuss on whether there is a necessity to update RF requirements for the support of capacity enhancements for uplink. From UE prospective, no impact on UE RF requirement or UE demod requirements is foreseen. However, the decision to update the SAN RF core requirements or SAN demodulation requirements to include OCC configurations may require further discussions. The impact on SAN demodulation performance requirements can be discussed in performance part, e.g. whether to update the test parameters with OCC.

Proposal 1: No need to update UE RF requirements for the support of capacity enhancements for uplink. FFS whether to update SAN RF requirements accordingly. The impact on SAN demodulation performance requirements can be discussed in performance part.

# 3  Conclusion

# Proposal 1: No need to update UE RF requirements for the support of capacity enhancements for uplink. FFS whether to update SAN RF requirements accordingly. The impact on SAN demodulation performance requirements can be discussed in performance part.

# References

- RP-241624, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. (Rapporteur)

- R2-2402941, Work Plan for Rel-19 IoT NTN, MediaTek Inc. (Rapporteur)