3GPP TSG-RAN WG4 Meeting #112	R4-2411472

Maastricht, NL, 19th - 23th Aug 2024

Agenda Item:	8.26.3

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for IoT NTN phase 3

WID/SID:        IoT_NTN_Ph3-Core

Document for:	Discussion

# 1  Introduction

In RAN#104 meeting, the “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” has been approved in [1], and the objectives of this WID are listed below. In this contribution, we provide our view on the RRM requirement impacts.

- Support of Capacity enhancements for uplink ……. - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

# 2  Requirement impact on RRM for EDT-like transaction

1. Overall Description:

In RAN2#126 meeting, the following agreements related to Msg3 transmission for uplink capacity enhancement for R19 IoT NTN have been achieved:

Agreements: => RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). => If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.

Based on the above agreements, RAN2 kindly asks RAN4 and RAN1:

Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2?

Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case?

RAN2 is studying on RACH-less (without Msg1 and RAR) and contention-based Msg3 transmission to complete an EDT-like transaction for the uplink capacity enhancement. An related incoming RAN2 LS [2] is attached below for the uplink synchronization just before sending msg3.

First, it can be observed that the requirement for RACH-less HO has been established in R18 NR NTN enhancement. This stipulates that the UE must update the pre-compensated TA for the incoming PUSCH transmission, considering the valid and applicable parameters such as ephemeris information, common TA, to maintain uplink synchronization. Besides, the corresponding RACH-less test cases has been introduced in R18 NR NTN enhancement. We believe that the similar approach can be applied to the msg3/PUSCH in new R19 contention-based procedure.

Besides, the PUR requirement introduced in R18 LTE_NBIOT_eMTC_NTN_req (specifically in 4.6A.3 in 36.133) shares commonalities with the new R19 EDT-like procedure, as both operate without Msg1/Msg2, it could also serve as a useful reference for the discussion. It can also be observed that the UE shall meet timing accuracy requirements for PUR for IoT NTN was also introduced in 7.20A in 36.133, as below.

## 7.20A	UE transmit timing for NB-IoT for Satellite Access

….

### 7.20A.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.20A.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR.

Proposal 1: For Q1, with the valid and applicable parameters such as ephemeris information, common TA, UE can maintain uplink synchronization by updating pre-compensated TA for the Msg3 transmission without Msg1/Msg2.

However, it can be observed that there was no RRM requirements defied for RACH-based EDT procedure, which was introduced in R15. Thus, whether to introduce new RRM requirements for the new R19 RACH-less contention-based (EDT-like) procedure can be further discussed in RAN4.

Proposal 2: RAN4 to further discuss whether and how to introduce RRM requirements for the new R19 contention-based (EDT-like) procedure.

# 3  Conclusion

Proposal 1: For Q1, with the valid and applicable parameters such as ephemeris information, common TA, UE can maintain uplink synchronization by updating pre-compensated TA for the Msg3 transmission without Msg1/Msg2.

Proposal 2: RAN4 to further discuss whether and how to introduce RRM requirements for the new R19 contention-based (EDT-like) procedure.

# Appendix: draft LS reply

Title:	Reply LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

Response to:	R2-2405769 LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

Release:	Rel-19

Work Item:	IoT_NTN_Ph3-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact Person:

Name:		Hsuanli Lin

E-mail Address:	Hsuanli.Lin@mediatek.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN4 would like to thank RAN2 for the liaison in R2-2405769 LS regarding UL synchronization for contention based Msg3 transmission without Msg1/Msg2 in IoT NTN.

From the RAN4 prospective, given the provision of the valid and applicable parameters such as ephemeris information, common TA, UE can maintain uplink synchronization by updating pre-compensated TA for the Msg3 transmission without Msg1/Msg2.

# 2	Actions

To RAN2

ACTION:  RAN4 respectfully asks RAN2 to take the above information into consideration in their future work.

# 3 	Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #112-bis	14 – 18 Oct 2023		Hefei

TSG RAN WG4 Meeting #113		          18 – 22 Nov 2023		Orlando

# References

- RP-241624, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. (Rapporteur)

- R2-2405769, LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, ZTE. May 2024.