3GPP TSG RAN WG4 Meeting # 112                                   R4-2411473

Maastricht, NL, Aug 19 - Aug 23, 2024

Title:	RRM Core requirements on NR mobility enhancements Phase 4

Source:	China Telecom

Agenda item:			8.23.2

Document for:	Discussion

- Introduction

A new R19 work item [1] has been approved to NR mobility enhancements:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we provide some initial thinking on RAN4 related objectives.

- Discussion

In previous RAN2 meeting, some conclusions have been achieved on event trigger L1 measurement report. It should be designed for selecting candidate beam(s)/cell(s) to trigger early synchronization and selecting target beam/cell to trigger LTM cell switch procedure.

And the events for triggering L1 measurement report have been defined preliminarily. The event A1-A5 for triggering L3 measurement report are reused. Event LTM1-Event LTM5 correspond to A1-A5 separately. But the need for Event LTM1 is still under discussion.

Usually, L1 measurement is used to beam level, so we think we can start to discuss the requirements for the case where use beam level measurement result for event evaluation and use it as baseline for cell level measurement.

Observation 1: Event trigger L1 measurement report should be designed for selecting candidate beam(s)/cell(s) to trigger early synchronization and selecting target beam/cell to trigger LTM cell switch procedure.

Observation 2: The event A1-A5 for triggering L3 measurement report are reused as Event LTM1-Event LTM5 and the need for Event LTM1 is still under discussion.

Proposal 1: Use beam level measurement result for event evaluation as baseline and FFS for cell level measurement

In R18 LTM, the UE based TA measurement has been supported in RAN1/2 specification. But RAN4 did not define dedicated requirements for UE based TA measurement in R18 with limited time and other cases.

What’s more, we think UE based TA measurement will be an important feature for conditional LTM. So, we think we can continue to discuss the requirements on UE based TA measurement in R19.

Proposal 2: We can continue to discuss the requirements on UE based TA measurement.

- Summary

In this paper, we provide our views on NR mobility enhancements Phase 4. From this discussion we have derived the following proposals:

Observation 1: Event trigger L1 measurement report should be designed for selecting candidate beam(s)/cell(s) to trigger early synchronization and selecting target beam/cell to trigger LTM cell switch procedure.

Observation 2: The event A1-A5 for triggering L3 measurement report are reused as Event LTM1-Event LTM5 and the need for Event LTM1 is still under discussion.

Proposal 1: Use beam level measurement result for event evaluation as baseline and FFS for cell level measurement

Proposal 2: Continue to discuss the requirements on UE based TA measurement.

- References

[1] RP-241515, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.