3GPP TSG-RAN WG4 Meeting #112                                   R4-2411476

Maastricht, Netherlands, 19th – 23rd August, 2024

Source: 	Murata Manufacturing Co Ltd.

Title: 	REFSENS and RSD for n28 UL 40MHz

Agenda Item:	7.20.3	UE RF requirements for introducing 40MHz	[NR_n28_PC2_40MHz_CBW-Core]

Document for:	Approval

Introduction

A new WID [1] was approved with the aim of introducing 40MHz UL BW for n28 in the RANP#104 meeting.

In this paper, we proposed REFSENS for PC3 and RSD for PC2.

Discussion

Rx band noise

When UL BW is 40MHz, the Rx band noise of PA is expected to significantly increase due to the IM3 products of (Tx signal, LO leak) and (Tx signal, IQ image) fall into Rx band, as shown in Figure 1.

Figure 1. Illustration of PA output signal

We measured the Rx band noise using traditional measurement conditions. The results showed that the Rx band noise is -12.8dBm for PC3, and -10.0dBm for PC2.

REFSENS and RSD estimation

FE architecture

The 2 kinds of FE architecture shown in Figure 2 are used to estimate REFSENS and RSD. For 2Tx/2Rx architecture, 1dB noise increase due to reverse IMD was considered.

Figure 2. FE architecture

Estimation results

The following assumptions were used for the estimation. The results are summarized in Table 1.

- Duplexer isolation at Tx/Rx: 50 dB

- Duplexer attenuation at Rx band: 45 dB

- FE Loss: 4 dB

- Antenna isolation: 10 dB

Table 1. REFSENS and RSD estimation results

Based on the above estimation results, we have the following proposals.

Proposal 1: Approve REFSENS and UL configurations for n28 UL 40MHz for PC3 highlighted in Table 2 and Table 3.

Table 2. n28 REFSENS for PC3

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n28 | 15 | -100.2 | -98.5 | -95.5 | -93.5 | -90.8 | -84.2 | -78.5 |  | -67.1 |  |  |
|  | 30 |  |  | -95.6 | -93.6 | -91.0 | -84.2 | -78.6 |  | -67.2 |  |  |

Table 3. Uplink configuration for n28 REFSENS

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |
| n28 | 15 | 15 | 25 | 251 | 251 | 251 | 251 | 251 |  | 251 |  |  |  |
|  | 30 |  |  | 101 | 101 | 101 | 101 | 101 |  | 101 |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: Approve RSD for n28 UL 40MHz for PC2 highlighted in Table 4.

Table 4. RSD for PC2 n28 40MHz

| PC2 | RSD 1Tx | dB | 2.8 |
| --- | --- | --- | --- |
|  | RSD 2Tx | dB | 8.8 |

Conclusion

In this paper, we presented the REFSENS of PC3 and the RSD of PC2 for n28 UL 40MHz.

Proposal 1: Approve REFSENS and UL configurations for n28 UL 40MHz for PC3 highlighted in Table 2 and Table 3.

Table 2. n28 REFSENS for PC3

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
| n28 | 15 | -100.2 | -98.5 | -95.5 | -93.5 | -90.8 | -84.2 | -78.5 |  | -67.1 |  |  |
|  | 30 |  |  | -95.6 | -93.6 | -91.0 | -84.2 | -78.6 |  | -67.2 |  |  |

Table 3. Uplink configuration for n28 REFSENS

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 |  |
| n28 | 15 | 15 | 25 | 251 | 251 | 251 | 251 | 251 |  | 251 |  |  |  |
|  | 30 |  |  | 101 | 101 | 101 | 101 | 101 |  | 101 |  |  |  |
| Note 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.3.2-1). |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 2: Approve RSD for n28 UL 40MHz for PC2 highlighted in Table 4.

Table 4. RSD for PC2 n28 40MHz

| PC2 | RSD 1Tx | dB | 2.8 |
| --- | --- | --- | --- |
|  | RSD 2Tx | dB | 8.8 |

References

[1] RP-241660 “New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28”, CMCC, CBN, 3GPP TSG RAN Meeting #104, Shanghai, June 17 – 20, 2024