3GPP TSG-RAN WG4 Meeting #112									R4-2411477

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on core requirements for FR2 multi-Rx

Source: 	OPPO

Agenda item:	5.13.1

Document for:	Discussion

Introduction

In last meeting, a topic summary [1] was discussed and a WF [2] was approved on RRM requirements for FR2 multi-Rx reception. In this contribution, we will provide discussion on the remaining 2 open issues in core part.

Discussion

Issue 1-1-1: When is UE considered to be in multi-rx operation - Proposals - Proposal 1: Keep the agreed definition - Given UE indication of its preference of multi-RX/single-RX operation has been allowed, it is OK to take the current definition of multi-RX operation - Proposal 2: Add following additional condition - P2a: - UE sent a recent valid Rel-17 group-based beam reporting (GBBR). - If UE recently reported ‘Not valid’ for one of the RSRP for a beam pair, this means UE is allow to fallback to single panel for the later reception QCL-ed to that beam pair. - P2b: Rel-17 group-based beam reporting (GBBR) is activated/triggered by the network. - Recommended WF: - Further discuss.

It was agreed that for fast beam sweeping the UE is in multi-Rx operation if the UE is configured with group-based beam reporting (GBBR) report. Some companies point out that, it’s possible that NW configure both GBBR and non-GBBR in mTRP for SDM and TDM, and if network only intend to configure GBBR for SDM but not for fast beam sweeping, UE may be forced to perform fast beam sweeping in non-GBBR.

However, there also exist another observation that fast beam sweeping could be enabled in two ways: One is NW operates in multi-TRP mode to schedule the UE so that GBBR is always configured for tracking and updating TCI states, and another is NW does not intend to operate in multi-TRP mode and GBBR is configured only for enabling fast beam sweeping. We slightly agree with this opinion. So we support to keep current condition, and also agree to update the wording to “Rel-17 group-based beam reporting (GBBR) is configured and activated by the network.”

Proposal 1: Update one of the conditions of multi-Rx operation as: Rel-17 group-based beam reporting (GBBR) is configured and activated by the network.

Issue 1-1-5: Update the condition of scheduling restriction relaxation for RLM/ (cell level) BFD: - Proposals - (From) [The CSI-RS and only one of the PDSCHs with different QCLed typeD are on the same OFDM symbol(s)] - (To) The CSI-RS and both of the PDSCHs, or the CSI-RS and one of the PDSCHs with different QCL typeD when partially overlapping PDSCHs are scheduled, are on the same OFDM symbol(s) - Recommended WF - Check if they are agreeable

For the overlapping conditions of CSI-RS resources and PDSCHs, companies are encouraged to bring further analysis.

- For the fully overlapping case when the two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol, it is feasible for the UE to perform simultaneous measurements and also simultaneous data reception.

- For the partial overlapping case when the two CSI-RS resources and one of the PDSCH are overlapped on the OFDM symbol, it is also feasible for UE to perform simultaneous measurements.

- For the non-overlapping case when the two CSI-RSs are not overlapped with any PDSCHs, e.g., no PDSCHs are scheduled in the slot, the UE can perform simultaneous measurements on the two CSI-RSs as long as they are QCL-ed with typeD to reference signals in a resource group in the latest Rel-17 group based beam report.

Thus, we propose the updated conditions for Scheduling availability and Measurement restrictions for CSI-RS based RLM/BFD as below.

- Both CSI-RSs are not in any CSI-RS resource set with repetition ON, and - One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH, and - the two CSI-RS resources are overlapped on the same OFDM symbol, where at least one of the PDSCHs is scheduled simultaneously. - Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

Proposal 2: Update one of the conditions of scheduling and measurement restriction relaxation as: the two CSI-RS resources are overlapped on the same OFDM symbol, where at least one of the PDSCHs is scheduled simultaneously.

Conclusions

In this paper we provided our views.

Proposal 1: Update one of the conditions of multi-Rx operation as: Rel-17 group-based beam reporting (GBBR) is configured and activated by the network.

Proposal 2: Update one of the conditions of scheduling and measurement restriction relaxation as: the two CSI-RS resources are overlapped on the same OFDM symbol, where at least one of the PDSCHs is scheduled simultaneously.

Reference

- R4-2408000, Topic summary for [111][203] FR2_multiRx_part1

- R4-2410356, WF on core maintenance and performance part for multi-Rx WI