# 3GPP TSG-RAN WG4 Meeting #112	R4-2411479

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	5.14.1

Source: 	OPPO

Title: 	On improvement of SCG activation requirements for FR1-FR1 NR-DC

Document for:	Approval

1	Introduction

A WF on Rel-18 R18 eFeRRM was approved in RAN4#111bis meeting [1]. In this contribution, we discuss the leftover issues of SCG activation/deactivation requirements for FR1-FR1 NR-DC.

- Issue 1-4: RACH-based PSCell activation delay FFS: If the PSCell is an unknown FR1 or FR2 PSCell configured with bfd-and-RLM with value true, provided [neither BFD nor RLF] has occurred, if Es/Iot ≥ -2 dB then for FR2 Tsearch = [12]* Trs ms and Tsearch = [1]* Trs ms for FR1. - FFS: Discuss conditions for reduced Tsearch at PSCell activation, when the UE is configured with bfd-and-RLM. - Issue 1-5: PSCell activation delay and PSCell DRX: FFS: - Option 1: - UE PDCCH monitoring requirement in the newly activated PSCell is ambiguous. - UE shall start monitoring PDCCH on the activated PSCell immediately after the SCG activation delay. - Send LS to RAN2 clarifying PDCCH monitoring assumption with RAN2.

2	Discussion

In this meeting, leftover issues (e.g., SCG activation/deactivation) of RRM requirements of FR1+FR1 NR-DC are to be discussed. We are encouraged to discuss conditions for reduced Tsearch at PSCell activation, when the UE is configured with bfd-and-RLM.

According to TS 38.331, the UE used to conduct RACH-less PSCell activation if bfd-and-RLM was configured to true, while UE conducts RACH based PSCell activation if bfd-and-RLM was not configured to true before the reception of the NR RRC message containing the RRCReconfiguration message.

TS 38.331 (clause 5.3.5): 3> else if the SCG was deactivated before the reception of the NR RRC message containing the RRCReconfiguration message: 4> if bfd-and-RLM was not configured to true before the reception of the RRCReconfiguration or RRCResume message containing the RRCReconfiguration message; or 4> if lower layers indicate that a Random Access procedure is needed for SCG activation: 5> initiate the Random Access procedure on the PSCell, as specified in TS 38.321 [3]; (=> RACH-based procedure) 4> else the procedure ends; (=> RACH-less procedure) 3> else the procedure ends;

Thus, we think if the deactivated PSCell is evaluated in good link quality and the UE is configured with bfd-and-RLM and no RLF and beam failure have been detected, we think UE is expected to conduct RACH-less PSCell activation, except that lower layers explicitly indicate that a Random Access procedure is needed for SCG activation.

It is not the common case for RACH-based PSCell activation that UE is configured with bfd-and-RLM with TRUE and no RLF and beam failure have been detected. In the contradictory, for RACH-less PSCell activation UE used to be configured with bfd-and-RLM with TRUE.

Observation 1: It is not the common case for RACH-based PSCell activation that UE is configured with bfd-and-RLM with TRUE.

On the other hand, for RACH-less based PSCell activation, if bfd-and-RLM is configured and TCI state is known, Tsearch = 0 ms, where known TCI state implies at least the relatively good side condition Ês/Iot ≥ -3dB is fulfilled for the RSs configured for bfd-and-RLM for the PSCell being activated. Similarly, for RACH-based PSCell activation, the UE which is configured with bfd-and-RLM =TRUE and has not experienced either beam failure or RLF on the deactivated PSCell while being deactivated, UE may do better to reduce the search time. However, based on current spec, the requirements are defined for the worst case. UE can do better RACH-based PSCell activation, but it does not mean we shall define new requirements for an uncommon case or configuration.

In short, we propose to keep current requirements for RACH-based PScell activation to leave more flexibility for UE implementation. It is not necessary improvement of SCG activation requirements for RACH-based PSCell activation. For RACH based PSCell activation, if the target cell is unknown PSCell, keep Tsearch even in case that UE is configured with bfd-and-RLM with value true and without detecting RLF or BFD.

If the FR2 PSCell is known, Tsearch = 0 ms. If the FR2 PSCell is unknown and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms. If the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch = 3* Trs ms.

Proposal 1：For RACH based PSCell activation, it is not necessary to improve the delay requirements for RACH-based PSCell activation. Keep the same Tsearch for all cases, including that UE is configured with bfd-and-RLM with value true and without detecting RLF or BFD.

3	Conclusion

In this paper, we propose the following observations and proposals for RRM requirements of FR1+FR1 NR-DC. It is not necessary improvement of SCG activation requirements for RACH-based PSCell activation.

Observation 1: It is not the common case for RACH-based PSCell activation that UE is configured with bfd-and-RLM with TRUE.

Proposal 1：For RACH based PSCell activation, it is not necessary to improve the delay requirements for RACH-based PSCell activation. Keep the same Tsearch for all cases, including that UE is configured with bfd-and-RLM with value true and without detecting RLF or BFD.

4	Reference

[1] R4-2410343, WF on RRM Core requirements for FR1-FR1 NR-DC, OPPO