3GPP TSG-RAN WG4 Meeting #112													    R4-2411480

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.24.1

Source:	OPPO

Title:	Discussion on L1/L2 based inter-cell mobility

Document for:	Discussion

- Introduction

Based on the approved WF in last meeting [1], there are still some remaining open issues. In this contribution, we provide further discussion on some open issues for maintenance.

- Discussion

Issue 2-2-1: Measurement period of serving cell L1-RSRP measurement <Way Forward >: Further discuss the following options: - Option 1 (Apple): - introduce NLayer in serving cell L1 RSRP measurement requirement and clarify that it is for UE capable of RTD>CP configured with L1 RSRP measurement on neighbour cell. - Option 2 (OPPO): - Consider to revisit the agreements for L1 RSRP measurement on neighbour cell, e.g., either follow the logic of serving cell L1-RSRP measurement or L3 intra-frequency measurement. - Option 3 (Ericsson, QC): - RAN4 not to modify the serving cell measurement period by scaling it with NLayer for UE supporting RTD > CP.

Generally, we agree that the assumption of L1 RSRP measurements on Serving cell and intra-frequency neighbour cell should be kept aligned. But it’s quite hard to understand why we achieved the agreements on top of sharing factor P to use NLayer to scale the measurement delay for intra-frequency layers of LTM based L1-RSPP measurement. Even NLayer includes both the number of intra-frequency layers configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18] and the number of inter-frequency layers without measurement gaps.

However, for L1 measurement on serving cell, the sharing among measurements on serving cells and intra-frequency neighbour cells within GAPs are considered, and all layers to be measured are equally counted. And P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

Table 9.14.5.1-2: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE capable of [capability of measurement with RTD>CP]

| Configuration | TL1-RSRP_Measurement_Period_SSB_intra (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC*NLayer) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC) *NLayer) |
| DRX cycle > 320ms | ceil(M*P)*TDRX*NLayer |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. [NLayer = the number of intra-frequency layers configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18] + the number of inter-frequency layers without measurement gaps which are configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18]]. Note 2: K = 1.5. Note 3: UE shall at least measure neighbour cells which are activated for TCI state within UE capability and additional cells to measure is up to UE implementation. |  |

Table 9.5.4.1-1: Measurement period TL1-RSRP_Measurement_Period_SSB for FR1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

We support option 3 that RAN4 not to modify the serving cell measurement period by scaling it with NLayer for UE supporting RTD > CP, considering the impact on the UEs already in the market.

Observation 1: The principle to define the delay requirements of intra-frequency RSRP measurements on Serving cell and on LTM neighbour cell should be kept aligned.

Proposal 1: RAN4 not to modify the serving cell measurement period by scaling it with NLayer for UE supporting RTD > CP.

In addition, even compared to intra-frequency L3 measurements on neighbour cells without gaps, the logic of defining sharing factor is not totally followed. The measurement delay is scaled by CSSFintra, but the measurement occasions on PCC and SCC are not equally shared besides considering the number of configured SCell(s) with L3 measurement without MG. To avoid the misalignment, we think option 2 also provides two directions to move forward.

Proposal 2: Consider to revisit the agreements for L1 RSRP measurement on neighbour cell, e.g., either follow the logic of serving cell L1-RSRP measurement or L3 intra-frequency measurement without gap.

Issue 3-3-1: known TCI state conditions Online Agreement < Agreement > - For FR1, the cell switch delay requirement is applicable for the case without L1 measurement and report, under certain conditions. - Further discuss the how the requirement applies together with the condition. < Agreement>: - In FR2: the RAN4 Rel-18 related LTM requirements are not applicable if L1 measurement of LTM candidate cell is not supported or configured. <Way Forward >: Further discuss the following options in FR1: - Option 1: change known TCI state conditions to make cell switch delay requirements applicable to the case without L1 measurement and report - Option 2: Allow cell switch delay requirements applicable to unknown TCI state case with conditions. - Regarding the conditions, further discuss the following options when SNR of the TCI state≥ -3dB - Condition 1: TCI state has been activated and - Condition 1A: the TCI state activation command was received not more than 1280 ms before the cell switch command - Condition 1B: Beam-level L3 measurement results have been reported in 1280 ms - Condition 1C: The RS associated to the target TCI state is available at least every 1280 ms after TCI state activation command - Condition 2: UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command <Way Forward >: Further discuss the following options in FR2: - Condition 3: TCI state has been activated and - Condition 3A: the TCI state activation command was received not more than 1280 ms before the cell switch command - Condition 3B: the TCI state activation command was received not more than 160 ms before the cell switch command - Condition 3C: The RS associated to the target TCI state is available at least every 1280 ms after TCI state activation command - Condition 3D: - Beam-level L3 measurement results have been reported in 1280ms - L1-RSRP measurement period is no larger than 1280ms in FR2 - Condition 4: UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command

For FR1, RAN4 agreed the cell switch delay requirement is applicable for the case without L1 measurement and report, and suggest to further discuss how the requirement applies together with the condition. We support to further clarify on activated TCI state with condition 1C. When SNR of the TCI state≥ -3dB and TCI state has been activated, it is also required that the RS associated to the target TCI state is available at least every 1280 ms after TCI state activation command.

Similar to FR1, the above can also apply to FR2.

Proposal 3: For the applicable condition of FR1and FR2, when SNR of the TCI state≥ -3dB and TCI state has been activated, it is also required that the RS associated to the target TCI state is available at least every 1280ms after TCI state activation command.

- Conclusion

In this contribution, we provide our observations and proposals as follows:

Observation 1: The principle to define the delay requirements of L1 RSRP measurements on Serving cell and intra-frequency neighbour cell should be kept aligned.

Proposal 1: RAN4 not to modify the serving cell measurement period by scaling it with NLayer for UE supporting RTD > CP.

Proposal 2: Consider to revisit the agreements for L1 RSRP measurement on neighbour cell, e.g., either follow the logic of serving cell L1-RSRP measurement or L3 intra-frequency measurement without gap.

Proposal 3: For the applicable condition of FR1and FR2, when SNR of the TCI state≥ -3dB and TCI state has been activated, it is also required that the RS associated to the target TCI state is available at least every 1280ms after TCI state activation command.

- References

- R4-2410302, WF on NR mobility enhancements (part 1), MediaTek Inc.