3GPP TSG-RAN WG4 Meeting #112	R4-2411481

Maastricht, Netherlands, 19th – 23rd August 2024

Agenda Item:	8.9.3 Introduction of Ku Band for NR NTN – System Parameters

Source:	Eutelsat Group

Title:	Analysis of numerology options for Ku band

Document for:	Discussion

# 1	Introduction

At RAN #104, the WID in [1] was approved to start the work to introduce the Ku band to the 3GPP specifications for NTN. The Ku band covering 10.7 GHz - 14.5 GHz falls directly between the long-standing boundaries of FR1 and FR2 and it is therefore not obvious which numerology to apply. Discussion about defining new frequency ranges such as FR3 is outside the scope of this WI, however, one of the early areas requiring decision for Ku is which numerology to use. It was decided at RAN #104 that a choice should be made by RAN #106 to use FR1 or FR2, and that other choices about numerology that would have RAN1 impact such as mixed numerology should be avoided.

This paper describes the key technical factors that influence the decision on whether to choose FR1 or FR2 for the Ku band. This decision needs to be made early in the work as it has a major impact on subsequent work and the structure of the specifications. The primary factors are technical although there are also constraints imposed by the scope of the WID which precludes solutions that require changes to RAN1 specifications. There are also softer factors such as performance, efficient use of spectrum and commercial aspects, which may influence the decision once the hard technical and standardization constraints have been fully understood.

# 2	Technical factors

Table 1 shows the basic FR1 and FR2 parameters which will be referenced in this paper.

Table 1. FR1 vs. FR2 numerology summary

| Attribute | FR1 | FR2 |
| --- | --- | --- |
| Available channel bandwidths per component carrier MHz | 5, 10, 15, 20, 25, 30, 35, 40, 50, 60, 70, 80, 90, 100 | 50, 100, 200, 400 |
| Subcarrier Spacings kHz | 15, 30, 60 | 60, 120, 240 |
| SSB burst duration ms, number of SSB | 5, 4 (< 3 GHz) 5, 8 (> 3 GHz) | 5, 64 |
| SSB period ms | 5, 10, 20, 40, 80, 160 | 5, 10, 20, 40, 80, 160 |
| Max. no. of subcarriers: | 3300 | 3300 |
| Carrier aggregation | Up to 16 carriers | Up to 16 carriers |
| Bandwidth parts | 4 with possibly different SCS | 4 with possibly different SCS |
| Modulation | QPSK, 16 QAM, 64 QAM, 256 QAM, 1024 QAM | QPSK, 16 QAM, 64 QAM, 256 QAM |
| Residual Doppler resistance | Low | High |
| ICI resistance | Low | High |
| Sensitivity to Phase Noise | Not critical at FR1 frequencies but worse at Ku band although PTRS are defined | Not an issue at Ku frequencies due to wider SCS spacings |
| Length of Radio frame ms | 10 | 10 |
| Subframe duration ms | 1 | 1 |
| Number of slots | 1, 2, 4 | 4. 8 |
| Number of symbols per slot | 14 | 14 |
| Duplex mode | FDD, TDD | TDD, (FDD defined for NTN-FR2) |
| Multiple Access Scheme | Downlink: CP-OFDM Up-link: CP-OFDM, DFT-s-OFDM | Downlink: CP-OFDM Up-link: CP-OFDM, DFT-s-OFDM |
| CP duration µs | 4.69, 2.34, 1.17 | 1.17, 0.59, 0.29 |

## 2.1	Channel bandwidths

A solution is needed for NTN that makes efficient use of current and future channelization. Current operation of the Ku band is organized around downlink channels of 250 MHz and uplink channels of 125 MHz for which a 6 x 20 MHz implementation exists. Legacy systems use narrower channels.

Neither FR1 nor FR2 provide the optimal native channel bandwidths to efficiently utilize Ku band. Definition of new channel bandwidths requiring new RAN1 specifications is currently deemed out of scope and so solutions based on intra-band contiguous CA are being considered.

Table 2. Channel bandwidth considerations

| Attribute | FR1 | FR2 | Comments |
| --- | --- | --- | --- |
| Available component carrier bandwidths MHz | 5, 10, 15, 20, 25, 30, 35, 40, 50, 60, 70, 80, 90, 100 | 50, 100, 200, 400 | Not all native bandwidths will be inckluded in the Ku band specification |
| 250MHz Downlink | 100 + 100 + 50 | 200 + 50 | FR1 requires three CC while FR2 requires two |
| 125MHz Uplink | 100 + 25 20 + 20 + 20 + 20 + 20 + 20 | 100 | FR2 limited to 100 MHz out of 125 MHz |
| 36,72 MHz legacy bandwidths | 35, 70 | 36 MHz not supported, 50 best fit for 72 MHz | Good match for FR1, no good FR2 solution |

The above table shows that using existing FR1 native channel bandwidths the needs of NTN can be efficiently accommodated. FR2 can handle the 250 MHz downlink but is not a good match for the 125 MHz uplink or the narrower legacy bandwidths of 36 and 72 MHz.

## 2.2	Phase noise

One of the implications of SCS is the implicit requirements for carrier phase noise. It is known that higher frequency systems exhibit more phase noise than lower frequency systems. This factor is mitigated by the wider SCS available in FR2 numerology.

Current FR1 systems with 15 and 30 KHz SCS operate well in the FR1 frequency range below 7.125 GHz. Current FR2 systems with 120 KHz SCS operate well starting at 24.25 GHz. If FR2 is used for Ku band from 10.7 – 14.5 GHz there would be no implications on existing FR2 phase noise performance. The question is whether extending FR1 systems using 15 and 30 KHz SCS up to 14.5 GHz puts unreasonable constraints on carrier phase noise. A mitigating factor to consider is that NTN will be operating at low SNR with lower modulation depth which will mitigate the need for the best phase noise performance.

From a deployment perspective, existing satellite systems using DVB-S with 15 kHz carrier spacing are in operation although this does lead to a need for high quality oscillators in the VSAT. A 30 kHz SCS would be easier to implement. A demonstration of an NTN-NR system in Ku band using FR1 numerology has been shown publicly so the issues may come down to implementation choices rather than feasibility. Phase tracking Reference Symbols (PTRS) have been introduced for FR1 and FR2 and may provide a mechanism to mitigate the impact of phase noise should FR1 be chosen.

## 2.3	UL timing – VSAT GNSS accuracy vs. CP length

A consequence of wider SCS is reduced slot length and lower latency. There is also a corresponding impact on the length of the cyclic prefix. A shorter CP means there is less time available to absorb the uncertainty in the UL timing which is a function of positioning uncertainty and the radio channel dispersion. Initial analysis has shown that the shorter CP from using a 120 kHz SCS might not have sufficient margin to offset the timing uncertainty in NTN systems while a narrower SCS of 60 kHz may be good enough. A thorough analysis of this critical issue is necessary since it could be a defining factor in selecting a reliable SCS for NTN systems, particularly for use with mobile VSAT.

A related issue is the impact of residual Doppler frequency error after pre-compensation. A wider SCS has more tolerance to this.

## 2.4	Beam hopping granularity

The overall capacity of the system with a beam hopping configuration is impacted by the duration of the minimum dwell time per hop, which is determined by slot duration. Higher SCS (as in FR2) have shorter slots enabling more hopping flexibility. Hop re-visit time is determined by a latency requirement. The number of Hex-tiles served per hop cycle = revisit time / min dwell time.

For longer dwells approaching 1ms, capacity with 250MHz bandwidth expected to be much greater than 1UE. Table 3 analyzes beam hopping granularity.

Table 3. Beam hopping granularity

| Aspect | FR1-NTN |  |  | FR2-NTN |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Subcarrier Spacings, kHz | 15 | 30 | 60 | 60 | 120 | 240 | Not all are implemented |
| Slot lengths, µs | 1000 | 500 | 250 | 250 | 125 | 62.5 |  |
| Hops per 5 ms cycle | 5 | 10 | 20 | 20 | 40 | 60 | Number of hex tiles supported per hopping beam |
| % time per dwell | 20% | 10% | 5% | 5% | 2.5% | 1.25% | Indicates finest granularity of a cell’s capacity |

The lower SCSs and longer slot lengths in FR1-NTN mean fewer hops per cycle with correspondingly larger capacity per hop. Capacity modelling needs to quantify the impact on overall system performance.

# 5	Summary

Proposal 1: At least the following technical factors should be considered before deciding whether FR1 and/or FR2 numerology could be applicable for Ku band:

- The possibility using a feasible SCS to meet the channel bandwidth requirements of NTN

- Phase noise

- UL timing synchronization

- Beam hopping granularity

Proposal 2: A thorough analysis of these factors should be conducted before considering other factors such as commercial considerations.

Proposal 3: Companies are encouraged to provide evidence with respect to FR1 and/or FR2 applicability for NTN Ku-band

# 6	References

[1] RP-241690 New WID on Introduction of Ku Band for NR NTN