3GPP TSG-RAN WG4 Meeting #112							 R4-2411483

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Source: 	OPPO

Agenda item:	8.15.2.1

Document for:	Discussion

Introduction

In last meeting, a WF[1] was approved on FR2-1 SSB based L3 measurement delay reduction.

Agreement: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion. Agreement: Only support multi-Rx L3 measurement for CONNECTED UE Agreement: RAN4 is not to change existing measurement performance requirement when consider optimization of Rx BSF in L3 measurement delay reduction.

In this contribution, we will provide discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor.

Discussion

Rx beam sweeping factor

In R18, for both FR2 SCell activation delay reduction (in eFeRRM WI) and FR2 L1 measurement in Multi-Rx operation (in multi-Rx WI), the work for FR2 delay reduction has already started and faster beam sweeping are considered. It is straightforward to leverage the method of fast beam sweeping (e.g., for FR2 multi-Rx reception) for UE supporting multiple-Rx simultaneous reception on single carrier. As agreed, L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable.

Due to possible enhanced UE capabilities and improved procedures, FR2 L3 measurement delay reduction is feasible under some new assumption, e.g., utilizing multi-Rx chain/multi-panel to perform fast beam sweeping.

| 30. NR_FR2_multiRX_DL | [30-2] | Fast beam sweeping for layer-1 measurement | Supports beam sweeping factor reduction for SSB-based layer-1 measurement. | Per band | FR2-1 only | Candidate values for Component 2: {2,4,6} for FR2-1 Note: It is only supported for power class 3. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- |

UE Power class: FFS the following options and moderator option: - Option 1 (Apple, OPPO, LGE, vivo, HW, Samsung): - RAN4 to consider UE supporting FR2-1 power class 3 as first priority. - Option 1a (LGE): - but RAN4 should consider if other power classes could apply the outcome of the WI discussion - Option 2 (CATT): - It is proposed not to restrict the number of configured carriers and applied power class. - Option 3 (Ericsson): - We can have generic requirements for all power classes, maybe PC6 can be precluded, if no specific use cases for some power classes are pursued. - Option 4 (Moderator) - Baseline: RAN4 to consider UE supporting FR2-1 power class 3 as first priority. - Note: whether other power classes could apply the outcome of the WI discussion can be FFS after concluding on PC3. These extra FFS parts will NOT delay the WI completion.

For applicability rule, we support to consider power class 3 as first priority, and the requirements should be defined for UE under multi-Rx operation.

RAN4 to consider UE supporting FR2-1 power class 3 as first priority.

Conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF: - FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature - Option 1 (Apple, CATT, ZTE, vivo, CTC, Intel): multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature. - Option 1a (ZTE): The UE is in multi-Rx operation if following condition is met: UE is configured with group-based beam reporting (GBBR) report - Option 1b (Intel): The UE is considered activated in multi-Rx simultaneous reception mode when the UE is configured with group-based beam reporting. The UE is considered activated for L3 reporting when the GBBR is configured not long prior to the expected L3 reporting. - Option 2 (LGE): The L3 measurement delay enhancement requirements for UE supporting multi-Rx simultaneous reception can apply if multiple panels are activate and SSBs in a SMTC window can be measured with multiple beams. - Option 3 (CMCC): the conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that multi-Rx simultaneous reception of UE is in active mode, as for whether the condition is same as that for Rel-18 multi-Rx simultaneous reception can be further discussed. - Option 4 (HW, Nokia, Ericsson): Do not reuse the same applicable conditions specified in Rel-18 multi-Rx - Option 4a (QC) : R19 Rx BSF is independent to R18 multi-Rx simultaneous reception feature includes conditions and requirement. - Option 5 (MTK): The conditions in R18 multi-Rx WI do not limit the discussion on having further conditions in R19 for UE supporting multi-Rx to enhance FR2-1 SSB based L3 measurement delay

We prefer to follow R18 conditions defined for multi-Rx simultaneous reception feature and support option 1.

multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature.

- FFS: UE’s mobility status, e.g., whether HST is precluded or not - Option 1 (Apple, ZTE(preclude HST), vivo, CTC, Samsung(preclude HST)): low mobility status, i.e., preclude HST - Option 2 (CATT, CMCC): enhanced BSF can also be used for HST - Option 3 (Ericsson): no scenario restriction (e.g., low mobility) is needed - Option 4 (MTK): For UE with high mobility

We also support not to consider the HST scenarios. Since the deployment and UE side conditions are quite specific under HST scenario, the requirements for faster beam sweeping to reduce FR2-1 L3 measurement delay could be different for UE with low mobility status. The normal work on common cases can be in high priority. So, we prefer option 1.

Focus on low mobility case in this WI.

- FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier - Option 1 (Apple): the legacy searcher assumption and legacy CSSF shall still be applied for L3 RRM measurement with two panels activated - Option 1a (Ericsson): - The WI shall prioritize the use case of the same receiver for search and measurement processing on one carrier simultaneously received from multiple panels, e.g. a single searcher receives and processes the same carrier on multiple panels. - Option 2 (vivo, HW): RRM measurement with two panels activated, two searchers are occupied by this single carrier - FFS: SSB processing delay/time for processing multiple beams received in a SMTC - Option 1 (vivo, QC, Intel, Apple): needs SSB processing delay/time for processing multiple beams received in a SMTC

We are open to consider the enhanced searcher assumption for R19 UE. If UE performs RRM measurement with two panels activated, two searchers can be occupied by this single carrier. But the impact on UE baseband and processing needs to be clarified before we agreed on that.

Open to support the enhanced searcher assumption that two searchers are occupied by this single carrier for L3 RRM measurement with two UE’s panels activated.

FFS on following scenarios to decide which ones are considered to use L3 measurement delay reduction by optimizing Rx BSF: - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Option 1(Apple, NTT DCM, CATT, Xiaomi, OPPO, LGE, CMCC, ZTE, CTC, HW, Ericsson): Yes - Option 1a (OPPO): For deactivated SCell and PSCell in FR2-1, the enhancement of TPSS/SSS_sync and TSSB_measurement_period can also apply. - Option 2: No - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Option 1(Apple, NTT DCM, CATT, Xiaomi, OPPO, LGE, CMCC, ZTE, CTC, HW, Ericsson): Yes - Option 2: No - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Option 1(Apple, NTT DCM, CATT, Xiaomi, OPPO, LGE, CMCC, ZTE, CTC, HW, Ericsson): Yes - Option 1a (OPPO): For deactivated SCell and PSCell in FR2-1, the enhancement of TPSS/SSS_sync and TSSB_measurement_period can also apply. - Option 2: No - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Option 1(Apple, NTT DCM, CATT, Xiaomi, OPPO, LGE, CMCC, ZTE, CTC, HW, Ericsson): Yes - Option 2: No - Scenario 5: Handover - Option 1(NTT DCM, CATT, CMCC, ZTE, CTC, Ericsson, MTK): Yes - Option 2(vivo): No - Option 3 (Apple, Xiaomi, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 6: PSCell addition - Option 1(NTT DCM, ZTE, MTK): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection - Option 1(NTT DCM, ZTE): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 8: SCell activation - Option 1(MTK): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 9: SCG activation - Option 1(ZTE, MTK): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario - Scenario 10: CGI identification - Option 1 (NTT DCM, ZTE): Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario. - Scenario 11: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB. The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded. - Option 1: Yes - Option 2(vivo): No - Option 3 (Apple, CATT, OPPO): After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios 1/2/3/4, the solutions(s) can be extended to this scenario

We prefer RAN4 to define the requirements of SSB based intra-frequency and inter-frequency measurements, and corresponding conditions for faster beam sweeping to reduce FR2-1 L3 measurement delay. Taking FR2 intra-frequency L3 measurements as example, the legacy M considers the beam sweep factor as 8 for different UE power classes, including Mpss/sss_sync_w/o_gaps, and Mmeas_period_w/o_gaps. Due to the signaling deriveSSB-IndexFromCell is always enabled for FR2-1, MSSB_index_intra is not considered.

For other procedures including HO, Scell activation and SCG activation, we think the logic of L3 measurement on neighbour cells can be reused. We can firstly define requirements of SSB based intra-frequency and inter-frequency L3 measurements with fast beam sweeping, and then decide whether to apply them for other L3 procedures.

RAN4 firstly to define requirements of SSB based intra-frequency and inter-frequency L3 measurements with fast beam sweeping, and then decide whether to apply them for other L3 procedures.

If RAN4 can agree on reduction of L3 measurements delay on neighbour cells, the enhancement can also apply to deactivated SCell and PSCell in FR2-1.

For deactivated SCell and PSCell in FR2-1, the enhancement of TPSS/SSS_sync and TSSB_measurement_period can also apply.

Conclusions

In this paper we provided our views on FR2-1 SSB based L3 measurement delay reduction for optimizing BSF. The followings are provided.

Proposal 1: RAN4 to consider UE supporting FR2-1 power class 3 as first priority.

Proposal 2: Multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature.

Proposal 3: Focus on low mobility case in this WI.

Proposal 4: Open to support the enhanced searcher assumption that two searchers are occupied by this single carrier for L3 RRM measurement with two UE’s panels activated.

Proposal 5: RAN4 firstly to define requirements of SSB based intra-frequency and inter-frequency L3 measurements with fast beam sweeping, and then decide whether to apply them for other L3 procedures.

Proposal 6: For deactivated SCell and PSCell in FR2-1, the enhancement of TPSS/SSS_sync and TSSB_measurement_period can also apply.

Reference

- R4-2410260, Way Forward of [111][228]NR_RRM_Ph5, Apple