3GPP TSG-RAN WG4 Meeting #112							R4-2411484

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on FR2-1 L3 measurement delay reduction by optimizing CSSF

Source: 	OPPO

Agenda item:	8.15.2.2

Document for:	Discussion

Introduction

In last meeting, a WF[1] was approved on FR2-1 SSB based L3 measurement delay reduction.

For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2

In this contribution, we will provide discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSF.

Discussion

CSSFoutside_gap

- Issue 2-2-1: Clarification on the bullets in WID for this CSSF optimization FFS: - Option 1 (Apple): Rel-19 CSSF optimization applies for the both cases: (1)UE is not capable of Rel-18 multi-Rx simultaneous reception, (2)UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently. - Option 2 (ZTE): Besides the case UE is not capable of R18 multi-Rx simultaneous reception, R19 CSSF optimization is also applied to the case that UE is capable of multi-Rx but not configured with GBBR report. - Option 3 (Ericsson): Clarify whether ‘the case UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently’. - Does it indicate that the UE only can use a single panel (and subsequent receiver including baseband) out of multiple panels for reception and measurement? If so, is there any degradation of reception and measurement from measuring on a single panel? - Option 4 (Nokia): Rel-19 discussion on the scenarios for CSSF optimization will be considered in CA/DC scenarios, independently of the UE support of multi-Rx capabilities.

About the clarification on “UE not in multiple-Rx simultaneous reception mode”, we support option 2. Case 1 is UE is not capable of R18 multi-Rx simultaneous reception, and Case 2 is that UE is capable of multi-Rx but not configured with GBBR report.

About the clarification on “UE not in multiple-Rx simultaneous reception mode”, consider both Case 1 that UE is not capable of R18 multi-Rx simultaneous reception, and Case 2 that UE is capable of multi-Rx but not configured with GBBR report.

- Issue 2-2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF FFS: - Option 1: the following scenarios in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be prioritized: - SSB based Intra-frequency measurement without MG (Apple, CATT, CMCC, vivo, CTC, HW, Nokia, MTK) - Alt1: including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Alt2: including TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra - SSB based Inter-frequency measurement without MG, (Apple, CATT, CMCC, vivo, CTC, HW, Nokia, MTK) - Alt1: including TPSS/SSS_sync_inter and TSSB_measurement_period_inter - Alt2: including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Inter-RAT SSB measurement without MG (CMCC, Ericsson) - NeedForGaps measurement without MG, including both with and without interruption (Ericsson) - NCSG measurement without MG without interruption (Ericsson)

In current spec, CSSFintra and CSSFinter are determined according to CSSFoutside_gap,i for measurement conducted outside measurement gaps. We support to consider both intra-frequency and inter-frequency measurement without MG.

Consider both intra-frequency and inter-frequency measurement without MG for L3 measurement delay reduction by optimizing CSSF.

Take CSSFoutside_gap,i for SA mode as example, the CSSFs are defined for FR2 carriers including:

- CSSFoutside_gap,i for FR2 PCC

- CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required

- CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required

- CSSFoutside_gap,i for inter-frequency MO with no measurement gap

- CSSFoutside_gap,i for E-UTRA inter-RAT MO with no measurement gap

For the case of FR2 only intra-CA or FR2 only inter-band CA, FR2 PCell occupies one searcher and all neighbor FR2 Scell share another searcher. CSSFoutside_gap,i for FR2 SCC equals to NSCC_SSB +Y+Z+2* NSCC_CSIRS regardless neighbor cell measurement is required or not, where

NSCC_SSB equals to the number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG.

Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0.

Z is the number of configured E-UTRA inter-RAT MOs without MG that are being measured outside of MG; otherwise, it is 0.

NSCC_CSIRS equals to the number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured

The candidate optimization could be done by reduce the components of CSSFoutside_gap,i. RAN4 needs to discuss the conditions of counting the number of configured SCell(s) or MOs without MG, because there may exist overlapping or close frequency layers for different Scell(s) or MO(s). By the way, inter-frequency MOs and E-UTRA inter-RAT MOs may share the same frequency layer and can be measured simultaneously.

RAN4 to discuss the optimization of counting the number of configured SCell(s) or MOs without MG for CSSFoutside_gap in FR2 L3 measurements.

Conclusions

In this paper we provided our views on FR2-1 SSB based L3 measurement delay reduction for connected mode. The followings are provided.

- Proposal 1: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx About the clarification on “UE not in multiple-Rx simultaneous reception mode”, consider both Case 1 that UE is not capable of R18 multi-Rx simultaneous reception, and Case 2 that UE is capable of multi-Rx but not configured with GBBR report.

Proposal 2: Consider both intra-frequency and inter-frequency measurement without MG for L3 measurement delay reduction by optimizing CSSF.

- Proposal 3: RAN4 to discuss the optimization of counting the number of configured SCell(s) or MOs without MG for CSSFoutside_gap in FR2 L3 measurements.

Reference

- R4-2410260, Way Forward of [111][228]NR_RRM_Ph5, Apple