3GPP TSG-RAN WG4 Meeting #112							  R4-2411485

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on RRM requirements of R19 NES

Source: 	OPPO

Agenda item:	8.21.2

Document for:	Discussion

Introduction

In this contribution, we will provide discussion on RRM requirements for Rel-19 NES. The followings are the objectives in the WID for information.

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

Discussion

According to the WID, 3 objectives could be related to RAN4 work. Due to on-demand SIB1 and adaptive SSB/RACH/Paging depend on RAN1/2 progress, RAN4 can first focus to specify the requirements for on-demand SSB(OD-SSB) SCell operation.

- On-demand SSB SCell operation, for RRC_CONNECTED

- On-demand SIB1, for RRC_IDLE/RRC_INACTIVE

- Adaptive SSB/RACH/Paging

Proposal 1: RAN4 starts from Objective#1 to define RRM requirements for on-demand SSB SCell operation.

Objective#1: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

About the scenarios of OD-SSB SCell operation, the followings can be considered for UE in connected mode configured with CA. For beam failure detection on SCell, since RS resource configuration in the set on SCell shall be periodic CSI-RS, BFD is out of scope for OD-SSB SCell operation.

  - L3 measurement on deactivated Scell

  - Scell activation

Thus, we prefer to consider to study 2 scenarios for R19 OD-SSB: deactivated Scell measurement, and Scell activation requirement.

Proposal 2: RAN4 to consider OD-SSB based L3 measurement on deactivated Scell and Scell activation.

As listed below, different feature/requirements related to SCell operation are introduced in previous releases. We are also ok to start from Rel-15 Scell activation as the baseline to define OD-SSB based Scell activation requirement.

| Release | The feature/requirements related to SCell operation |
| --- | --- |
| Rel-15 | Single Scell activation |
| Rel-16 | Multiple Scell activation Direct Scell activation |
| Rel-17 | PUCCH Scell activation TRS based fast Scell activation |
| Rel-18 | enhanced FR2 Scell activation SSB-less inter-band co-located Scell activation |

Proposal 3: RAN4 to define OD-SSB based Scell activation requirement based on Rel-15 Scell activation as the baseline.

RAN1 has agreed two cases including No always-on SSB and Always-on SSB periodically transmitted. RAN4 should consider both for the known and unknown SCell.

Proposal 4: RAN4 to consider Not-always-on SSB and Always-on SSB for the known and unknown SCell.

RAN1 and RAN2 are still discussing the signaling method, which may have impact on RAN4 requirements. Since this is the first RAN4 meeting, we can discuss all possible procedures of OD-SSB based SCell activation, including OD-SSB indication before/along with/after SCell activation command.

Proposal 5: Wait for RAN1/2’s conclusion on signaling method(s) to support OD-SSB based SCell activation.

Conclusions

In this paper we provided our views.

Proposal 1: RAN4 starts from Objective#1 to define RRM requirements for on-demand SSB SCell operation.

Proposal 2: RAN4 to consider OD-SSB based L3 measurement on deactivated Scell and Scell activation.

Proposal 3: RAN4 to define OD-SSB based Scell activation requirement based on Rel-15 Scell activation as the baseline.

Proposal 4: RAN4 to consider Not-always-on SSB and Always-on SSB for the known and unknown SCell.

Proposal 5: Wait for RAN1/2’s conclusion on signaling method(s) to support OD-SSB based SCell activation.

Reference

- RP-240170, Revised WID for Enhancements of network energy savings for NR