3GPP TSG-RAN WG4 Meeting #112	R4-2411488

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	6.1.2.2

Source: 	OPPO

Title: 	Discussion on performance requirements for SL positioning

Document for:	Approval

1	Introduction

As per the conclusions reached in previous meeting [1], this contribution will provide our further considerations on the performance requirements for SL positioning.

2	Discussion

### Issue 1-2-2: Margin for RF calibration Agreements: - Define the RF calibration margin for SL RSTD measurements in FR1 using the following structure: PRS BW (RB number); Margin (Tc). SCS=15kHz; SCS=30kHz; SCS=60kHz. ≥ 48; ≥ 24; N/A; Z1. ≥ 96; ≥ 48; ≥ 24; Z2. - Define the RF calibration margin for SL Rx-Tx measurements in FR1 using the following structure: [Min(SL PRS Rx BW, SL PRS Tx BW) (RB)]; Margin (Tc). SCS = 15 kHz; SCS = 30 kHz; SCS = 60 kHz. ≥ 48; ≥ 24; N/A; 1. ≥ 96; ≥ 48; ≥ 24; 2. - FFS the detailed values in the above tables. Margins due to frequency drift (Y): - For SL RSTD, the existing margins due to frequency drift for DL PRS can be reused for SL PRS. - Consider time margins due to frequency drift to Rx-Tx time difference accuracy requirement if the UE reports the transmission timestamp of a SL PRS measurement - FFS the exact time margin values, e.g., the ones for RSTD could be reused.

Table 10.1.23.2-5: Margin for RSTD measurement accuracy in FR1

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 24 | N/A | N/A | 120 |
| ≥ 52 | ≥ 24 | N/A | 72 |
| ≥ 104 | ≥ 48 | ≥ 24 | 36 |
| N/A | ≥ 132 | ≥ 64 | 16 |
| N/A | N/A | ≥ 132 | 12 |

Table 10.1.25.2-5: Margin for UE Rx-Tx time difference measurement accuracy in FR1

| Min(PRS BW, SRS BW) (RB) |  |  | Margin (Tc Note 1) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 24 | N/A | N/A | 160 |
| ≥ 52 | ≥ 24 | N/A | 80 |
| ≥ 104 | ≥ 48 | ≥ 24 | 56 |
| N/A | ≥ 132 | ≥ 64 | 24 |
| N/A | N/A | ≥ 132 | 24 |
| NOTE 1: Tc is the basic timing unit defined in TS 38.211 [6]. NOTE 2: If SRS and PRS have different SCS, the margin corresponding to the smallest RS BW in MHz applies. |  |  |  |

As shown above, margins for RF calibration and frequency drift are agreed to be introduced for SL RSTD and Rx-Tx timing difference accuracy requirements. While, the exact margin values are left for further discussion. In our view, there is no much difference between DL UE and SL UE from the perspective of RF calibration. Although the supported bandwidths (in the unit of RB) for DL PRS and SL PRS are slightly different, we think the highlighted rows in the tables above could be considered as the starting point to define the RF calibration margins for SL PRS measurements, that is

Proposal 1: The RF calibration margins for SL RSTD and Rx-Tx measurements can be defined as the tables below.

  - The RF calibration margin for SL RSTD measurements in FR1:

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 48 | ≥ 24 | N/A | Z1=72 |
| ≥ 96 | ≥ 48 | ≥ 24 | Z2=36 |

  - The RF calibration margin for SL Rx-Tx measurements in FR1:

| [Min(SL PRS Rx BW, SL PRS Tx BW) (RB)] |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 48 | ≥ 24 | N/A | 1=80 |
| ≥ 96 | ≥ 48 | ≥ 24 | 2=56 |

Based on TS 38.101, the same frequency drift is assumed for DL and SL receivers. And the existing margins due to frequency drift for DL PRS are agreed to be reused for SL RSTD in the last meeting. Therefore, the same margins for RSTD can also be reused for SL Rx-Tx time measurements.

Proposal 2: The existing frequency drift margins for RSTD can be reused for SL Rx-Tx measurements.

### Issue 1-2-1: SINR side conditions Agreements: - SL RSTD: (0, -3) dB - SL Rx-Tx/RSRP/RSRPP: -3 dB

### Issue 1-2-7: Whether to verify accuracy of legacy measurements in SL PRS-RSRP(P) TCs Proposals: - Option 1: - Consider the following rules: - If RAN1/2 confirms that the SL PRS-RSRP(P) is possible to be requested and/or reported standalone, the SL PRS-RSRP(P) accuracy is verified alone in separate section. - If RAN1/2 confirms that the SL PRS-RSRP(P) cannot be requested and/or reported standalone, the SL PRS-RSRP(P) accuracy should be verified with RSTD/RX-TX accuracy together in one section. - Option 2: - Verify accuracy requirements for SL PRS-RSRP together with SL Rx-Tx in the same section, but different test cases with and without SL PRS-RSRP. - Define test case to verify accuracy requirements for SL PRS-RSRPP in a separate section. - Option 3: - Support option 1a to define separate sections for testing SL PRS-RSRP/PRS-RSRPP, without verifying the accuracy of the other (SL RSTD/Rx-Tx) measurement, respectively.

The next remaining issue is whether to define separate section for testing SL PRS RSRP/RSRPP accuracy requirements. Some companies believe this issue depends on the RAN1/2 conclusions and RAN4 already sent a LS to ask their opinions. In this meeting, their reply LSs are received as shown below and both RAN1 and RAN2 confirm that SL RSRP/RSRPP cannot be request and/or reported standalone without any other SL positioning measurement. From this perspective, we are supportive to verify SL RSRP/RSRPP together with RSTD/Rx-Tx in the same section. As agreed in issue 1-2-1 that the same SINR side condition is defined for SL Rx-Tx/RSRP/RSRPP measurements except SL RSTD measurement, it is better to verify both Rx-Tx and SL RSRP accuracy. However, a dedicated test case is needed for SL RSRPP considering the different propagation channel types. The two-tap channel model is used for defining SL RSRPP accuracy while AWGN/TDL channel model is considered for RSTD/RSRP/Rx-Tx time difference measurement.

Proposal 3: Verify accuracy requirements for SL PRS-RSRP together with SL Rx-Tx in the same section, but different test cases with and without SL PRS-RSRP.

Proposal 4: Define test case to verify accuracy requirements for SL PRS-RSRPP alone in a separate section.

### RAN1 reply in R1-2405511: Regarding the following question raised by RAN4 • whether it is possible to request and/or report only SL PRS-RSRP or SL PRS-RSRPP for Rel-18 NR SL positioning in general.. RAN1 confirms requesting and/or reporting only SL PRS-RSRP or SL PRS-RSRPP without any other SL positioning measurements is not supported.

### RAN2 reply in R2-2405872: Hence, regarding the following question from RAN4, RAN4 respectfully asks RAN2 to clarify: - whether SL PRS-RSRP or SL PRS-RSRPP can be requested and/or reported standalone specifically for SL-AoA positioning.. RAN2 agreed that SL PRS-RSRP or SL PRS-RSRPP cannot be requested and/or reported standalone for SL-AoA positioning.

3	Conclusion

Based on latest progress, the remaining RRM performance requirements for SL positioning are discussed and the following proposals are given:

Proposal 1: The RF calibration margins for SL RSTD and Rx-Tx measurements can be defined as the tables below.

  - The RF calibration margin for SL RSTD measurements in FR1:

| PRS BW (RB number) |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS=15kHz | SCS=30kHz | SCS=60kHz |  |
| ≥ 48 | ≥ 24 | N/A | Z1=72 |
| ≥ 96 | ≥ 48 | ≥ 24 | Z2=36 |

  - The RF calibration margin for SL Rx-Tx measurements in FR1:

| [Min(SL PRS Rx BW, SL PRS Tx BW) (RB)] |  |  | Margin (Tc) |
| --- | --- | --- | --- |
| SCS = 15 kHz | SCS = 30 kHz | SCS = 60 kHz |  |
| ≥ 48 | ≥ 24 | N/A | 1=80 |
| ≥ 96 | ≥ 48 | ≥ 24 | 2=56 |

Proposal 2: The existing frequency drift margins for RSTD can be reused for SL Rx-Tx measurements.

Proposal 3: Verify accuracy requirements for SL PRS-RSRP together with SL Rx-Tx in the same section, but different test cases with and without SL PRS-RSRP.

Proposal 4: Define test case to verify accuracy requirements for SL PRS-RSRPP alone in a separate section.

4	Reference

- R4-2410193, WF on SL positioning and carrier phase positioning, CATT

- R1-2405511, Reply LS on SL positioning measurement, RAN1, Huawei

- R2-2405872, Reply LS on SL positioning measurement, RAN2, Huawei