pos3GPP TSG-RAN WG4 Meeting #112	R4-2411492

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda Item:	6.1.2.6

Source: 	OPPO

Title: 	Discussion on performance requirements for carrier phase positioning

Document for:	Approval

1	Introduction

As per the conclusions reached in previous meeting [1], this contribution will provide our further considerations on the performance requirements for carrier phase positioning.

2	Discussion

### Issue 2-2-1: Whether to verify the accuracy of legacy measurements in RSCPD/RSCP TCs Proposals: - Option 1: - Verify both the accuracies of legacy measurements and CPP measurements in one TC with a 90% success rate to reflect UE’s real positioning performance in the deployment. - Option 2: - Not verify the accuracy requirements for legacy RSTD/Rx-Tx measurement in the RSCPD/RSCP TC.

We do not support to verify the legacy RSTD/Rx-Tx requirements in the RSCPD/RSCP TC. Firstly, the accuracy requirements for RSTD/Rx-Tx measurements have already been verified by the existing test cases and it is not necessary to verify them again. Secondly, if both RSCPD/RSCP and legacy RSTD/Rx-Tx are verified, the passing rate will be lower.

Proposal 1: Not verify the accuracy requirements for legacy RSTD/Rx-Tx measurement in the RSCPD/RSCP TC.

### Issue 2-2-2: Additional margins for frequency drift and RF calibration Proposals: - Option 1: Define extra margin in requirements - FFS: need for extra simulations. - Option 2: Do not define extra margin in requirements - Do not update simulation assumptions for carrier phase measurement. Further discuss in the next meeting, proponents are requested to bring results justifying the additional margin for Option 1.

Similar as RSTD, the accuracy requirements for carrier phase measurement is highly impacted by the time/frequency synchronization performance. Therefore, the frequency drift between two PRS resources to estimate a single RSCPD or a relative RSCP will introduce additional error. To handle it, additional margins due to frequency drift are needed. As for the RF calibration margin, we think it should be considered as well since error between RF and baseband processing units exists for all types of measurements including RSRP, timing and carrier phase.

It is hard to simulate the exact values for the measurement margins. For simplicity, we can keep the current simulation assumptions and add the additional margins on top of the simulation results.

Proposal 2: Define extra margins for frequency drift and RF calibration, and no extra simulation is needed.

3	Conclusion

Based on latest progress, RRM performance requirements for carrier phase positioning are discussed and the following proposals are given:

Proposal 1: Not verify the accuracy requirements for legacy RSTD/Rx-Tx measurement in the RSCPD/RSCP TC.

Proposal 2: Define extra margins for frequency drift and RF calibration, and no extra simulation is needed.

4	Reference

- R4-2410193, WF on SL positioning and carrier phase positioning, CATT