3GPP TSG-RAN WG4 Meeting #112	R4-2411493

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.22.4.2

Source:	OPPO

Title:	Discussion on core requirements for LP-WUS/WUR

Document for:	Approval

- Introduction

RAN4 #111 meeting discussed and reached some agreements regarding LP-WUS/WUR in [1]. This contribution will provide our further considerations on the open issues captured in [2].

- Discussion

Issue 1-1-2: Cases/states to be considered for RRM relaxation Agreement: Discuss the RAN4 requirements first for the following case #1, and FFS for case #2 to #5. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

As shown above, 4 cases were proposed during the last meeting and only case #1 was agreed. The FFS part for case #1 is the condition and details to turn off neighboring cell measurement by MR. The main concern is that UE will miss a neighbor cell with higher priority if configured. In the first stage, we can focus on the scenario without inter-frequency with higher priority configured and then the FFS part in case #1 can be removed. If no inter-frequency with higher priority is configured, there is no need to turn on MR for neighboring cell measurement only in case #4 either. Following the same principle, the whole case #4 could be deprioritized.

Proposal 1: Focus on the scenario when inter-frequency with higher priority is not configured, and then the FFS part in case #1 can be removed and the whole case #4 could be deprioritized.

For the other two relaxed cases, case #3 is more reasonable and important in our view. Firstly, if the UE already turns on the MR for serving cell measurement, it could also perform neighboring cell measurement with MR without much extra power consumption. Secondly, working in relaxed cases rather than fully offloading case #1 means that the serving cell quality is not good enough and is likely to further deteriorate, e.g. does not meet the criterion S any longer. To make sure the UE could reselect to the suitable neighboring cell in time, even with equal or lower priority, neighboring cell measurement is necessary. Therefore, we propose to prioritize case #3 over case #2.

Proposal 2: Case #3 should be prioritized over case #2.

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation - Proposals - P1: RAN4 specifies MR relaxation requirements for both serving cell and neighbor cell measurements at idle/inactive mode for UE supporting LP-WUR. (Apple xiaomi oppo LG CT ZTE) - RAN4 can investigate the mobility performance to quantify the relaxation, e.g., scaling factor for measurement period. If both LP-WUR and MR are ON, RAN4 to discuss whether UE uses LP-WUR measurement to decide MR relaxation or UE uses both LP-WUR and MR measurement to decide MR relaxation (Apple) - P1-1: RAN4 to specify MR RRM relaxation requirements for serving cell measurements in Idle/Inactive mode when (MTK) - LP-SS based LP-WUR RRM measurement is activated, or - PSS/SSS based LP-WUR RRM measurement is activated Not to consider RRM relaxation based on the hybrid measurements of both (LP-SS and SSS).

Based on the WID [3], further RRM relaxation for both serving cell and neighbor cells using MR should be specified. It is straightforward to support P1 firstly. As for the whether MR RRM relaxation is supported when LP-WUR RRM measurement is activated or how to use the measurement results, can be further discussed.

Proposal 3: Specify MR relaxation requirements for both serving cell and neighbor cell measurements in RRC idle/inactive mode.

Issue 1-1-5: Criteria (entry/exit conditions) for LP-WUR measurement - Proposals - P1: RAN2 is be the main group for criteria (entry/exit conditions) design (Apple oppo vivo) - P2: RAN4 shall lead the discussion regarding criterion or scenario (entry/exit condition) for LP-WUR measurement and MR RRM measurement relaxation with the involvement of RAN1 and RAN2 (Samsung) - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package - It’s not precluded multiple states can be introduced with different assumption on side condition - P3: RAN4 to firstly discuss the differences between entry/exit conditions for LP-WUS monitoring defined in RAN1/RAN2 and conditions for LP-WUR measurement. RAN4 should at least consider LP-SS and PSS/SSS when discussing the entry/exit conditions for LP-WUR measurement. RAN4 to wait for RAN1/RAN2, if the entry/exit conditions for LP-WUS monitoring are introduced by RAN1/RAN2, the thresholds on serving cell measurement result can be further discussed in RAN4. (CATT) - P4: Both serving cell quality and UE mobility should be considered for the criteria(entry/exit conditions) for MR RRM measurement relaxation &LP-WUR measurement (State2) and only LP-WUR measurement (State 3). Whether network configure the high priority frequency carrier measurement or not should also be considered when defining the criteria (entry/exit conditions). (CMCC) - P5: When UE enters the State 3 (Measurement only based on LP-WUR), The serving cell quality should at least be Srxlev > SIntraSearchP and Squal > SIntraSearchQ (CMCC) - P6: Study other conditions for the entry/exit condition for LP-WUS monitoring other than the measurement based condition from the RAN4 pespective (LG) - P7: RAN4 to discuss serving cell evaluation for entry/exit conditions for PSS/SSS and LP-SS based LR. PSS/SSS based entry/exit evaluation can be prioritised. (Nokia)

Issue 1-1-6: Criteria (entry/exit conditions) for MR RRM measurement relaxation - Proposals - P1: RAN2 is be the main group for criteria (entry/exit conditions) design (oppo vivo Apple) - RAN4 can work on the RRM measurement relaxations (e.g., Scaling factor) and offloading mechanisms based on the criteria defined by RAN2. (Apple) - P2: RAN4 should firstly discuss the meaning of MR RRM measurement relaxation on which phases and what purposes are included for RRM relaxation in the entire procedure, i.e. for cell selection/reselection or handover, for serving cell measurement or neighbor cell measurement, etc. (CATT) - P3: RAN4 to discuss and provide input to RAN2 such as possible serving/neighbour measurement relaxzation conditions or which receiver (i.e., MR or LP-WUR) to base it on for MR based RRM measurement relaxation (LG) - P3a: RAN4 shall lead the discussion regarding criterion or scenario (entry/exit condition) for LP-WUR measurement and MR RRM measurement relaxation with the involvement of RAN1 and RAN2 (Samsung) - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package - It’s not precluded multiple states can be introduced with different assumption on side condition

Issue 1-1-7: Criteria (entry/exit conditions) for LP-WUS monitoring - Proposals - P1: RAN4’s involvement on entry/exit conditions for LP-WUS monitoring can be triggered by other groups if necessary (Apple) - P2: For Entry/exit condition for LP-WUS monitoring, the performance of miss detection rate on LP-WUS can be considered as side condition in addition to RSRP and RSRQ (Samsung) - P3-1: The entry/exit conditions for LP-WUR measurement should be consistent with the entry/exit conditions LP-WUS monitoring (xiaomi) - P3-2: The entry/exit condition for LP-WUS monitoring and the entry/exit condition for LP-WUR measurement should be considered jointly.(CMCC) - P3-3: Same criteria and same/different entry/exit conditions can be used for LP-WUS monitoring and LP-WUR RRM measurement. (vivo) - P3-4: RAN4 to confirm that same entry/exit condition for LP-WUS monitoring applies for serving cell RRM measurement offloading (from MR to LP-WUR). (HW) Notes: MR measurements results shall be larger than the threshold and satisfy the related MR relaxation conditions. (ZTE) - P4: If there are other conditions that should be considered such as UE mobility besides serving cell RRM measurement for measurement offloading from MR to LP-WUR, RAN4 needs to provide input to RAN2 (LG) - P5: For the entry/exit conditions for LP-WUS monitoring, the threshold for the serving cell measurement performed by MR or LR shall be decided by RAN4 (ZTE)

Companies’ views on the criteria (entry/exit conditions) for LP-WUR measurement, MR RRM relaxation and LP-WUR monitoring are provided and summarized in the last meeting. To our knowledge, these issues are also discussed in RAN2/RAN1 groups and the details should be up to their decision. What RAN4 needs to discuss now is to specify all the necessary cases under issue 1-1-2 and their relationship so that the other groups could work on the criteria.

Proposal 4: The criteria (entry/exit conditions) for LP-WUR measurement, MR RRM relaxation and LP-WUS monitoring should be determined by RAN2/RAN1.

Issue 1-2-3: Accuracy for LP-WUR measurement - Proposals - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (Apple xiaomi LG Ericsson Huawei MTK ZTE) - P2: RAN4 to use the legacy measurement accuracy for CONNECTED mode in Clause 10.1.2 TS 38.133 as baseline. FFS on whether or how to define dedicated accuracy requirements. (CATT CMCC) - P3: RAN4 to consider a common target accuracy when defining LP-SS based and PSS/SSS based RRM delay requirements for LP-WUR. (xiaomi) - P4: First determine the measurement accuracy based on the legacy RAN4 requirement and target SNR/SINR based on RAN1 Rel-19 study. Determine required measurement samples based on RAN4 simulations. Determine measurement delay based on sampling number and reference signal periodicity. RAN4 to decide whether the legacy EMR Idle mode accuracy requirement and Connected mode accuracy requirement can be reused for LP-WUR measurements later based on the simulation results (CT) - P5: RAN4 to define the dedicated accuracy requirement in the performance section (CT) - P6: The accuracy requirement defined for Redcap with 1Rx can be used as the base when defining requirements for LP-WUR serving cell measurement. (vivo) - P6-1: Further relaxation on the accuracy target based on reference accuracy maybe needed. The amount of relaxation could be based on RAN4’s simulation outcome. (vivo)

For RRM measurements in RRC idle/inactive mode, no measurement report is introduced and the measurement results are used by the UE itself to determine cell reselection and RRM relaxation. Due to the testability issue, RAN4 does not define accuracy requirement for RRC measurements in RRC idle/inactive mode, except for EMR case. Similarly, no dedicated accuracy requirement for LP-WUR based RRM measurement should be defined. As for how to reflect the accuracy performance, defining a margin in the core requirements is acceptable for us and the details can be further discussed once the majority issues are solved.

Proposal 5: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in RRC idle/inactive mode.

Issue 1-2-4: Measurement metrics - Proposals - P1: Follow RAN1 agreement that LP-RSRP for OOK-based LP-WUR should be used, wait RAN1’s further progress on LP-RSRQ. (oppo CMCC Ericsson ZTE) - P2: For the definition of LP-RSSI, suggest to use the linear average of total received power in all LP-SS OOK symbols. (CMCC)

Agreement (in RAN1 #116bis) From RAN1 perspective, at least the following metrics can be supported for RRM serving cell measurement performed by OOK-based receiver based on LP-SS: - LP-RSRP - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols. - FFS: How to determine the received power of LP-SS in OOK ON symbols - LP-RSRQ - LP-RSRQ = LP-RSRP/LP-RSSI - For the definition of LP-RSSI for determination of LP-RSRQ, further consider the following options: - Option 1: LP-RSSI is the linear average of total received power in all LP-SS OOK symbols. - Option 2: LP-RSSI is the linear average of total received power in LP-SS OOK OFF symbols. - Option 3: LP-RSSI is the linear average of total received power in LP-SS OOK ON symbols. - FFS: LP-SINR, Power ratio of OOK-ON symbol and OOK-OFF symbol Note: RAN1 will send an LS to RAN2 and RAN4 on the measurement metrics that can be supported from RAN1 perspective, to facilitate RAN2/RAN4 discussions. The exact metrics for OOK-based receiver to be used and defined in the specifications depend on the outcome of [RAN1]/RAN2/RAN4 discussions.

Agreement (in RAN1 #117) For LP-RSSI definition for LP-RSRQ, down-select between the following two options: - Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols. Note: Above does not constrain LP-SS sequence design for OOK.

As mentioned before, the measurement metric like RSRP/RSRQ is considered when defining the entry/exit conditions of LP-WUR measurement. Considering the newly introduced OOK waveform, RAN1 agreed to support at least LP-RSRP/LP-RSRQ based on OOK symbols as measurement metrics in RAN1 #116bis meeting. And the LP-RSSI definition for LP-RSRQ is further determined in RAN1 #117 meeting. If no technology issue is identified, the agreed LP-RSRP/LP-RSRQ should be considered in RAN4.

Proposal 6: Follow RAN1 agreement that at least LP-RSRP/LP-RSRQ for OOK-based LP-WUR should be considered as measurement metrics.

Issue 1-5-1: LP-WUR at CONNECTED mode - Proposals - P1: No RRM objectives is for connected mode in this WI. (Apple Docomo oppo LG CT vivo ZTE) - P2: Postponed until more progress (Ericsson) - P3: FFS whether need to introduce LP-WUS monitoring activation and deactivation delay requirements pending on RAN2/RAN1 progress (Samsung)

WID objective: - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

Based on the WID objective as excerpted above [3], MR ultra-deep sleep is not considered and the RRM/RLM/BFD/CSI measurements are supposed to be performed by MR in RRC connected mode. With this guidance, at least we can agree on that no impact is needed for the existing measurement delay and accuracy requirements, including RRM/RLM/BFD/L1-RSRP. Whether/how to define requirement for LP-WUS monitoring activation and deactivation delay or other procedure can be further discussed or trigged by RAN1/RAN2 conclusions at a later stage.

Proposal 7: No impact on the existing RRM/RLM/BFD/L1-RSRP measurement requirements in RRC connected mode.

- Conclusion

This contribution gave the following proposals on the core requirements for LP-WUR/WUS.

Proposal 1: Focus on the scenario when inter-frequency with higher priority is not configured, and then the FFS part in case #1 can be removed and the whole case #4 could be deprioritized.

Proposal 2: Case #3 should be prioritized over case #2.

Proposal 3: Specify MR relaxation requirements for both serving cell and neighbor cell measurements in RRC idle/inactive mode.

Proposal 4: The criteria (entry/exit conditions) for LP-WUR measurement, MR RRM relaxation and LP-WUS monitoring should be determined by RAN2/RAN1.

Proposal 5: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in RRC idle/inactive mode.

Proposal 6: Follow RAN1 agreement that at least LP-RSRP/LP-RSRQ for OOK-based LP-WUR should be considered as measurement metrics.

Proposal 7: No impact on the existing RRM/RLM/BFD/L1-RSRP measurement requirements in RRC connected mode.

References

[1] R4-2410296, WF for RRM core requirements for LP-WUS/WUR, vivo, RAN4 #111

[2] R4-2408026, Topic summary for [111][229] NR_LPWUS, vivo, RAN4 #111

[3] RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony, RANP #103