3GPP TSG-RAN WG4 Meeting#112		R4-2411494

Maastricht, the Netherlands, Aug 19th -23rd, 2024

Agenda item:	8.22.2.2

Source: 	Spreadtrum Communications

Title: 	Discussion on RX requirements of REFSENS for LP-WUR

Document for:	Discussion

Introduction

In RAN4#111 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

REFSENS requirements

For below 6GHz, the REFSENS level can be calculated by the equation below.

Sensitivity (dBm) = -174dBm (KT)+NF+10log (RX BW)-Diversity gain+ SNR+IM

IM value for REFSENS

As we know, we need to consider the Implementation Margin (IM) because handheld UE/CPE and so on terminals have so many antenna combinations. Now existing NR UE use 2.5dB for IM value. In our understanding, LP-WUR also is applied in handheld UE and other form factor terminals but still have so many antenna combinations. Thus, we could reuse 2.5dB IM value for LP-WUR. In addition, there is no obvious difference between OOK-based and OFDM-based receivers with regard to antenna combinations.

Proposal 1: IM value could use 2.5dB for OOK-based and OFDM-based receivers.

NF GAP between LR and MR for REFSENS (assume MR as 9dB)

Agreement: - RAN4 further discuss potential NF for OOK-based and OFDM-based receiver. - Linked to resolution of Issue 2-2-8

As to noise figure (NF), we could refer to NF part in RAN1 evaluation. RAN1’s conclusion in RAN1#116bis about NF is that all three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported. Based on the above information, NF can be define as delta NF on top of MR. I.e., OOK based LP-WUS is + 8 dB and OFDM based LP-WUS is +2 dB on top of MR but NF of MR is 9dB.

Proposal 2: NF can be defined as delta NF on top of MR (e.g., OOK based LP-WUS is + 8 dB and OFDM based LP-WUS is +2 dB on top of MR (assume MR NF is 9dB).

Whether a baseline architecture is needed for LP-WUS receiver

Agreement: - RAN4 will further discuss and decide baseline architecture(s) for OOK-based receiver and OFMD-based receiver.

According to TR 38.369, there are three types of receiver architectures for LP-WUR that are considered suitable for OOK modulation. They are respectively architecture with RF envelope detection, heterodyne architecture with IF envelop detection and homodyne/zero-IF architecture with baseband envelope detection. Regarding RF envelope detection, the support of multiple bands and/or carriers may require multiple high-Q matching networks and/or RF BPFs or multiple off-chip components, which will bring additional cost. It can be found that, the NF of homodyne/zero-IF architecture with baseband envelop detection is lower than RF envelop detection and higher than IF envelop detection, and that the power consumption of homodyne/zero-IF architecture with baseband envelop detection is higher than RF envelope detection and lower than IF envelope detection. As we all known, power consumption and performance are a trade-off. On the whole, homodyne/zero-IF architecture with baseband envelope detection is a good compromise with regard to both power consumption and performance (i.e., NF).Hence, in our understanding, homodyne/zero-IF architecture with baseband envelope detection could be a baseline receiver for OOK-based receiver. For the OFDM-based receiver, the sole architecture should be the baseline architecture.

Proposal 3: Homodyne/zero-IF architecture could be a baseline receiver for OOK-based receiver and OFDM architecture could be a baseline receiver for OFDM-based receiver.

Conclusions

In this paper, we provide our views on RX requirements of REFSENS, we have made the following proposals.

Proposal 1: IM value could use 2.5dB for OOK-based and OFDM-based receiver.

Proposal 2: NF can be defined as delta NF on top of MR (e.g., OOK based LP-WUS is + 8 dB and OFDM based LP-WUS is +2 dB on top of MR (assume MR NF is 9dB).

Proposal 3: Homodyne/zero-IF architecture could be a baseline receiver for OOK-based receiver and OFDM architecture could be a baseline receiver for OFDM-based receiver.

- Reference

- R4-2410569, WF on NR LP-WUS UE requirements, vivo, TSG-RAN WG4 111 meeting