3GPP TSG-RAN WG4 Meeting#112		R4-2411495

Maastricht, the Netherlands, Aug 19th -23rd, 2024

Agenda item:	8.22.2.1

Source: 	Spreadtrum Communications

Title: 	Discussion on systems parameters for LP-WUR

Document for:	Discussion

Introduction

In RAN4#111 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

Channel raster for LP-WUR

Agreements: RAN4 further check whether no channel raster is needed for LP-WUR.

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The centre of the channel bandwidth configured by gNB for UE should be aligned with the channel raster. Therefore, it is necessary for gNB to know the channel raster. According to the previous agreement, we focus on IMT bands. No matter whether LR and MR use the same frequency band or different bands, these bands belong to the IMT bands. Based on the above analysis, gNB could know the channel raster based on existing channel raster for IMT bands. Thus, the existing channel raster could be reused for LP-WUR. No channel raster is needed for LP-WUR. In addition, from RAN1’s perspective, the same grid of serving cells can be shared by MR and LP-WUR. Hence, so no channel raster is needed for LP-WUR.

Proposal 1: Existing channel raster could be reused for LP-WUR, no channel raster is needed for LP-WUR.

Metric for LP-WUR requirements

Agreements: - MDR is used as the metric for LP_WUR Rx RF requirements - FFS MDR values. - FAR will be defined as demodulation requirements - FFS FAR values.

Missed detection rate

The performance metric is 1% probability of miss-detection of the downlink scheduling grant for PDCCH, which can be seen in Table 5.3.2.1.1-1 in TS 38.101-4. A. Therefore, it is reasonable to use 1% missed detection rate as the performance metric for LP-WUS RF requirements.

Table 5.3.2.1.1-1: Minimum performance for PDCCH with 15 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.1 FDD | TDLA30-10 | 1x2 Low | 1 | 8.1 |
| 2 | 10 | 24 | 2 | 2 | R.PDCCH. 1-2.3 FDD | TDLC300-100 | 1x2 Low | 1 | 8.2 |
| 3 | 10 | 48 | 2 | 4 | R.PDCCH. 1-2.4 FDD | TDLA30-10 | 1x2 Low | 1 | 5.5 |
| 4 | 10 | 48 | 1 | 4 | R.PDCCH.1-1.1 FDD | TDLA30-10 | 1x2 Low | 1 | 4.4 |
| 5 | 10 | 48 | 2 | 16 | R.PDCCH. 1-2.6 FDD | TDLA30-10 | 1x2 Low | 1 | -2.1 |

According to RAN1’s conclusion, coverage target of LP-WUR is PUCCH msg3. Generally speaking, the coverage of PDCCH is better than of PUSCH msg3. However, from the point view of system performance, lower MDR maybe means lower power consumption and overhead. Thus, we can reuse the existing PDCCCH requirements. The performance metric is 1% probability of miss-detection of the downlink scheduling grant for PDCCH, thus we could reuse 1% MDR is suitable for LP-WR RX RF requirements.

Proposal 2: Use 1% MDR as the metric for LP-WUR RX RF requirements.

False alarm rate

As we all known, the false alarm rate is related to CRC length. The lower the false alarm rate value if the longer CRC length is. Even though RAN1 has no concluded that CRC is existing or OOK sequences based on information carrying, but the false alarm rate value is related to power consumption and signalling overhead (e.g., OOK sequences based information carrying or CRC length). In addition, MDR and FAR will influence each other, when FAR is higher, MDR will be lower, in fact, it isn’t a real phenomenon. In order to meet the better system performance, 1% FAR should be considered. In our understanding, a 1% FAR value is suitable as a demodulated requirement.

Proposal 3: Use 1% FAR as the metric for LP-WUR demodulation requirements.

Conclusions

In this paper, we provide our views on 6RX UE RF requirements, we have made the following proposals.

Proposal 1: Existing channel raster could be reused for LP-WUR, no channel raster is needed for LP-WUR.

Proposal 2: Use 1% MDR as the metric for LP-WUR RX RF requirements.

Proposal 3: Use 1% FAR as the metric for LP-WUR demodulation requirements.

- Reference

- R4-2410569, WF for [111][136] NR_LP-WUS_UERF, vivo, TSG-RAN WG4 111 meeting