3GPP TSG-RAN WG4 Meeting#112		R4-2411496

Maastricht, the Netherlands, Aug 19th -23th, 2024

Agenda item:	8.25.2.1

Source: 	Spreadtrum Communications

Title: 	Discussion on NTN (e)Redcap UE RF requirements

Document for:	Discussion

Introduction

In RAN4#111 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

REFSENS for NTN HD-FDD Redcap

Agreement: [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112. Note: RAN4#111 maintenance agreement on n256 refsens in R4-2408787 was not considered for this agreement

According to the agreement, we can start the discussions on the RF requirements for NTN Redcap with HD-FDD. The one important of RF requirements is REFSNES.As we all know, the REFSENS requirements for NR FDD and NR NTN band are band dependent.

We can follow R17 Redcap REFSENS, adopt the same scale for NTN Redcap REFNSENS requirements.

Table1: HD-FDD REFSENS tightening from FD-FDD for NR

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.8 |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -94.2 |  |  |  |  |  |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

According to the table1, and Table 7.3.2-1, it can be found that n256 for 2RX HD-FDD Redcap REFSENS could be tightening 0.5 dB compared with 2RX FD-FDD REFSENS, n255 for 2RX HD-FDD Redcap REFSENS could be tightening 0 dB compared with 2RX FD-FDD REFSENS. Based on the amylase above, it is proposed:

Proposal 1: Adopting the following REFSENS for NTN HD-FDD Redcap UE:

- 2RX HD-FDD Redcap REFSENS:

- n256: Existing n256 2RX FD-FDD RESENS-0.5 dB

- n255: Existing n255 2RX FD-FDD RESENS-0 dB

Maximum output power

options for further discussion: Option 1: Consider increase in PC3 power level for NTN RedCap UE which only supports HD-FDD operation for a band. Option 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE

As is known to all, FD-FDD with duplex filter. However, in order to supress out-of-band emission and interference, HD-FDD also needs two simplex filters, the one for TX and the other for RX. However, according to practical experience, there is no obvious difference in insertion loss (e.g., 0.2dB) between duplex filter and simplex filter. Hence, in our understanding, there is no room for increasing TX power in terms of insertion loss. We should keep PC3 power level is the same for non-Redcap NR NTN UE and (e)Redcap NR NTN UE.

Proposal 2: Support Option 2. PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE.

Conclusions

In this paper, we provide our views on on NTN (e)Redcap UE RF requirements, we have made the following

proposals.

Proposal 1: Adopting the following REFSENS for NTN HD-FDD Redcap UE:

- 2RX HD-FDD Redcap REFSENS:

- n256: Existing n256 2RX FD-FDD RESENS-0.5 dB

- n255: Existing n255 2RX FD-FDD RESENS-0 dB

Proposal 2: Support option 2. PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE.

- Reference

- R4-2410574, WF on NR_NTN_Ph3_UERF, Qualcomm INC.TSG-RAN4#111 meeting