3GPP TSG-RAN WG4 Meeting #112	R4-2411497

# Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	8.1.1.3.1

Source:	MediaTek Inc.

Title:	Discussion on NR 6RX UE RF REFSENS requirements

Document for:	Approval

- Introduction

In RAN#103 plenary meeting, the WID on UE RF enhancement for NR FR1/FR2 and EN-DC was approved in [1]. One of the main objectives is to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario. In RAN4#111 meeting, the WF[2] for guiding further discussion on 6RX ΔRIB,6R value were proposed.

In this contribution, our proposal about UE RF REFSENS requirements for 6Rx UE is provided for further discussion.

- Discussion

2.1		6Rx UE RF REFSENS requirements for single-carrier bands (frequency >2.5GHz)

The Rel-19 WID [1] objectives of UE RF enhancement for NR FR1/FR2 and EN-DC about 6Rx are shown below.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

The Rel-19 UE 6Rx WF [2] in RAN4#111 meeting indicated the following works as follows.

Sub-topic 1-1: General considerations for specifying ΔRIB,6R value Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA Way Forward: Further discuss the following options. - Option 1: RAN4 needs to determine whether to define different ΔRIB,6R value for handheld UE and FWA separately - Option 2: Same value for handheld UE and FWA - Option 3: Different value for handheld UE and FWA

Sub-topic 1-2: ΔRIB,6R values for handheld UE and FWA Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA Way Forward: Further discuss the following options for the ΔRIB,6R values. Ultimately, the decision to specify separate or same requirements for handheld UE and FWA will be dependent on outcome of Issue 1-1-2. - Option 1: Consider range of ΔRIB,6R in brackets for each case below for further discussion. Operating bands; ΔRIB,6R for FWA; ΔRIB,6R for Handheld. n77, n78, n79, [n104]; [-3.0 ~ -3.3] dB; [-3.0 ~ -3.2] dB. n41; [-3.0 ~ -4.0] dB; [-3.0 ~ -4.0] dB. - Option 2: Consider average value of ΔRIB,6R from company proposals for further discussion. - Option 3: Other options are not precluded.

According to 6RX REFSENS from WF[2], the ΔRIB,6R values from different companies are provided in Table 1 for reference.

Table 1: In WF[2], companies’ proposed ΔRIB,6R values for handheld UE and FWA

Proposed ΔRIB,6R values for handheld UE and FWA: o Option 1: -3.3dB for n41; -3dB for n77, n78, n79, n104 (Apple) o Option 2: -3.4dB for n41; -3.1dB for n77, n78, n79, n104 (MediaTek) o Option 3: -3.2dB for n41; -3dB for n77, n78, n79, n104 (Xiaomi) o Option 4: ΔRIB,6R can be defined considering following range. (LGE) -  ΔRIB,6R = -4.77 (ideal ΔRIB,6R) + Relaxation6Rx -  [0.3 ~0.8] dB< Relaxation6Rx <[1.5~2] dB o Option 5: -3.3dB/-3.6dB for n41 handheld/FWA; -3dB/-3.2dB for n77, n78, n79, n104 handheld/FWA (Spreadtrum) o Option 6: [-3.3dB] for n41; [-3dB] for n77, n78, n79, n104 (Meta) o Option 7: -3dB for n41; -3dB for n77, n78, n79, n104 (Vivo) o Option 8: -3.2dB/-3.4dB for n41 handheld/FWA; -3dB/-3.2dB for n77, n78, n79, n104 handheld/FWA(Vivo) o Option 9: -3.5dB for n41; -3dB for n77, n78, n79, n104 (ZTE, Sanechips, Qualcomm) o Option 10: -4dB for n41 (Nokia) o Option 11: -3.6dB for n41; -3.2dB for n77, n78, n79, n104 (OPPO, Ericsson) o Option 12: -3.4dB/-3.7dB for n41 handheld/FWA; -3dB/-3.3dB for n77, n78, n79 handheld/FWA(Google) o Option 13: -3dB for n41; -3dB for n77, n78, n79 (Huawei) o Option 14: Consider range of ΔRIB,6R in brackets for each case based on existing proposals above for further discussion.

Observation 1: Regarding n77/n78/n79/[n104], in WF[2], the range ofΔRIB,6R values are [-3.0~-3.3] dB and [-3.0~-3.2] dB for FWA and handheld UE, respectively. The median values of ΔRIB,6R are -3.15 dB and -3.1 dB for FWA and handheld UE, respectively.

Observation 2: Regarding n77/n78/n79/n104, in R4-2407313[3], theΔRIB,6R values of -3.1 dB and -3.1 dB were proposed for FWA and handheld UE, respectively.

Observation 3: Regarding n41, in WF[2], the range ofΔRIB,6R values are [-3.0~-4.0] dB and [-3.0~-4.0] dB for FWA and handheld UE, respectively. The median values of ΔRIB,6R are -3.5dB and -3.5dB for FWA and handheld UE, respectively.

Observation 4: Regarding n41, in R4-2407313[3], theΔRIB,6R values of -3.4 dB and -3.4 dB were proposed for FWA and handheld UE, respectively.

Proposal 1: Based on observations 1 and 3, if there is little difference in ΔRIB, 6R values between handheld UE and FWA, to define the same ΔRIB, 6R value for handheld UE and FWA.

Proposal 2: Regarding n77/n78/n79/n104, based on observations 1 and 2, to defineΔRIB,6R values of [-3.1] dB and [-3.1] dB for FWA and handheld UE, respectively.

Proposal 3: Regarding n41, based on observations 3 and 4, to defineΔRIB,6R values of [-3.4] dB and [-3.4] dB for FWA and handheld UE, respectively.

3 Conclusion

In this paper, our views on NR 6Rx RF REFSENS requirements for handheld and FWA UE are provided as follows:

Observation 1: Regarding n77/n78/n79/[n104], in WF[2], the range ofΔRIB,6R values are [-3.0~-3.3] dB and [-3.0~-3.2] dB for FWA and handheld UE, respectively. The median values of ΔRIB,6R are -3.15 dB and -3.1 dB for FWA and handheld UE, respectively.

Observation 2: Regarding n77/n78/n79/n104, in R4-2407313[3], theΔRIB,6R values of -3.1 dB and -3.1 dB were proposed for FWA and handheld UE, respectively.

Observation 3: Regarding n41, in WF[2], the range ofΔRIB,6R values are [-3.0~-4.0] dB and [-3.0~-4.0] dB for FWA and handheld UE, respectively. The median values of ΔRIB,6R are -3.5dB and -3.5dB for FWA and handheld UE, respectively.

Observation 4: Regarding n41, in R4-2407313[3], theΔRIB,6R values of -3.4 dB and -3.4 dB were proposed for FWA and handheld UE, respectively.

Proposal 1: Based on observations 1 and 3, if there is little difference in ΔRIB, 6R values between handheld UE and FWA, to define the same ΔRIB, 6R value for handheld UE and FWA.

Proposal 2: Regarding n77/n78/n79/n104, based on observations 1 and 2, to defineΔRIB,6R values of [-3.1] dB and [-3.1] dB for FWA and handheld UE, respectively.

Proposal 3: Regarding n41, based on observations 3 and 4, to defineΔRIB,6R values of [-3.4] dB and [-3.4] dB for FWA and handheld UE, respectively.

Reference

- RP- 240828, “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, AT&T

- R4-2410751, “WF on requirements for 6Rx”, AT&T

- R4-2407313, “Discussion on NR 6RX UE RF REFSENS requirements”, MediaTek Inc.