3GPP TSG-RAN WG4 Meeting #112	R4-2411498

# Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	8.25.2.1

Source:	MediaTek Inc.

Title:	Discussion on Rel-19 NR-NTN RedCap UE RF requirements

Document for:	Discussion

- Introduction

In RAN#102 plenary meeting, the Rel-19 WID of Non-Terrestrial Networks (NTN) for NR Phase 3 was approved in [1]. One of the main objectives is to support Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN band.

In RAN4#111 meeting, the WF [3] for guiding further discussion on residual issues were proposed.

In this contribution, our views about the residual issues are provided.

- Discussion

2.1		Rel-19 phase 3 NR-NTN UE RF requirements

The Rel-19 WID [1] objectives indicate that NTN for NR Phase 3 supports Redcap and eRedcap with NR NTN operating in FR1-NTN band. The objectives are shown below.

4.1 Objective of SI or Core part WI or Testing part WI Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

The Rel-19 NR_NTN_Ph3_UERF WF [3] in RAN4#111 meeting provided the guidance as follows.

Issue 1-2: HD-FDD refsens for 2 Rx Agreement: [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112. Note: RAN4#111 maintenance agreement on n256 refsens in R4-2408787 was not considered for this agreement.

Issue 1-4: Frequency error Agreement: Discuss changes common to non-Redcap NTN and (e)RedCap NTN NR UE frequency error in maintenance agenda. (e)RedCap UE frequency error requirement may be impacted by this maintenance discussion. Option for further discussionn, not an agreement: - As a starting point, consider (e)Redcap UE frequency error based on existing NR UE requirements. Issue 1-5: Maximum output power options for further discussion: Option 1: Consider increase in PC3 power level for NTN RedCap UE which only supports HD-FDD operation for a band. Option 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE

2.2		HD-FDD refsens for 2 Rx

Regarding HD-FDD REFSENS for 2 Rx, in RAN4#111 meeting, because NTN connection performance between SAN and UE is important, some comments indicate that it is desired to achieve more than 0.5 dB tightening on n254 and n256. Therefore, WF [3] indicates that [0.5] dB tightening for n254 and n256, FFS for n255, and n256 to be confirmed in RAN4#112.

After checking TS 38.101-1 [2] and TS 38.101-5 [4], some tables are shown below for reference.

TS 38.101-5 [4] Table 7.3.2-1: Two antenna port reference sensitivity QPSK REFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.8 |  |  |
|  | 30 |  | -96.6 | -94.6 | -94 |  |  |
|  | 60 |  | -97 | -94.9 | -94.2 |  |  |
| n255 | 15 | -100 | -96.8 | -95 | -93.8 |  |  |
|  | 30 |  | -97.1 | -95.1 | -94 |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |
| n254 | 15 | -99.5 | -96.3 | -94.5 |  |  |  |
|  | 30 |  | -96.6 | -94.6 |  |  |  |
|  | 60 |  | -97 | -94.9 |  |  |  |

Table 1: NR-TN and NR-NTN operating bands

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high |
| --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz |
| n24 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz |

Observation 1: Regarding NTN HD-FDD REFSENS for 2Rx, in RAN4#111 meeting, because NTN connection performance between SAN and UE is important, some comments indicate that it is desired to achieve more than 0.5 dB tightening on n254 and n256. Therefore, WF [3] indicates that [0.5] dB tightening for n254 and n256, FFS for n255, and n256 to be confirmed in RAN4#112.

Observation 2: In TS 38.101-5[4], 2Rx FD-FDD REFSENS are -99.5dBm, -100dBm, and -99.5dBm for n256, n255 and n254 for 5MHz CBW, respectively.

Observation 3: Switching from FD-FDD mode to HD-FDD mode would obviously improve REFSENS due to lower front-end insertion loss but may only show a little improvement where original FD-FDD REFSENS is already quite good.

Proposal 1: Based on observations 1 to 3, regarding HD-FDD REFSENS for 2 Rx, consider whether 0.5 dB tightening for n254 (i.e., 2Rx HD-FDD REFSENS is -100dBm/5MHz) would be applicable.

Proposal 2: Based on observations 1 to 3, regarding HD-FDD REFSENS for 2 Rx, consider whether 0.6 dB tightening for n256 (i.e., 2Rx HD-FDD REFSENS is -100.1 dBm/5MHz) would be applicable.

Proposal 3: Based on observations 1 to 3, regarding HD-FDD REFSENS for 2 Rx, consider whether 0.2 dB tightening for n255 (i.e., 2Rx HD-FDD REFSENS is -100.2 dBm/5MHz) would be applicable.

2.3		Maximum output power

Observation 4: In RAN4#111 meeting, some company indicated that the HD-FDD RF front-end (FE) insertion loss is lower compared to FD-FDD FE insertion loss and can improve the RX REFSENS and TX maximum output power simultaneously.

Observation 5: When NTN-RedCap UE supports both HD-FDD and FD-FDD operations for a band, if HD-FDD FE and FD-FDD FE are implemented separately, it is observed that TX maximum output power would increase when FD-FDD operation is switched to HD-FDD operation.

Observation 6: When NTN-RedCap UE supports both HD-FDD and FD-FDD operations for a band, if only FD-FDD FE is implemented, it is observed that the same TX maximum output power may be applicable for both FD-FDD and HD-FDD operations due to the same FE insertion loss.

Observation 7: When NTN-RedCap UE only supports HD-FDD operation for a band, it is observed that the HD-FDD TX maximum output power would be obviously better compared to FD-FDD TX maximum output power.

Proposal 4: Clarify whether only FD-FDD FE circuits are implemented and additional HD-FDD FE circuits are precluded when NTN-RedCap UE supports both HD-FDD and FD-FDD operations for a NTN band.

Proposal 5: For a NTN band, if implementing additional HD-FDD FE is not precluded for NTN-RedCap UE which supports both HD-FDD and FD-FDD operations, consider PC3 maximum output power under HD-FDD operation as:

Option 1: PC3 of [23.2~23.4] dBm for NTN-RedCap UE under HD-FDD operation for a NTN band.

Option 2: PC3 of 23 dBm with +[2.4]/-2 dB power tolerance for NTN-RedCap UE under HD-FDD operation for a NTN band.

Option 3: PC3 of [23.2] dBm with +[2.2]/-2 dB power tolerance for NTN-RedCap UE under HD-FDD operation for a NTN band.

Proposal 6: Consider PC3 maximum output power:

Option 1: For a NTN band, PC3 of [23.4~23.8] dBm for NTN-RedCap UE under only HD-FDD support.

Option 2: For a NTN band, PC3 of 23 dBm with +[2.8]/-2 dB power tolerance for NTN-RedCap UE under only HD-FDD support.

Option 3: For a NTN band, PC3 of [23.4] dBm with +[2.4]/-2 dB power tolerance for NTN-RedCap UE under only HD-FDD support.

3 Conclusion

In this paper, our views on NTN RedCap RF requirements are provided as follows:

Observation 1: Regarding NTN HD-FDD REFSENS for 2Rx, in RAN4#111 meeting, because NTN connection performance between SAN and UE is important, some comments indicate that it is desired to achieve more than 0.5 dB tightening on n254 and n256. Therefore, WF [3] indicates that [0.5] dB tightening for n254 and n256, FFS for n255, and n256 to be confirmed in RAN4#112.

Observation 2: In TS 38.101-5[4], 2Rx FD-FDD REFSENS are -99.5dBm, -100dBm, and -99.5dBm for n256, n255 and n254 for 5MHz CBW, respectively.

Observation 3: Switching from FD-FDD mode to HD-FDD mode would obviously improve REFSENS due to lower front-end insertion loss but may only show a little improvement where original FD-FDD REFSENS is already quite good.

Proposal 1: Based on observations 1 to 3, regarding HD-FDD REFSENS for 2 Rx, consider whether 0.5 dB tightening for n254 (i.e., 2Rx HD-FDD REFSENS is -100dBm/5MHz) would be applicable.

Proposal 2: Based on observations 1 to 3, regarding HD-FDD REFSENS for 2 Rx, consider whether 0.6 dB tightening for n256 (i.e., 2Rx HD-FDD REFSENS is -100.1 dBm/5MHz) would be applicable.

Proposal 3: Based on observations 1 to 3, regarding HD-FDD REFSENS for 2 Rx, consider whether 0.2 dB tightening for n255 (i.e., 2Rx HD-FDD REFSENS is -100.2 dBm/5MHz) would be applicable.

Observation 4: In RAN4#111 meeting, some company indicated that the HD-FDD RF front-end (FE) insertion loss is lower compared to FD-FDD FE insertion loss and can improve the RX REFSENS and TX maximum output power simultaneously.

Observation 5: When NTN-RedCap UE supports both HD-FDD and FD-FDD operations for a band, if HD-FDD FE and FD-FDD FE are implemented separately, it is observed that TX maximum output power would increase when FD-FDD operation is switched to HD-FDD operation.

Observation 6: When NTN-RedCap UE supports both HD-FDD and FD-FDD operations for a band, if only FD-FDD FE is implemented, it is observed that the same TX maximum output power may be applicable for both FD-FDD and HD-FDD operations due to the same FE insertion loss.

Observation 7: When NTN-RedCap UE only supports HD-FDD operation for a band, it is observed that the HD-FDD TX maximum output power would be obviously better compared to FD-FDD TX maximum output power.

Proposal 4: Clarify whether only FD-FDD FE circuits are implemented and additional HD-FDD FE circuits are precluded when NTN-RedCap UE supports both HD-FDD and FD-FDD operations for a NTN band.

Proposal 5: For a NTN band, if implementing additional HD-FDD FE is not precluded for NTN-RedCap UE which supports both HD-FDD and FD-FDD operations, consider PC3 maximum output power under HD-FDD operation as:

Option 1: PC3 of [23.2~23.4] dBm for NTN-RedCap UE under HD-FDD operation for a NTN band.

Option 2: PC3 of 23 dBm with +[2.4]/-2 dB power tolerance for NTN-RedCap UE under HD-FDD operation for a NTN band.

Option 3: PC3 of [23.2] dBm with +[2.2]/-2 dB power tolerance for NTN-RedCap UE under HD-FDD operation for a NTN band.

Proposal 6: Consider PC3 maximum output power:

Option 1: For a NTN band, PC3 of [23.4~23.8] dBm for NTN-RedCap UE under only HD-FDD support.

Option 2: For a NTN band, PC3 of 23 dBm with +[2.8]/-2 dB power tolerance for NTN-RedCap UE under only HD-FDD support.

Option 3: For a NTN band, PC3 of [23.4] dBm with +[2.4]/-2 dB power tolerance for NTN-RedCap UE under only HD-FDD support.

Reference

- RP-234078, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”

- TS 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- R4-2410574, “WF on Rel-19 phase 3 NR-NTN UE RF requirements”, Qualcomm

- TS 38.101-5, “NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”