3GPP TSG-RAN WG4 Meeting #112	R4-2411499

# Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	8.8.2.2

Source:	MediaTek Inc.

Title:	Discussion on NTN HPUE TX RF requirements

Document for:	Approval

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable PC2 high power UE (HPUE) with defining RF requirements targeting at support of NR and IoT NTN UEs for FR1 single carrier scenario.

In this contribution, our views about PC2 HPUE TX RF requirements for NR and IoT NTN UE are provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

According to specification of HPUE UE ACLR for FR1 NR TN bands, the ACLR values in TS 38.101-1 [2] are provided in Table below for reference.

Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

Observation 1: Based on UE implementation, the PC2 ACLR of -31dB from NR TN UE may be used as a reference for specifying PC2 ACLR for NR NTN UE.

Proposal 1: Discuss whether to use PC2 ACLR of [-31dB] as a starting point. Decide the final ACLR value pending on the NTN PC2 coexistence analysis results.

According to specification of IoT UE ACLR for FR1 LTE TN bands, the ACLR value in TS 36.101 [3] are provided in Table below for reference.

Table 6.6.2.3.1-1: General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +1.4 / -1.4 | +3.0 / -3.0 | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |

Table 6.6.2.3.1-2: Additional E-UTRAACLR requirements for Power Class 1

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 37 dB | 37 dB | 37 dB | 37 dB | 37 dB | 37 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +1.4 / -1.4 | +3.0 / -3.0 | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |
| NOTE 1: E-UTRAACLR1 shall be applicable for >23dBm |  |  |  |  |  |  |

Table 6.6.2.3.1-3: Additional E-UTRAACLR requirements for Power Class 2

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | N/A | N/A | 31 dB | 31 dB | 31 dB | 31 dB |
| E-UTRA channel Measurement bandwidth | N/A | N/A | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | N/A | N/A | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |

Based on TS 36.101 [3] Table 6.6.2.3.1-3, it is observed that there is no PC2 ACLR specification for IoT TN Cat-M1 UE. Another observation is that PC3 and PC1 ACLR values specified in TS 36.101 [3] for IoT TN Cat-M1 UE are the same as PC3 and PC1 ACLR values specified in TS 38.101-1 [2] used for NR TN UE.

Observation 2: The PC3 and PC1 ACLR values of -30dB and -37dB in TS 36.101 [3] are the same as PC3 and PC1 ACLR values in TS 38.101-1 [2]. The PC2 ACLR of -31dB for CBW of 5MHz to 20MHz is used in TS 38.101-1 and TS 36.101. There is no PC2 ACLR specified in TS 36.101 for CBW of 1.4MHz and 3MHz.

Proposal 2: Discuss whether to use TS 38.101-1 [2] PC2 ACLR of -31dB as a starting reference for specifying PC2 ACLR values for CBW of 1.4MHz and 3MHz in TS 36.101 [3] and TS 36.102[4]. Decide the final ACLR values pending on the NTN PC2 coexistence analysis results.

It is observed that there is no PC2 ACLR specification in TS 36.101 [3] for IoT TN Cat-NB1/NB2 UE.

Table 6.6.2F.3-1: category NB1 and NB2 UE ACLR requirements

|  | GSMACLR | UTRAACLR |
| --- | --- | --- |
| ACLR | 20 dB | 37 dB |
| Adjacent channel center frequency offset from category NB1 or NB2 Channel edge | ±200 kHz | ±2.5 MHz |
| Adjacent channel measurement bandwidth | 180 kHz | 3.84 MHz |
| Measurement filter | Rectangular | RRC-filter α=0.22 |
| Category NB1 and NB2 channel measurement bandwidth | 180 kHz | 180 kHz |
| Category NB1 and NB2 channel Measurement filter | Rectangular | Rectangular |

Observation 3: There is no PC2 ACLR specified in TS 36.101 [3] for IoT TN Cat-NB1/NB2 UE.

Observation 4: There is no PC2 ACLR specified in TS 36.102 [4] for IoT NTN Cat-NB1/NB2 UE.

Proposal 3: Regarding IoT NTN Cat-NB1/NB2 UE, discuss whether to use TS 36.102 [4] PC3 ACLR values with [1dB] improvement as a starting reference for specifying PC2 ACLR values in TS 36.102. Decide the final ACLR values pending on the NTN PC2 coexistence analysis results.

Based on TS 38.101-1 [2], it is observed that there is PC2 MPR specification for NR TN UE. The MPR is relevant to ACLR and modulation EVM requirements. We think to reuse PC2 MPR from NR TN UE for NR NTN UE would be one way for consideration. It is also indicated that the final PC2 MPR values would be decided after PC2 ACLR is completed.

TS 38.101-1 Table 6.2.2-1 Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52,3 | ≤ 0.52 | 02,4 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52,3 | 02 | 02,4 |
|  | QPSK | ≤ 1 |  | 05 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

TS 38.101-1 Table 6.2.2-2 Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 02 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

In TS 38.101-5 6.2.2 UE maximum output power reduction UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 3, the allowed maximum power reduction (MPR) is defined as Table 6.2.2-1 in 3GPP TS 38.101-1[5] clause 6.2.2.

Observation 5: Based on TS 38.101-1 [2] and TS 38.101-5 [5], it is observed that the PC3 MPR specification for NR TN UE is reused for NR NTN UE.

Observation 6: Based on TS 38.101-1 [2], it is observed that there is PC2 MPR specification for NR TN UE. The MPR is relevant to ACLR and modulation EVM requirements.

Proposal 4: Consider reusing TS.38.101-1 [2] PC2 MPR specification from NR TN UE for NR NTN UE as starting point. For NR NTN UE, the final PC2 MPR values in TS 38.101-5 [5] would be decided after NR NTN PC2 ACLR is completed.

3 Conclusion

In this paper, our views about PC2 HPUE TX RF requirements for NR and IoT NTN UE are provided as follows:

Observation 1: Based on UE implementation, the PC2 ACLR of -31dB from NR TN UE may be used as a reference for specifying PC2 ACLR for NR NTN UE.

Proposal 1: Discuss whether to use PC2 ACLR of [-31dB] as a starting point. Decide the final ACLR value pending on the NTN PC2 coexistence analysis results.

Observation 2: The PC3 and PC1 ACLR values of -30dB and -37dB in TS 36.101 [3] are the same as PC3 and PC1 ACLR values in TS 38.101-1 [2]. The PC2 ACLR of -31dB for CBW of 5MHz to 20MHz is used in TS 38.101-1 and TS 36.101. There is no PC2 ACLR specified in TS 36.101 for CBW of 1.4MHz and 3MHz.

Proposal 2: Discuss whether to use TS 38.101-1 [2] PC2 ACLR of -31dB as a starting reference for specifying PC2 ACLR values for CBW of 1.4MHz and 3MHz in TS 36.101 [3] and TS 36.102[4]. Decide the final ACLR values pending on the NTN PC2 coexistence analysis results.

Observation 3: There is no PC2 ACLR specified in TS 36.101 [3] for IoT TN Cat-NB1/NB2 UE.

Observation 4: There is no PC2 ACLR specified in TS 36.102 [4] for IoT NTN Cat-NB1/NB2 UE.

Proposal 3: Regarding IoT NTN Cat-NB1/NB2 UE, discuss whether to use TS 36.102 [4] PC3 ACLR values with [1dB] improvement as a starting reference for specifying PC2 ACLR values in TS 36.102. Decide the final ACLR values pending on the NTN PC2 coexistence analysis results.

Observation 5: Based on TS 38.101-1 [2] and TS 38.101-5 [5], it is observed that the PC3 MPR specification for NR TN UE is reused for NR NTN UE.

Observation 6: Based on TS 38.101-1 [2], it is observed that there is PC2 MPR specification for NR TN UE. The MPR is relevant to ACLR and modulation EVM requirements.

Proposal 4: Consider reusing TS.38.101-1 [2] PC2 MPR specification from NR TN UE for NR NTN UE as starting point. For NR NTN UE, the final PC2 MPR values in TS 38.101-5 [5] would be decided after NR NTN PC2 ACLR is completed.

Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- TS 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- TS 36.101, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception”

- TS 36.102, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access”

- TS 38.101-5, “NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”