3GPP TSG-RAN WG4 Meeting #112	R4-2411500

# Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	8.8.2.3

Source:	MediaTek Inc.

Title:	Discussion on NTN HPUE RX RSD requirements

Document for:	Approval

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable PC2 high power UE (HPUE) with defining RF requirements targeting at support of NR and IoT NTN UEs for FR1 single carrier scenario.

In this contribution, our evaluation about PC2 HPUE RX reference sensitivity degradation (RSD) requirements for NR and IoT NTN is provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

Regarding IoT NTN category NB1 and NB2 UE, our observation is that the power class change from PC3 to PC2 for FDD bands 256 and 255 do not cause RX RSD because IoT NTN NB1/NB2 UE is operated in half-duplex FDD (HD-FDD) mode.

Proposal 1: Regarding PC2 IoT NTN IoT NB1 and NB2 UEs operated in half-duplex FDD mode, no RX RSD for bands 256 and 255.

Although NTN IoT category M1 (Cat-M1) supports both FDD and HD-FDD modes, no RX RSD for bands 256 and 255 when NTN IoT PC2 Cat-M1 UE operated in half-duplex FDD mode.

Proposal 2: Although IoT NTN category M1 (Cat-M1) supports both FDD and HD-FDD modes, RX RSD is not needed for bands 256 and 255 when PC2 IoT NTN Cat-M1 UE operates in HD-FDD mode.

According to survey of PC2 UE RX RSD from FR1 TN bands, the RSD values in TS 38.101-1 [2] are provided in Tables below for reference.

Table 7.3.2-1c Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n2 | 0.8 | 0.9 | 1.1 | 1.2 | 1.3 | 2.7 | 2.8 | 3.5 |  |  |
| n3 | 0.5 | 0.5 | 0.5 | 0.5 | 0.6 | 0.8 | 1.1 | 1.5 | 2.3 | 2.8 |
| n8 | 0.5 | 0.7 | 0.8 | 2.3 | 2.8 | 3.2 | 3.1 |  |  |  |
| n14 | 0.6 | 0.8 |  |  |  |  |  |  |  |  |
| n25 | 0.8 | 0.8 | 0.9 | 1.1 | 1.3 | 2.7 | 2.8 | 3.5 | 3.7 |  |
| n66 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| n71 | 0.5 | 0.9 | 0.9 | 2.2 | 2.42 2.53 | 2.52 2.43 | 2.92 3.13 |  |  |  |
| n70 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-1d Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n2 | 1.2 | 1.2 | 1.3 | 1.2 | 1.2 | 5.8 | 6.0 | 6.5 |  |  |
| n3 | 1.4 | 1.5 | 1.5 | 1.5 | 1.6 | 1.7 | 2.8 | 5 | 5.5 | 6.0 |
| n8 | 1.3 | 1.4 | 2.1 | 5.8 | 6.1 | 6.5 | 7.0 |  |  |  |
| n14 | 1.1 | 1.3 |  |  |  |  |  |  |  |  |
| n25 | 1.5 | 1.5 | 1.6 | 1.6 | 1.7 | 6.0 | 6.2 | 6.7 | 7.1 |  |
| n66 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| n71 | 1.1 | 1.1 | 1.7 | 5.5 | 5.92 6.93 | 6.22 7.23 | 6.52 7.33 |  |  |  |
| n70 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |

It is observed that RX RSD values are needed for some NR TN mid-bands when UE operates in PC2 TX. Therefore, it is expected that RX RSD may be also needed for NR NTN band.

According to our evaluation, because of high TX-RX duplex spacing frequency, no RX RSD is needed for NR NTN band n256 when UE enables PC2 TX. Regarding NR NTN band n255, the RX RSD is needed when UE enables PC2 TX.

Proposal 3: Regarding NR NTN UE operated in PC2, define the RX RSD values as shown in tables below for bands n256 and n255.

Table 1 RSD from PC3 to PC2 for FDD bands for NR NTN UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0 | 0 |
| n255 | [0.4] | [0.4] | [0.5] | [0.5] |

Table 2 RSD from PC3 to PC2 for FDD bands for NR NTN UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0 | 0 |
| n255 | [0.8] | [0.8] | [0.9] | [0.9] |

3 Conclusion

In this paper, our views on RX RSD requirements for IoT/NR NTN PC2 UE for bands 256/255/n256/n255 are provided as follows:

Proposal 1: Regarding PC2 IoT NTN IoT NB1 and NB2 UEs operated in half-duplex FDD mode, no RX RSD for bands 256 and 255.

Proposal 2: Although IoT NTN category M1 (Cat-M1) supports both FDD and HD-FDD modes, RX RSD is not needed for bands 256 and 255 when PC2 IoT NTN Cat-M1 UE operates in HD-FDD mode.

Proposal 3: Regarding NR NTN UE operated in PC2, define the RX RSD values as shown in tables below for bands n256 and n255.

Table 1 RSD from PC3 to PC2 for FDD bands for NR NTN UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0 | 0 |
| n255 | [0.4] | [0.4] | [0.5] | [0.5] |

Table 2 RSD from PC3 to PC2 for FDD bands for NR NTN UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0 | 0 |
| n255 | [0.8] | [0.8] | [0.9] | [0.9] |

Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- TS 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”