3GPP TSG-RAN WG4 Meeting #112	R4-2411505

# Maastricht, Netherlands, 19 – 23 August, 2024

Agenda Item:	8.8.2

Source:	MediaTek Inc.

Title:	Discussion on RF requirements in NR_IoT_NTN_req_test_enh WI for NTN HPUE

Document for:	Discussion

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable high power UE (HPUE) with defining RF requirements targeting at support of NR and IoT NTN UEs for FR1 single carrier scenario. Before defining the UE RF requirements, the coexistence analysis would be completed to derive the ACLR and ACS.

In this contribution, our views about RF requirements in WI [1] are provided for discussion.

- Discussion

2.1		WI Core-part requirements

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

The first step is to conduct the coexistence analysis based on TR 38.863 [2] and then evaluate the NTN HPUE impact on the uplink performance of TN. PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS.

In TR 38.863 [2], the aggressor and victim combinations are listed in Table 6.1-2. In TR 38.863, the co-existence results suggest ACLR and ACS of NR-NTN as listed in Table 6.1-2.

TR 38.863 Table 6.1-2: Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes | Study Phase |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. | Phase 1 |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. | Phase 1 |
| 7 | NTN with NTN | NTN DL | NTN DL | HAPS-HAPS | Phase 2 |
|  |  | NTN UL | NTN UL | HAPS-HAPS | Phase 2 |

TR 38.863 Table 6.5.1-3: Co-existence results suggested ACLR and ACS of NR-NTN

| Scenario | Contributing | ACLR ACS values |
| --- | --- | --- |
| 1 | NTN UE ACS | 27.91 dB (AAS) |
| 2 | NTN SAN ACS | 37.06 dB (non-AAS) |
| 3 | NTN SAN ACLR | LEO: 23.81 dB GEO: 13.39 dB |
| 4 | NTN UE ACLR | 28.18 dB (AAS) |
| 5 | NTN UE ACLR | 27.51 dB (AAS) |
| 6 | NTN SAN ACS | 37.4 dB |
| 7 | HAPS ACLR | 24.97 dB |
| 8 | HAPS ACS | 30 dB |

Observation 1: When the aggressor NTN UE UL increases the maximum TX power (e.g., change TX power from PC3 to PC2), scenarios 4 and 5 are relevant to NTN UE ACLR requirement as indicated in TR 38.863 Table 6.5.1-3. Currently, in WI [1], it mentions that evaluation focuses on NTN UL aggressor to TN UL victim (i.e., scenario 4) and does not need evaluation about NTN UL aggressor to TN DL victim (i.e., scenario 5).

Observation 2: ACLR requirement evaluation for scenario 5 is also important for TN-bands DL protection. Scenario-5 ACLR value also affects UE-UE coexistence A-MPR results.

Proposal 1: To clarify whether ACLR evaluation for scenario 5 would be needed as well.

Observation 3: Scenario-1 evaluation is relevant to NTN UE ACS. Evaluation of scenarios 2 and 6 is relevant to NTN SAN ACS.

Observation 4: In WI [1], it mentions that ACS evaluation would be needed. It is ambiguous whether the ACS evaluation would be for NTN UE ACS or NTN SAN ACS.

Observation 5: Additionally, regarding NTN HPUE, the purpose is to evaluate the NTN-HPUE aggressor impact on the victim uplink performance of TN. It is ambiguous whether ACS evaluation would be necessary.

Proposal 2: Clarify whether the ACS evaluation would be for NTN-UE ACS or NTN-SAN ACS. Or clarify whether ACS evaluation would be necessary.

3 Conclusion

In this paper, our views about RF requirements in WI [1] are provided for discussion:

Observation 1: When the aggressor NTN UE UL increases the maximum TX power (e.g., change TX power from PC3 to PC2), scenarios 4 and 5 are relevant to NTN UE ACLR requirement as indicated in TR 38.863 Table 6.5.1-3. Currently, in WI [1], it mentions that evaluation focuses on NTN UL aggressor to TN UL victim (i.e., scenario 4) and does not need evaluation about NTN UL aggressor to TN DL victim (i.e., scenario 5).

Observation 2: ACLR requirement evaluation for scenario 5 is also important for TN-bands DL protection. Scenario-5 ACLR value also affects UE-UE coexistence A-MPR results.

Proposal 1: To clarify whether ACLR evaluation for scenario 5 would be needed as well.

Observation 3: Scenario-1 evaluation is relevant to NTN UE ACS. Evaluation of scenarios 2 and 6 is relevant to NTN SAN ACS.

Observation 4: In WI [1], it mentions that ACS evaluation would be needed. It is ambiguous whether the ACS evaluation would be for NTN UE ACS or NTN SAN ACS.

Observation 5: Additionally, regarding NTN HPUE, the purpose is to evaluate the NTN-HPUE aggressor impact on the victim uplink performance of TN. It is ambiguous whether ACS evaluation would be necessary.

Proposal 2: Clarify whether the ACS evaluation would be for NTN-UE ACS or NTN-SAN ACS. Or clarify whether ACS evaluation would be necessary.

Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- TR 38.863, “Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects”