3GPP TSG RAN4 Meeting #112							R4-2411507

Maastricht, Netherlands 19-23 August 2024

Agenda Item:		8.9.2

Source:			Intelsat

Title:				Ku Band Co-existence

Document for:		Discussion and Decision

# Introduction

Following the approval of the Ku band WID at the RAN#104 in Shanghai, several topics need to be discussed and appropriate decision must be to be made during the work.

One of the key topics of the Ku band is the co-existence with services within the Ku Band and adjacent to it.

Due to the nature and Ku Band, with one large downlink and two uplink bands, it was decided at the RAN#104, to split the Ku Band into two portions, Priority 1, and Priority 2. See following Figure 1.

The aim of this contribution to commence the discussions on the co-existence of the Ku Band with service adjacent to it. As we have highlighted in our previous contributions [R4-2400510] at least for a part of the Ku Band, the co-existence with IMT services are very relaxed due to lack of IMT / Mobile services.

The purpose of the co-existence study is to arrive at a realistic and practical values of the Adjacent Channel Leakage Ratio (ACLR) and Adjacent Channel Selectivity (ACS) values between Satellite Services in the Ku Band and possible terrestrial/mobile services.

For co-existence analysis with non-3GPP services such as scientific services and radioastronomy etc., we will have to use the ITU protection levels to ascertain the ACS and ACLR values.

In this study we are assuming stationary VSAT UE device and three satellite types.

# Ku Band Spectrum

Before we proceed to the co-existence analysis of Priority 1 and 2, it is necessary to revisit the current live services which are active in and around the Ku Band.

As it can be observed from Figure 1, the electromagnetic spectrum around the Ku Band is very crowded and there are established and licensed services operating in the Ku Band.

Figure 1 Electromagnetic Spectrum around the Ku Band

With reference to Figure 1 we can observer that:

- There are Radio Astronomy Bands between 10.68 and 10.70 GHz.

- In Regions 1 and 3, there are Multiple Scientific Services Bands between 10.40 and 10.68 GHz.

- In Regions 1 and 3, spectrum between 10.0-10.40 is reserved for Scientific Services.

- In Region 2, 10.0-10.5 GHz band was identified for IMT(5G/6G) services by WRC-23 in 12 countries. Please note that part of this band (10.0-10,4 GHz) band is shared with Scientific Services.

- The spectrum between 12.75-13.25 GHz is reserved for Fixed Satellite Services.

- The spectrum between 13.25-13.75 GHz is reserved for Scientific Services and military NATO Radar.

- The spectrum between 13.75-14.80 GHz is reserved for Fixed Satellite Services.

- The spectrum between 14.80-15.35 GHz has been selected to be studied for IMT services for WRC-27 meeting: this band is currently under study and no services have yet been assigned.

Thus, we can observe that there are no terrestrial services adjacent to the Ku Band.

# Ku Band Satellite Summary

We begin these discussions by highlighting the proposed Ku Band Satellite summary for three main Satellite types i.e. GEO, MEO and LEO. Other segments and satellite type to be considered during the Normative Phase of the work. Following Table 1 illustrates typical Ku Band Satellite Parameters and the following Table 2 common VSAT Parameters.

Table 1 Ku Band Satellite Parameters

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 14 GHz for UL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | FFS dBW/MHz | FFS dBW/MHz | FFS dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.295 deg | 2.948 deg | 2.948deg |
| Satellite beam diameter (Note 2) |  | 184 km | 62 km | 31 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 12 GHz for DL ) | 5 m | 0.5 m | 0.5 m |
| G/T |  | 28.3 dB K-1 | 8.3 dB K-1 | 8.3 dB K-1 |
| Satellite RX max Gain |  | 55.4 dBi | 35.4 dBi | 35.4 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite. NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

*Please note that the Ku Band Satellite’s EIRP density is still to be defined.

And for the VSAT we assume the following parameters see following Table 2:

Table 2 Ku Band VSAT Parameters

| Characteristics | VSAT (Note 1) |
| --- | --- |
| Frequency band | Ku-band (i.e. 14 GHz UL and 12 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 36.7 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 35.1 dBi |
| NOTE 1: VSAT terminal characteristics could be implemented with phased array antenna |  |

# Discussion

In practical deployment, Uplink and Downlink services are assigned in either in Priority 1 OR in Priority 2 configurations; for instance, Downlink Services are paired with either Uplink Band A OR with Upline Band B.

## Priority 1 Configuration

For Priority 1 the in regions 1, and 3, currently there are no live IMT services running. However, there are plans to introduce IMT services in the 2027 following feasibility studies of 5G/6G service for WRC 27. Realistically, we would see live service after 2027. See following Figure 2.

Figure 2 Priority 1 Configuration

Situation with region 2 is slightly different:

In WRC 23 it was decided to allocate 10-10.5 GHz, on a shared basis to 12 counties in the Region 2 (Americas) for 5G/6G Services.

Furthermore, as with Regions 1 and 3, the 14.80-15.35 GHz is earmarked for 5G/6G Study for WRC 27 with possible live services after 2027.

## Priority 2 Configuration

The situation with Priority 2 is somewhat simpler as there are no IMT services in Regions 1 and 3 close to the Uplink Band A nor any services near the Downlink band. See following Figure 3

Figure 3 Priority 2 Configuration

However, in Region 2, there could be interference between the planned shared spectrum between 10-10.5 GHz for the Americas, and the Downlink of the Ku Band.

## Interferers

To study the interference between a satellite system and a terrestrial system, bearing in mind that the nearest terrestrial services may be far away from the Ku Band along the electromagnetic spectrum, we are left with a few frequency locations to conduct this co-existence study.

As Ku Band is a mature band, with live commercial, scientific, and military services, the following must be taken into consideration when planning a co-existence study:

- There is an existing radioastronomy, a scientific, and a radar band adjacent to and within the Ku Band.

- There are no allocated terrestrial band in the vicinity of the Ku Band and as such there will not be any terrestrial services in the foreseeable future around the Ku Band.

- The only area that we can perform Ku Uplink Band co-existence study would be between 13.75-14.50 GHz and the IMT Services between 14.80-15.35 GHz to be studied 2027 (this band is currently under study and no services have yet been assigned).

- The only area that we can perform Ku Downlink Band co-existence study would be in Region 2 and it would be between 10.7-12.7 GHz and the IMT Services between 10.0-10.5 GHz (assigned).

- All current radioastronomy, scientific and military services to be protected. This follows that all scientific services adjacent to the planned IMT services between 10.0-10.5 GHz must be protected.

Therefor the interferers that we could use for the study are shown in the following Figure 4 and Figure 5:

Figure 4 Priority 1 Configuration and Interfering Carriers

And

Figure 5 Priority 2 Configuration and Interfering Carriers

The following Table 3, lists the interferes that can be used for the co-existence study:

Table 3 Frequency Carriers for the Co-existence Study

| Frequency Carriers |  |  |  |
| --- | --- | --- | --- |
| Priority | Regions | Interfering Carrier (GHz) | Remarks |
| 1 | 1 and 3 | 14.80 |  |
|  |  | 14.50 |  |
|  | 2 | 14.80 |  |
|  |  | 14.50 |  |
|  |  | 10.70 |  |
|  |  | 10.50 |  |
| 2 | 1, 3 | No interferers | No live IMT Services |
|  | 2 | 10.70 |  |
|  |  | 10.50 |  |

Proposal 1: 	We propose to proceed with the co-existence study in the order of priority:

- Priority 1 Downlink with Uplink Band B in Regions 1,2 and 3 as illustrated in following Figure 6

Figure 6 Priority 1 Configuration

- Priority 2 Downlink with Uplink Band A in Regions 1, 3 and 2 excluding US, see following Figure 7

Figure 7 Priority 2 Configuration

Proposal 2:  	To streamline the analysis and to avoid duplication, we propose to use the following interfering carriers, from Table 3, for the co-existence scenarios.

- Priority 1 10.50GHz and 14.50 GHz

- Priority 2 10.50 GHz

# Interference Scenarios

Various interference scenarios are illustrated in the following Figure 6.

Figure 8 Possible Interferance Scenairos between VSAT, UE, gNodeB and the Satellite

Bearing in mind the Ku Band spectrum, see Figure 1, there are a few possible interference combinations available which are tabulated in the following Table 4

Table 4 Interference Scenarios

| No. | Combination | Aggressor | Victim | Notes on the Ku Band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | VSAT UL | UE UL | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 2 | TN with NTN | UE UL | VSAT UL | UE uplink interfering with Ku Band Satellite receiver. | The ACLR of the UE is fixed. The ACS of the satellite to be varied/defined. |
| 3 | TN with NTN | VSAT UL | gNodeB DL | Ku Band VSAT uplink interfering with UE receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the UE is fixed. |
| 4 | TN with NTN | gNodeB DL | VSAT UL | gNodeB downlink interfering with Ku Band Satellite receiver. | The ACLR of the gNodeB is fixed. The ACS of the Satellite to be varied/defined. |
| 5 | TN with NTN | gNodeB DL | Satellite DL | gNodeB downlink interfering with Ku Band VSAT receiver. | The ACLR of the gNodeB is fixed. The ACS of the VSAT to be varied/defined. |
| 6 | TN with NTN | Satellite DL | gNodeB DL | Ku Band Satellite downlink interfering with UE receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the UE is fixed. |
| 7 | TN with NTN | Satellite DL | UE UL | Ku Band Satellite downlink interfering with gNodeB receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the gNodeB is fixed. |
| 8 | TN with NTN | UE UL | Satellite DL | UE uplink interfering with Ku Band VSAT receiver. | The ACLR of the UE is fixed. The ACS of the VSAT to be varied/defined. |
| NOTE 1: For coexistence between Ku Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |

See also Section 9.1 for the illustration of the co-existence scenarios

Proposal 3: 	We propose to use the interference scenarios, as illustrated in Table 4, for the co-existence study.

# Conclusion and Proposals

In this contribution we have illustrated various scenario in which co-existence studies could be carried out in the Ku Band.

From the discussions presented in this contribution, we can conclude that:

- All mobile/terrestrial and IMT services around the Ku Band have already been assigned

- There are no vacant spectrum remaining for any possible fictitious or virtual IMT bands

- Interferers that have been illustrated can be used for co-existence analysis

- To arrive at ACLR and ACS values between the Ku Band services and non-3GPP service ITU Protection level could be used

## Proposals

We would like to propose the following:

Proposal 1: We propose to proceed with the co-existence study in the order of priority:

Priority 1 Downlink with Uplink Band B in Regions 1,2 and 3 as illustrated in following Figure 6

Figure 6 Priority 1 Configuration

Priority 2 Downlink with Uplink Band A in Regions 1, 3 and 2 excluding US, see following Figure 7

Figure 7 Priority 2 Configuration

Proposal 2:  	To streamline the analysis and to avoid duplication, we propose to use the following interfering carriers, from Table 3, for the co-existence scenarios.

- Priority 1 10.50GHz and 14.50 GHz

- Priority 2 10.50 GHz

Proposal 3: We propose to use the interference scenarios, see following Table 4,  for the co-existence study

Table 4 Interference Scenarios

| No. | Combination | Aggressor | Victim | Notes on the Ku Band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | VSAT UL | UE UL | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 2 | TN with NTN | UE UL | VSAT UL | UE uplink interfering with Ku Band Satellite receiver. | The ACLR of the UE is fixed. The ACS of the satellite to be varied/defined. |
| 3 | TN with NTN | VSAT UL | gNodeB DL | Ku Band VSAT uplink interfering with UE receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the UE is fixed. |
| 4 | TN with NTN | gNodeB DL | VSAT UL | gNodeB downlink interfering with Ku Band Satellite receiver. | The ACLR of the gNodeB is fixed. The ACS of the Satellite to be varied/defined. |
| 5 | TN with NTN | gNodeB DL | Satellite DL | gNodeB downlink interfering with Ku Band VSAT receiver. | The ACLR of the gNodeB is fixed. The ACS of the VSAT to be varied/defined. |
| 6 | TN with NTN | Satellite DL | gNodeB DL | Ku Band Satellite downlink interfering with UE receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the UE is fixed. |
| 7 | TN with NTN | Satellite DL | UE UL | Ku Band Satellite downlink interfering with gNodeB receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the gNodeB is fixed. |
| 8 | TN with NTN | UE UL | Satellite DL | UE uplink interfering with Ku Band VSAT receiver. | The ACLR of the UE is fixed. The ACS of the VSAT to be varied/defined. |
| NOTE 1: For coexistence between Ku Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |

Proposal 4 We propose to use the following Satellite parameters, see Table 1, for the co-existence study

Table 1 Ku Band Satellite Parameters

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 14 GHz for UL) | 5 m | 0.5 m | 0.5 m |
| Satellite EIRP density |  | FFS dBW/MHz | FFS dBW/MHz | FFS dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.295 deg | 2.948 deg | 2.948deg |
| Satellite beam diameter (Note 2) |  | 184 km | 62 km | 31 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 12 GHz for DL ) | 5 m | 0.5 m | 0.5 m |
| G/T |  | 28.3 dB K-1 | 8.3 dB K-1 | 8.3 dB K-1 |
| Satellite RX max Gain |  | 55.4 dBi | 35.4 dBi | 35.4 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite. NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

*Please note that the Ku Band Satellite’s EIRP density is still to be defined.

Proposal 5 We propose to use the following VSAT parameters , see Table 2, for the co-existence analysis

Table 2 Ku Band VSAT Parameters

| Characteristics | VSAT (Note 1) |
| --- | --- |
| Frequency band | Ku-band (i.e. 14 GHz UL and 12 GHz DL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 36.7 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 2 W (33 dBm) |
| Tx antenna gain | 35.1 dBi |
| NOTE 1: VSAT terminal characteristics could be implemented with phased array antenna |  |

# Reference

- R4-2400510 Ku Band Standardization: Topics on Coexistence Studies

- R4-2320973

- R4-2400509 Regulatory Analysis of the Ku Band

- ITU Regulatory Articles Clause 5.510

# Technical Annex

## Co-existence Scenarios

In this section all eight co-existence scenarios are illustrated.

Figure 9 Coexistence Scenario 1: VSAT uplink interfering with gNodeB receiver.

Figure 10 Co-existence Scenario 2: UE uplink interfering with Satellite receiver.

Figure 11 Co-existence scenario 3: VSAT uplink interfering with UE receiver.

Figure 12 Co-existence Scenario 4: gNodeB downlink interfering with Satellite receiver.

Figure 13 Co-existence Scenario 5: gNodeB downlink interfering with VSAT receiver.

Figure 14 Co-existence Scenario 6: Satellite downlink Interfering with UE receiver.

Figure 15 Co-existence Scenario 7: Satellite downlink interfering with gNodeB receiver.

Figure 16 Co-existence Scenario 8: UE uplink interfering with VSAT receiver.

## ACLR and ACS: What are we looking for?

In this series of analysis, it is essential to determine the level of RF interference from an interferer into the victim’s receiver: and this is measured as Adjacent Channel Leakage Ratio (ACLR) levels. This is measured in dBc and with respect from the carrier frequency’s carrier power levels.

For the receiver to be able to deter the interference signal, it needs to reject or attenuate the interfering signal in its pass band, and this measure in the Adjacent Channel Selectivity (ACS). The ACS is usually measured as a ratio in decibels (dB). See following Figure 15.

Figure 17 ACLR and ACS

In practical terms, the ACLR and ACS are measure of the Scattering Parameters of a typical filter characteristics where:

- ACLR is the passthrough response, also known as the S21, which represents the level of RF signal passed in the passband and in the stop band

- ACS is the attenuation level of the filter in the stop band, also know as the S11, is the level of the RF signal attenuated in the stop band

A typical Filter Response is shown in the following Figure 16

Figure 18 Typical Bandpass Filter Response

In practical implementations, the Transmit Mask and Filter characteristics, must be specified and designed within realistic and practical limits, as shown in the following Figure 17.

Figure 19 Practical Bounds of Transmit Mak and Filter Characteristics.