3GPP TSG RAN4 Meeting #112							R4-2411508

Maastricht, Netherlands 19-23 August 2024

Agenda Item:		8.9.3

Source:			Intelsat

Title:					Ku Band Parameters and evaluation of FR1-NTN and

FR2-NTN numerologies

Document for:		Discussion and Decision

# Introduction

The purpose of this document is to define the Ku Band parameters under the assumption of two numerologies: (1) FR1-NTN and (2) FR2-NTN.  The parameters include the band number, channel bandwidth, channel configuration, channel raster, Absolute Radio Frequency Channel Number (ARFCN), sync raster and Global Synchronization Channel Number (GSCN).  Specifically, these parameters are defined for Ku Band covering the following frequency ranges, considering targeted deployment scenarios:

  - Ku Band #1a for ITU regions 1 and 3:

    - Downlink 10.70 – 12.75 GHz

    - Uplink 13.75-14.5 GHz

  - Ku Band #1b for ITU regions 2 excluding US:

    - Downlink 10.70 – 12.70 GHz

    - Uplink 13.75-14.5 GHz

  - Ku Band #2a for ITU regions 1 and 3:

    - Downlink 10.70 – 12.75 GHz

    - Uplink 12.75 - 13.25 GHz

  - Ku Band #2b for ITU regions 2 excluding US:

    - Downlink 10.70 – 12.70 GHz

    - Uplink 12.70 - 13.25 GHz

Results are provided for the two potential approaches assuming Ku Band numerology are FR1-NTN or FR2-NTN, from which one shall be selected including but not limited to:

  - Ku band #1a and #1b and Ku band #2a and #2b as FR1-NTN numerology supporting 5 MHz, 10 MHz, 15 MHz, 20 MHz, 25 MHz, 30 MHz, 35 MHz, 40 MHz, 45 MHz, 50 MHz, 60 MHz, 70 MHz, 80 MHz, 90 MHz, and 100 MHz channel bandwidths with 15 kHz, 30 kHz, and 60 kHz subcarrier spacing.

  - Ku band #1a and #1b and Ku band #2a and #2b as FR2-NTN numerology supporting 50 MHz, 100 MHz, 200 MHz, and 400 MHz channel bandwidths with 60 kHz and 120 kHz subcarrier spacing.

The document is organized as follows.  Introduction materials are provided for the channel raster and ARFCN followed by synchronization raster and GSCN.  Afterwards, the band number, channel bandwidth, ARFCN and channel raster table, and GSCN and synchronization raster table are provided respectively assuming numerologies for FR1-NTN and FR2-NTN.

# Channel Raster and ARFCN

The global frequency raster (FREF) defines a set of RF reference frequencies for 5G NTN.  The RF reference frequency is essential for signalling to identify the position of RF channels, the position of the Synchronization Signal block (SSB) and other elements.  As shown in Table 1, the global frequency raster is defined for all frequencies from 0 to 100 GHz.  The granularity of the global frequency raster is DFGlobal.

Table 1: ARFCN parameters for the global frequency raster.

| Range of frequencies (MHz) | DFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 -599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |
| 24250 - 100000 | 60 | 24250.08 | 2016667 | 2016667 - 3279165 |

The global frequency raster in MHz is defined by the following equation:

FREF = FREF-Offs + DFGlobal (NREF – NREF-Offs)   [E.1]

where FREF-Offs and NREF-Offs are given in Table 1, and NREF is the Absolute Radio Frequency Channel Number (ARFCN) - code that specifies a pair of reference frequencies used for transmission and reception (see TS 138.104 Table 5.4.2.1-1).  In a FDD system one ARFCN number is required for downlink and another for uplink because the downlink and uplink frequencies are different.  Conversely, the ARFCN can be calculated using E.1 given the global frequency raster:

NREF = (FREF -FREF-Offs)/DFGlobal + NREF-Offs   [E.2]

As shown in Table 1, the granularity of the global frequency raster at Ku Band is 15kHz.  The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. For each operating band, a subset of frequencies from the global frequency raster are applicable for that Band and forms a channel raster with a granularity DFRaster, which may be equal to or larger than DFGlobal.

As an example, Table 2 provides the channel frequency raster and ARFCN for Ku Band #1a downlink ranging from 10700 - 12750 MHz where the spacing between each channel raster frequency is 15kHz.  There are 136,667 unique ARFCN values inside this band.

Table 2: Example of global frequency raster and ARFCN at Ku Band downlink (10700 - 12750 MHz)

| Channel raster in MHz | ARFCN (NREF) |
| --- | --- |
| 10700.01 | 1113334 |
| 10700.03 | 1113335 |
| 10700.04 | 1113336 |
| 10700.06 | 1113337 |
| 10700.07 | 1113338 |
| 10700.09 | 1113339 |
| 10700.10 | 1113340 |
| 10700.12 | 1113341 |
| 10700.13 | 1113342 |
| 10700.15 | 1113343 |
| 10700.16 | 1113344 |
| 10700.18 | 1113345 |
| 10700.19 | 1113346 |
| 10700.21 | 1113347 |
| 10700.22 | 1113348 |
| *** | *** |
| 12749.81 | 1249987 |
| 12749.82 | 1249988 |
| 12749.84 | 1249989 |
| 12749.85 | 1249990 |
| 12749.87 | 1249991 |
| 12749.88 | 1249992 |
| 12749.90 | 1249993 |
| 12749.91 | 1249994 |
| 12749.93 | 1249995 |
| 12749.94 | 1249996 |
| 12749.96 | 1249997 |
| 12749.97 | 1249998 |
| 12749.99 | 1249999 |
| 12750.00 | 1250000 |

# Synchronization Raster and GSCN

The synchronization raster indicates the frequency positions of the synchronization block that can be used by the UE for system acquisition.  As shown in Table 3 (see TS 138.104 Table 5.4.3.1-1), the global synchronization raster is defined for all frequencies from 0 to 100 GHz.  The frequency position of the Synchronization Signal block (SSB) is defined by SSREF with the corresponding Global Synchronization Channel Number (GSCN). The GSCN identifies the center frequency for the SSB.  The parameters defining the SSREF and GSCN for all the frequency ranges are provided in Table 3.  The SS block frequency position spacing for Ku Band is 1.44 MHz.  The granularity of synchronization raster is wider than that of global frequency raster.

Table 3: GSCN parameters for the global frequency raster.

| Range of frequencies (MHz) | SS block frequency position SSREF | GSCN | Range of GSCN |
| --- | --- | --- | --- |
| 0 – 3000 | N * 1200 kHz + M * 50 kHz, N = 1:2499, M ϵ {1,3,5} | 3N + (M-3)/2 | 2 – 7498 |
| 3000 – 24250 | 3000 MHz + N * 1.44 MHz, N = 0:14756 | 7499 + N | 7499 – 22255 |
| 24250 – 100000 | 24250.08 MHz + N * 17.28 MHz, N = 0:4383 | 22256 + N | 22256 – 26639 |

As an example, Table 4 provides the index N, SS block frequency position (SSREF), and GSCN for Ku Band #1a downlink ranging from 10700 - 12750 MHz assuming step size of one.  The GSCN considers the guard band at the beginning and end of the band.  There are 1391 unique GSCN values inside this band.

Table 4: Example of sync raster and GSCN at Ku Band downlink (10700 - 12750 MHz)

| N | SS block frequency (MHz) | GSCN |
| --- | --- | --- |
| 5364 | 10724.16 | 12863 |
| 5365 | 10725.60 | 12864 |
| 5366 | 10727.04 | 12865 |
| 5367 | 10728.48 | 12866 |
| 5368 | 10729.92 | 12867 |
| 5369 | 10731.36 | 12868 |
| 5370 | 10732.80 | 12869 |
| 5371 | 10734.24 | 12870 |
| 5372 | 10735.68 | 12871 |
| 5373 | 10737.12 | 12872 |
| 5374 | 10738.56 | 12873 |
| 5375 | 10740.00 | 12874 |
| 5376 | 10741.44 | 12875 |
| 5377 | 10742.88 | 12876 |
| 5378 | 10744.32 | 12877 |
| *** | *** | *** |
| 6741 | 12707.04 | 14240 |
| 6742 | 12708.48 | 14241 |
| 6743 | 12709.92 | 14242 |
| 6744 | 12711.36 | 14243 |
| 6745 | 12712.80 | 14244 |
| 6746 | 12714.24 | 14245 |
| 6747 | 12715.68 | 14246 |
| 6748 | 12717.12 | 14247 |
| 6749 | 12718.56 | 14248 |
| 6750 | 12720.00 | 14249 |
| 6751 | 12721.44 | 14250 |
| 6752 | 12722.88 | 14251 |
| 6753 | 12724.32 | 14252 |
| 6754 | 12725.76 | 14253 |

# Ku Band Numerology FR1-NTN

With the assumption that Ku Band numerology is designated as FR1-NTN, the band number, channel bandwidth, ARFCN and channel raster tables, and GSCN and synchronization raster table are defined in the following subsections.

## Band number.

The FR1-NTN Ku Band are illustrated in Table 5 for the different ITU regions band numbering.  For all cases, the duplex mode is FDD. The band number has been defined as nXYZ1, nXYZ2, nXYZ3, and nXYZ4 for placeholder until the band numbers are formally defined.

Table 5: FR1-NTN band definition.

| Band number | Downlink (DL) | Uplink (UL) | Duplex mode | ITU region |
| --- | --- | --- | --- | --- |
| nXYZ1 | 10700 – 12750 MHz | 13750 – 14500 MHz | FDD | #1a for ITU regions 1 and 3 |
| nXYZ2 | 10700 – 12700 MHz | 13750 – 14500 MHz | FDD | #1b for ITU regions 2 ex US |
| nXYZ3 | 10700 – 12750 MHz | 12750 – 13250 MHz | FDD | #2a for ITU regions 1 and 3 |
| nXYZ4 | 10700 – 12700 MHz | 12700 – 13250 MHz | FDD | #2b for ITU regions 2 ex US |

## Channel Bandwidth

The FR1-NTN Ku Band channel bandwidths are illustrated in Table 6 for the different NTN bands.  The channel bandwidths are defined as both BS channel bandwidths and UE channel bandwidths except where indicated.  Subcarrier spacing of 15kHz, 30kHz and 60kHz are assumed for FR1-NTN. For channel bandwidths of 60 MHz, 70 MHz, 80 MHz, 90 MHz, and 100 MHz, 15kHz SCS is not supported because the size of the FFT exceeds 4096 points. The SSB is not defined with 60kHz SCS.

Table 6: FR1-NTN channel bandwidth.

| FR1-NTN band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band number | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| nXYZ1 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| nXYZ2 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| nXYZ3 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
| nXYZ4 | 15 | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |
|  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |

## ARFCN and Channel Raster

The FR1-NTN Ku Band RF channel positions on the channel raster are respectively given through the applicable ARFCN in Table 7 (downlink) and Table 8 (uplink) for each NTN operating band.  The last column in the tables provides the total number of NREF in the NTN band.

Table 7: FR1-NTN ARFCN per downlink Ku operating band.

| NTN operating band | ΔFRaster (kHz) | Downlink range of NREF (First - <Step size>- Last) | Number of NREF |
| --- | --- | --- | --- |
| nXYZ1 | 60 | 1113334 - <4> - 1249998 | 34167 |
|  | 120 | 1113334 - <8> - 1249998 | 17084 |
| nXYZ2 | 60 | 1113334 - <4> - 1246666 | 33334 |
|  | 120 | 1113334 - <8> - 1246662 | 16667 |
| nXYZ3 | 60 | 1113334 - <4> - 1249998 | 34167 |
|  | 120 | 1113334 - <8> - 1249998 | 17084 |
| nZYZ4 | 60 | 1113334 - <4> - 1246666 | 33334 |
|  | 120 | 1113334 - <8> - 1246662 | 16667 |

Table 8: FR1-NTN ARFCN per uplink Ku operating band.

| NTN operating band | ΔFRaster (kHz) | Uplink range of NREF (First - <Step size>- Last) | Number of NREF |
| --- | --- | --- | --- |
| nXYZ1 | 15 | 1316667 - <1> - 1366666 | 50000 |
|  | 30 | 1316667 - <2> - 1366665 | 25000 |
| nXYZ2 | 15 | 1316667 - <1> - 1366666 | 50000 |
|  | 30 | 1316667 - <2> - 1366665 | 25000 |
| nXYZ3 | 15 | 1250000 - <1> - 1283333 | 33334 |
|  | 30 | 1250000 - <2> - 1283332 | 16667 |
| nXYZ4 | 15 | 1246667 - <1> - 1283333 | 36667 |
|  | 30 | 1246667 - <2> - 1283333 | 18334 |

## GSCN and Synchronization Raster

The FR1-NTN Ku Band SS block SCS, SS block pattern, GSCN range/step size, and the number of GSCN in the band are illustrated in Table 9.  The distance between applicable GSCN entries is given by the <step size> as indicated in Table 9.  The SS Block patterns are defined in clause 4.1 in TS 38.213 but with frequency range up to Ku Band instead of 6 GHz.  In Ku Band, minimizing the number of GSCN is essential to reduce the UE initial search time for the Synchronization Signal block (SSB).

Table 9: FR1-NTN Applicable SS raster entries per operating band.

| NTN operating band | SS Block SCS | SS Block Pattern | Range of GSCN | Number of GSCN |
| --- | --- | --- | --- | --- |
| nXYZ1 | 15 kHz | Case A | 12863 – <1> – 14253 | 1391 |
|  | 30 kHz | Case B, Case C | 12880 – <2> – 14236 | 679 |
| nXYZ2 | 15 kHz | Case A | 12863 – <1> – 14218 | 1356 |
|  | 30 kHz | Case B, Case C | 12880 – <2> – 14200 | 661 |
| nXYZ3 | 15 kHz | Case A | 12863 – <1> – 14253 | 1391 |
|  | 30 kHz | Case B, Case C | 12880 – <2> – 14236 | 679 |
| nXYZ4 | 15 kHz | Case A | 12863 – <1> – 14218 | 1356 |
|  | 30 kHz | Case B, Case C | 12880 – <2> – 14200 | 661 |

# Ku Band Numerology FR2-NTN

With the assumption that Ku Band numerology is designated as FR2-NTN, the band number, channel bandwidth, ARFCN and channel raster tables, and GSCN and synchronization raster table are defined in the following subsections.

## Band Number

The FR2-NTN Ku Band are illustrated in Table 10 for the different ITU region band number.  For all cases, the duplex mode is FDD. The band number has been defined as nXYZ5, nXYZ6, nXYZ7, and nXYZ8 for placeholder until the band numbers are formally defined.

Table 10: FR2-NTN band definition.

| Band number | DL | UL | Duplex mode | ITU region |
| --- | --- | --- | --- | --- |
| nXYZ5 | 10700 – 12750 MHz | 13750 – 14500 MHz | FDD | #1a for ITU regions 1 and 3 |
| nXYZ6 | 10700 – 12700 MHz | 13750 – 14500 MHz | FDD | #1b for ITU regions 2 ex US |
| nXYZ7 | 10700 – 12750 MHz | 12750 – 13250 MHz | FDD | #2a for ITU regions 1 and 3 |
| nXYZ8 | 10700 – 12700 MHz | 12700 – 13250 MHz | FDD | #2b for ITU regions 2 ex US |

## Channel Bandwidth

The FR2-NTN Ku Band channel bandwidths are illustrated in Table 11 for the different NTN bands.  The channel bandwidths are defined as both BS channel bandwidths and UE channel bandwidths except where indicated.  Subcarrier spacing of 60kHz and 120kHz are assumed for FR2-NTN.  The SSB is not defined with 60kHz SCS.

Table 11: FR2-NTN channel bandwidth.

| FR2-NTN band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| band number | SCS kHz | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| nXYZ5 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| nXYZ6 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| nXYZ7 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |
| nXYZ8 | 60 | Yes | Yes | Yes |  |
|  | 120 | Yes | Yes | Yes | Yes |

## ARFCN and Channel Raster

The FR2-NTN Ku Band RF channel positions on the channel raster are respectively given through the applicable ARFCN in Table 12 (downlink) and Table 13 Table 8 (uplink) for each NTN operating band.  The ARFCN values for FR2-NTN are identical to ARFCN for FR1-NTN illustrated in Table 7 and Table 8.  The last column in the tables provides the total number of NREF in the NTN band.

Table 12: FR2-NTN ARFCN per downlink Ku operating band.

| NTN operating band | ΔFRaster (kHz) | Downlink range of NREF (First - <Step size>- Last) | Number of NREF |
| --- | --- | --- | --- |
| nXYZ5 | 60 | 1113334 - <4> - 1249998 | 34167 |
|  | 120 | 1113334 - <8> - 1249998 | 17084 |
| nXYZ6 | 60 | 1113334 - <4> - 1246666 | 33334 |
|  | 120 | 1113334 - <8> - 1246662 | 16667 |
| nXYZ7 | 60 | 1113334 - <4> - 1249998 | 34167 |
|  | 120 | 1113334 - <8> - 1249998 | 17084 |
| nXYZ8 | 60 | 1113334 - <4> - 1246666 | 33334 |
|  | 120 | 1113334 - <8> - 1246662 | 16667 |

Table 13: FR2-NTN ARFCN per uplink Ku operating band.

| NTN operating band | ΔFRaster (kHz) | Uplink range of NREF (First - <Step size>- Last) | Number of NREF |
| --- | --- | --- | --- |
| nXYZ5 | 15 | 1316667 - <1> - 1366666 | 50000 |
|  | 30 | 1316667 - <2> - 1366665 | 25000 |
| nXYZ6 | 15 | 1316667 - <1> - 1366666 | 50000 |
|  | 30 | 1316667 - <2> - 1366665 | 25000 |
| nXYZ7 | 15 | 1250000 - <1> - 1283333 | 33334 |
|  | 30 | 1250000 - <2> - 1283332 | 16667 |
| nXYZ8 | 15 | 1246667 - <1> - 1283333 | 36667 |
|  | 30 | 1246667 - <2> - 1283333 | 18334 |

## GSCN and Synchronization Raster

The FR2-NTN Ku Band SCS, SS block pattern, GSCN range/step size, and number of GSCN in the band are illustrated in Table 14.  The distance between applicable GSCN entries is given by the <Step size> indicated in  Table 14.  The GSCN values for FR2-NTN are slightly different from FR1-NTN because the SCS assumptions are different.  In Ku Band, minimizing the number of GSCN is essential to reduce the UE initial search time for the Synchronization Signal block (SSB).

Table 14: FR2-NTN Applicable SS raster entries per operating band.

| NTN operating band | SS Block SCS | SS Block Pattern (note) | Range of GSCN | Number of GSCN |
| --- | --- | --- | --- | --- |
| nXYZ5 | 120 kHz | Case D | 12864 < 12 < 14256 | 117 |
| nXYZ6 | 120 kHz | Case D | 12864 < 12 < 14220 | 114 |
| nXYZ7 | 120 kHz | Case D | 12864 < 12 < 14256 | 117 |
| nXYZ8 | 120 kHz | Case D | 12864 < 12 < 14220 | 114 |

# Discussion

The document defined the Ku Band parameters for two numerologies, namely: FR1-NTN and FR2-NTN with the understanding that one of the two numerologies shall be down selected at a future date.  The calculations presented in this contribution, as agreed in the WID[1], avoids any new bands in FR1-NTN and FR2-NTN numerologies - thus there is no impact on the physical layer and nor on RAN1 Working Group.

For FR1-NTN Numerology:

- The band definition illustrated in Table 5 show the band number for the four Ku Band combinations of uplink and downlink frequencies.  The band numbers provided are a placeholder until they are formally defined.

- The channel bandwidths illustrated in Table 6 show the widely used bandwidth for various services and applications.

- The ARFCN for downlink and uplink illustrated in Table 7 and Table 8 respectively show standard ranges and step sizes.

- The GSCN illustrated in Table 9 show standard ranges and step sizes for different subcarrier spacings.

For FR2-NTN Numerology:

- The band definition illustrated in Table 10 show the band number for the four Ku Band combinations of uplink and downlink frequencies.  The band numbers provided are a placeholder until they are formally defined.

- The channel bandwidths illustrated in Table 11 show the widely used bandwidth for various services and applications.

- The ARFCN for downlink and uplink illustrated in Table 12 and Table 13 respectively show standard ranges and step sizes.

- The GSCN illustrated in Table 14 show standard ranges and step sizes for different subcarrier spacings.

The current FR2 Numerology and corresponding available bandwidths do not provide a wide-ranging option for a varied service offering.

# Proposals and Conclusions

In this contribution we have illustrated several channel bandwidths and other fundamental band parameters under FR1-NTN and FR2-NTN Numerologies for the Ku Band.

The NTN Band numbering shown in the tables are yet to be agreed by RAN4.

We propose to capture the band calculations illustrated in this contribution for the Ku Band.

# References

[1]	RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104,