3GPP TSG-RAN4 Meeting #112	R4-2411512

Maastricht, Netherlands, 21st – 25th August, 2024

Agenda item:	8.19.1

Source: 	Qualcomm Incorporated

Title: 	Views on general aspects for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#111, a WF was agreed in [3] based on some of the topics listed in the moderator summary, that is summarized in [2]. In this contribution we share our views on general aspects for for Rel-19 SBFD WI within RAN4.

# General aspects

## SBFD as band specific or general feature to all TDD

In [2], two options were considered as shown below on how to specify SBFD bands in the core specification TS 38.104, TS 38.141-1, and TS 38.141-2.

- SBFD is a feature with requirements of which can potentially be applied to all TDD band, under the following considerations: - Option 1: Declaration based method - The band supported for SBFD shall be manufacturer declaration based - The channel bandwidth supported for SBFD shall be manufacturer declaration based - The supported channel bandwidth can be impacted by the subband/guard band size discussion - Option 2: General principle but with consideration(s) for a specific band - General principle: The channel bandwidth shall be larger than X MHz - FFS the value of X - X can be different for high and middle TDD bands - For a specific band, the following further restriction(s) can be considered: - Certain restriction can be provided by operator(s) for certain band - Other restrictions are not precluded.

In general, any TDD defined band in Section 5.2 in TS 38.104 could be a target band for SBFD operation, since SBFD is as an enhancement of TDD duplex operation via allowing the simultaneous existence of downlink and uplink subband at the gNB side within a TDD carrier in a conventional TDD band (hence, the term subband non-overlapping full duplex). This can be accommodated under option 1, where vendors declare which bands can support SBFD feature. This is still applicable when the following note is added in TS 38.104 “subband full duplex can be applied to TDD bands given in Table 5.2-1.”.

Observation 1: SBFD can be operated in any TDD with sufficient channel bandwidth.

Proposal 1: If RAN4 agreed to consider SBFD support for a given TDD band as declaration based, it is viable to include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with bandwidth > X MHz, given in Table 5.2-1.

For option 2, our interpretation that a given TDD band can support SBFD under some general constraints (e.g., channel bandwidth is larger than X MHz). Such constraint is viable as it is not foreseen that bands with low channel bandwidths (e.g., less than 50MHz) will be attractive for SBFD deployments. For the restriction point captured in option 2, it is not clear to us how would RAN4 decide a given restriction for a given band, aside from its channel bandwidth. RAN4 need to consider a generic, technical criteria for the exclusion of given TDD bands when considering SBFD feature.

Proposal 2: RAN4 should consider a band-generic and technical criteria for exclusion of certain TDD bands when considering the SBFD feature. Such a natural criterion is the band bandwidth being less than X MHz.

For the given value of X, during the SI phase, RAN4 has generally considered channel bandwidth values of 100MHz and 200MHz for FR1 and FR2-1, respectively, with a given DL and UL ratio of 80% and 20%, respectively. Since RAN4 is still discussing the need to specify a minimum subband configuration size for a given direction and the minimum required guardband, it is a viable direction to decide on the value of X after agreeing on the guardband value and the corresponding subband sizes for a given SBFD configuration.

Proposal 3: RAN4 to agree on the value of X after agreeing on the minimum guard band requirements and subband sizes for both DUD and DU subband configurations.

## CLI handling impact on RF requirement

In [2], the following options were agreed regarding impact of CLI handling on RF requirement:

- Option 1: RAN4 define RF requirements for the frequency outside of wanted carrier based on no CLI handling as baseline. - Option 2: FFS CLI handling impact on RF requirements

Currently RAN1 is investigating several techniques related to inter-gNB and inter-UE CLI handling techniques (see RP-241286 for more information), such as beam nulling, non-transparent UL resource muting, spatial-domain based schemes for FR2-1, etc. Since it is still work in progress in RAN1, it is difficult to quantify the impact of the different CLI handling techniques on the definition of the different RF requirements within RAN4. Accordingly, it is foreseeable that RAN4 would define RF requirements with no CLI handling as a baseline and FFS whether any of those redefined CLI handling needs to be revisited pending on RAN1 progress on the CLI handling techniques.

Proposal 4: RAN4 to define RF requirements with no CLI handling as a baseline and revisited particular RF requirements pending on RAN1 progress on the CLI handling work.

## Guardband considerations

In [2], the following was agreed regarding how to handle guard band and subband configurations in specification meeting the following related to the semi-static indication of SBFD frequency location​ was agreed:

- Add clarification that only DUD and DU patterns are specified for the sub-band configuration. - FFS which channel bandwidth(s) or all channel bandwidths shall be defined for RF requirements - For a certain channel bandwidth which RAN4 agree to introduce RF requirements: - FFS RAN4 only define the UL/DL subbands configuration(s) for RF requirements - FFS which UL/DL subbands configuration(s) will be defined in RAN4 - FFS Guard band size is declaration based and can be different for different BS classes - FFS the limitation on the maximum guard band - FFS possible range for UL/DL subband sizes - From RAN4 perspective, FFS restriction or no restriction to RAN1 definition for UL/DL subband sizes within the transmission configuration for this channel bandwidth, except: - 1RB granularity (already introduced in RAN1)

For an SBFD-capable gNB, a guardband may be needed at the gNB to protect UL reception within the UL subband and reduce the impact of self-interference. Generally, there is a dependency between the amount of guardband needed and the gNB self-interference cancellation capability. Additionally, how much guardband is needed also depends on the employed subband configuration (e.g., DUD configuration might have different guardband assumptions compared to DU configuration). Accordingly, RAN4 should first address how to capture the subband configuration aspect prior to specifying the guardband requirements. In our views, the subband configurations for FR1 and FR2-1 should be captured within Clause 5.3 of TS 38.104 (BS Channel Bandwidth). So far, the typical subband configurations that has been studied within the SI phase are DUD and DU configurations. For the question of which channel bandwidth(s) or all channel bandwidths shall be defined for RF requirements, we propose for RAN4 to consider minimum the definition of the RF requirements as a function of the supported channel bandwidth. Hence, the definition of SBFD-capable gNB RF requirements should be done for all channel bandwidth supported according to the minimum channel bandwidth criteria (i.e., X MHz).

Proposal 5: RAN4 to define RF requirements for all channel bandwidths that satisfy the minimum channel bandwidth criteria (i.e., larger than X MHz).

Regarding the definition of the UL/DL subbands configurations for RF requirements, it is viable to consider a configuration-agnostic approach, such as the RF requirements are defined independent of the subband configuration. The supported configuration themselves can be declaration-based in order not to restrict SBFD deployment to a given set of configurations.

Observation 2: An agnostic configuration approach in relation to the subband configuration should be considered in order not to restrict the normative work to a set of subband configurations.

Additionally, RAN4 would need to specify the portion of each subband. A typical number that was assumed during the SI was 80% for the DL subband and 20% for the UL subband. There was no agreement in [4] on whether the subband bandwidth for either DL or UL should be matching to the supported channel bandwidth. Also from RAN1 perspective, no restriction is considered in the normative phase on the sizes of the DL/UL subband configurations. It is desirable that RAN4 follow similar approach when defining the RF requirements for the gNB-capable SBFD.

Proposal 6: RAN4 to consider no restriction on the subband sizes when defining the RF requirements.

## BS RF Specification structure for SBFD requirements

In [2], the following was agreed regarding how to include SBFD requirements in the core specifications

- How to introduce BS RF new requirements for SBFD-capable BS: - FFS firstly the feasibility of creating new and standalone sub-clauses (e.g., with a suffix) in TS 38.104 for SBFD-specific existing or new gNB RF requirements - FFS detailed how to implement such approach in the later phase of WI. - FFS secondly the new specification for SBFD-capable BS

In our view, option one, i.e., creating new and standalone sub-clauses, is preferred as it makes the readability of the spec easier and aligns with UE core specification (TS 38.101-1). Such an approach has also been adopted in other studies (e.g., incorporation of mobile IAB feature into the IAB core and conformance specs TS 38.174, TS 38.176-1, and TS 38.176-2). An additional clause (e.g., Suffix B) should be used for indicating SBFD-specific requirements. Additionally, a SBFD-capable gNB, which supports the SBFD feature needs to meet both the general requirements in the core spec and the additional requirement contained in the additional clause (suffixes B). In case there is a difference in requirement between the general requirements and the additional clause requirements (suffix B), the tighter requirements are applicable unless stated otherwise in the additional clause.

Proposal 7: To enhance the readability of the spec, it is preferable to have standalone sub-clauses in core specs for SBFD-specific existing or new gNB RF requirements.

# Conclusion

In this contribution, we provided our views on general aspects for SBFD operation with the following observations and proposals:

Observation 1: SBFD can be operated in any TDD with sufficient channel bandwidth.

Proposal 1: If RAN4 agreed to consider SBFD support for a given TDD band as declaration based, it is viable to include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands given in Table 5.2-1.

Proposal 2: RAN4 should consider a band-generic and technical criteria for exclusion of certain TDD bands when considering the SBFD feature. Such a natural criterion is the band bandwidth being less than X MHz.

Proposal 3: RAN4 to agree on the value of X after agreeing on the minimum guard band requirements and subband sizes for both DUD and DU subband configurations.

Proposal 4: RAN4 to define RF requirements with no CLI handling as a baseline and revisited particular RF requirements pending on RAN1 progress on the CLI handling work.

Proposal 5: RAN4 to define RF requirements for all channel bandwidths that satisfy the minimum channel bandwidth criteria (i.e., larger than X MHz).

Observation 2: An agnostic configuration approach in relation to the subband configuration should be considered in order not to restrict the normative work to a set of subband configurations.

Proposal 6: RAN4 to consider no restriction on the subband sizes when defining the RF requirements.

Proposal 7: To enhance the readability of the spec, it is preferable to have standalone sub-clauses in core specs for SBFD-specific existing or new gNB RF requirements.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, RAN4#111, Samsung.

- R4-2410108, “Topic summary for [111][313] NR_duplex_evo”, RAN4#111, Samsung.

- TR 38.858, “Study on evolution of NR duplex operation”, 3GPP.