3GPP TSG-RAN4 Meeting #112	R4-2411513

Maastricht, Netherlands, 21st – 25th August, 2024

Agenda item:	8.19.2.1

Source: 	Qualcomm Incorporated

Title: 	Views on new requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#110-bis, a WF was agreed in [2]. In this contribution we share our views on existing BS RF requirements for Rel-19 SBFD.

# New BS RF requirements for SBFD operation

## Requirement for transient period between SBFD and non-SBFD

In [2] the following was agreed:

- Agreement: - FFS requirement and cases for transient period between SBFD and non-SBFD: - Option 1: The existing TDD BS transmitter transient period, i.e., 10us for FR1 and 3us for FR2-1 - Other options are not precluded. - FFS location of transient period between SBFD and non-SBFD: - Option 1: The transmitter transient period shall be located within the SBFD slot - Other options are not precluded.

In our understanding the motivation of proposing this new requirement is that SBFD-capable gNB might need additional time to adjust its operation when transitioning between SBFD and non-SBFD slot/symbol. Such time might be needed as an example to adjust the Tx/Rx arrays considered for SBFD and legacy DL operation, update the Tx power, modify the beamforming algorithms, etc.

Observation 1: Additional time might be needed for an SBFD-capable gNB to adjust its operation when transitioning between SBFD and non-SBFD slot/symbol.

For transmitter transient period between SBFD and non-SBFD, the following cases are identified regarding the transitioning between SBFD and non-SBFD slot/symbol:

(1)    Case-A (SBFD to DL): SBFD-capable gNB Tx OFF-to-ON in SBFD UL band and guard band(s)

(2)    Case-B (SBFD to UL): SBFD-capable gNB Tx ON-to-OFF in SBFD DL band

(3)    Case-C (DL to SBFD): SBFD-capable gNB Tx ON-to-OFF in SBFD UL band and guard band(s)

(4)    Case-D (UL to SBFD): SBFD-capable gNB Tx OFF-to-ON in SBFD DL band

Regarding the appropriate values of the transmit transient period. Recall that for legacy TDD gNBs, the transmitter transient period represents the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa, with the following requirements (as captured in TS 38.104). For the SBFD-capable gNB, we think it is viable to reuse the existing TDD legacy transient period requirements in Section 6.4.2.2 and 9.5.3 for the FR1 and FR2-1.

Proposal 2: RAN4 to adopt legacy TDD transient period requirements in Section 6.4.2.2 and 9.5.3 for the FR1 and FR2-1 SBFD-capable gNB.

In terms of the requirement definition, at this stage, it is preferable that RAN4 would consider the requirement applicable independent of a given SBFD configuration (i..e, DUD or DU).

Proposal 3: RAN4 to define transient period requirements impendent of a given SBFD configuration.

## Additional new requirements

As discussed in Section 10.1.4 [3], the following requirements are expected to be added for SBFD BS:

- In-channel adjacent subband leakage ratio

- In-channel adjacent subband Blocking and adjacent subband selectivity

- Transmitter transient period

Within in-band operation, the in-channel adjacent subband leakage will impact the receiving subband of the same BS. Such leakage might contribute to REFSENSE. It was noted during the SI [3] that OTA REFSENSE will be derived considering self-interference, inter-site and inter-sector interference. The in-channel adjacent subband leakage (selectivity) is the same as the ACLR (ACS), but on a subband in-channel granularity level. Consequently, there is a direct implication of the coexistence scenarios and assumptions on the subband leakage and selectivity definition.

Observation 2: The in-channel subband sub-band Tx leakage and selectivity new requirements depend on the self-interference, inter-site and inter-sector interference considerations.

During the SI phase, to progress the adjacent channel coexistence, RAN4 agreed to reuse legacy ACLR and ACS requirement for the in-channel adjacent subband leakage and selectivity. There was lengthy discussions on whether to consider some enhanced performance at the BS side, owing to additional filtering or enhanced interference mitigation capabilities. Accordingly, RAN4 is expected to define scenarios and assumptions upon which the new requirements will be defined for. 

Proposal 4: RAN4 to first define scenarios and assumptions required for the definition of the in-channel adjacent subband leakage and selectivity.

# Conclusion

In this contribution, we provided our views on BS RF requirements for SBFD with the following observations and proposals:

Observation 1: Additional time might be needed for an SBFD-capable gNB to adjust its operation when transitioning between SBFD and non-SBFD slot/symbol.

Proposal 2: RAN4 to adopt legacy TDD transient period requirements in Section 6.4.2.2 and 9.5.3 for the FR1 and FR2-1 SBFD-capable gNB.

Proposal 3: RAN4 to define transient period requirements impendent of a given SBFD configuration.

Observation 2: The in-channel subband sub-band Tx leakage and selectivity new requirements depend on the self-interference, inter-site and inter-sector interference considerations.

Proposal 4: RAN4 to first define scenarios and assumptions required for the definition of the in-channel adjacent subband leakage and selectivity.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, RAN4#111, Samsung.

- R4-2410108, “Topic summary for [111][313] NR_duplex_evo”, RAN4#111, Samsung.

- TR 38.858, “Study on evolution of NR duplex operation”, 3GPP.