3GPP TSG-RAN4 Meeting #112	R4-2411514

Maastricht, Netherlands, 21st – 25th August, 2024

Agenda item:	8.19.2.2

Source: 	Qualcomm Incorporated

Title: 	Views on existing Tx BS RF requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#111, a WF was agreed in [3] based on some of the topics listed in the moderator summary, that is summarized in [2]. In this contribution, we share our views on existing BS RF requirements for Rel-19 SBFD.

# Impact on existing BS RF requirements

## Tx requirements

### The necessity of Co-location ACLR requirement

In [2], the following was agreed:

- RAN4 further study the ACLR requirement by applying the existing ACLR requirement for SBFD-capable BS in SBFD symbols/slots - FFS the applicable deployment scenario where different ACLR requirement could be useful, whether it justify the different ACLR requirement.

In Section 11 in [4], RAN4 has concluded that outcome of the SBFD and legacy TDD adjacent channel coexistence. To ensure feasible coexistence, RAN4 has concluded that no coexistence issues are identified when coexisting with legacy TDD systems in the adjacent channel. This under the working condition, considered in Rel-19 normative phase, that the SBFD is allowed to be operated in DL and Flexible symbols. Accordingly, adopting legacy ACLR requirement should ensure feasible adjacent channel coexistence.

Observation 1: Adopting legacy gNB ACLR requirement for the SBFD-capabe gNB is sufficient to ensure feasible coexistence with adjacent channel deployed legacy TDD systems.

On the other hand, for the coexistence case with gNB-capable SBFD BS in the adjacent channel, RAN4 has identified coexistence challenges due to inter-gNB CLI on the UL subbands. Within the SI phase, RAN4 has considered only ACIR enhancements as a solution to ensure feasible coexistence.

Observation 2: When coexisting with SBFD-capable gNB, legacy ACLR requirement might not be sufficient to ensure feasible coexistence. Additionally, RAN4 has only considered ACIR enhancement as a possible solution during the SI phase.

As a way forward, it is viable to consider urban macro deployment since it is the most challenging deployment scenario. RAN4 then should consider a case where additional new ACLR requirement is considered to allow feasible coexistence with new SBFD-capable gNB. However, RAN4 need to consider the overall ACIR (gNB ACLR/ACS) in order to determine the appropriate ACLR value.

Proposal 1: RAN4 to consider additional ACLR requirement to ensure feasible coexistence with SBFD-capable gNB. As a starting point, urban macro deployments should be considered as that represent the challenging scenario.

### Unwanted emissions

In [2], the following was agreed regarding the OBUE study for SBFD-capable gNB:

- Further discuss how to define OBUE requirement for SBFD capable BS in order to ensure feasible co-existence performance for SBFD BSs operating in adjacent frequency.

Similar to the ACLR requirement, it is a viable way forward to consider different OBUE requirements, one for coexistence with legacy TDD BSs and the second for coexistence with SBFD-capable BSs. For the former, legacy BS OBUE requirement should be considered as a starting point. For coexistence with SBFD-capable BS, RAN4 should consider similar deployment scenario (i.e., UMa or wide-area BS) as a starting point to quantify what is the level of enhancement requirement for the OBUE requirement to ensure feasible coexistence.

Proposal 2: For OBUE, RAN4 to consider similar approach as ACLR (i.e., two different requirements to address coexistence with legacy TDD and SBFD-capable gNBs.

## Rx Requirements

### In-band selectivity and blocking

As agreed, in [2], the in-band blocking requirements are expected to be derived based on RAN4 adjacent channel coexistence as agreed below:

- Companies are encouraged to provide the evaluation results for in-band blocking in the next meeting: - The scenarios to be studied are Scenario 1, 3, 5, 6, 9, defined in TR 38.858 - The case to be studied is Case 3 (TDD DL to SBFD UL subband). - FFS the power level which should be collected from the simulation results to derive in-band blocking levels: - Option-1: absolute power level of interference from adjacent channel only, i.e., SBFD network internal interference should not be considered in the simulation, from the perspective of definition of in-band blocking requirement. - Other options are not precluded. - FFS the reference point for the power level which should be before or after RX beamforming. - Option-1: after RX subarray beamforming gain and before array gain, i.e, at TAB. - FFS different grid-shift values should be considered. - Option-1: 10%. - Option-2: Other percentage values are not precluded. - FFS how to use the power level probability distributions to derive the in-band blocking levels - Option-1: Find the X% tile of the UL SBFD wideband received power CDF to define the blocking requirement in each of the down-selected scenarios. - FFS the value of X - FFS the impact of CLI handling scheme introduced in RAN1.

Based on the WF; the following scenarios, captured in [4], are to be studied through the normative phase, with the corresponding BS class for each scenario, for the case with TDD DL to SBFD UL subband.

| Scenario | FR | Aggressor | Victim | Corresponding BS class |
| --- | --- | --- | --- | --- |
| 1 | FR1 | Urban Macro | Urban Macro | Wide area BS |
| 3 | FR1 | Indoor | Indoor | Local area BS |
| 5 | FR1 | Micro | Micro | Medium range BS |
| 6 | FR2-1 | Urban Macro | Urban Macro | Wide area BS |
| 9 | FR2-1 | Indoor | Indoor | Local area BS |

We consider the same simulation framework that is in [4] Annex E with the following highlights:

- UMa with 50% GS

- Option 1 in the LoS probability

- Baseline AAS configuration considering two panels with 46 dBm conducted power per polarization

The metric considered on the x-axis is the absolute power level of interference from adjacent channel only, i.e., SBFD network internal interference should not be considered in the simulation. In Figure 1 the CDF of the received power level in dBm is presented along with the 99% percentile point. In order to derive the in-band requirement for that particular case, it is recommended to consider the 99% percentile point of the received power for SBFD UL as a victim case for each scenario and then get the average point.

Proposal 3: To derive the in-band requirement for that particular case, it is recommended to consider the 99% percentile point of the received power for SBFD UL as a victim case for each scenario and then get the average point.

Figure 1 Received power level CDF for UMa when TDD DL is aggressor and SBFD UL is victim

Regarding the impact of CLI handling schemes, since it is still under discussion in RAN1, it is proposed that RAN4 do not consider any CLI handling scheme when defining the in-band blocking requirement at this stage in the normative phase. 

Proposal 4: RAN4 to not consider any CLI handling scheme effects when defining the in-band blocking requirements.

# Conclusion

In this contribution, we provided our views on BS RF requirements for SBFD with the following observations and proposals:

Observation 1: Adopting legacy gNB ACLR requirement for the SBFD-capabe gNB is sufficient to ensure feasible coexistence with adjacent channel deployed legacy TDD systems.

Observation 2: When coexisting with SBFD-capable gNB, legacy ACLR requirement might not be sufficient to ensure feasible coexistence. Additionally, RAN4 has only considered ACIR enhancement as a possible solution during the SI phase.

Proposal 1: RAN4 to consider additional ACLR requirement to ensure feasible coexistence with SBFD-capable gNB. As a starting point, urban macro deployments should be considered as that represent the challenging scenario.

Proposal 2: For OBUE, RAN4 to consider similar approach as ACLR (i.e., two different requirements to address coexistence with legacy TDD and SBFD-capable gNBs.

Proposal 3: To derive the in-band requirement for that particular case, it is recommended to consider the 99% percentile point of the received power for SBFD UL as a victim case for each scenario and then get the average point.

Proposal 4: RAN4 to not consider any CLI handling scheme effects when defining the in-band blocking requirements.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, RAN4#111, Samsung.

- R4-2410108, “Topic summary for [111][313] NR_duplex_evo”, RAN4#111, Samsung.

- TR 38.858, “Study on evolution of NR duplex operation”, 3GPP.