3GPP TSG-RAN4 Meeting #112	R4-2411516

Maastricht, Netherlands, 19th  – 23rd August, 2024

Title:	Views on EIRP mask considerations for upper 6GHz

Source:	Qualcomm Incorporated

Agenda item:	8.11.2

Document for:	Approval

# 1	Introduction

In RAN#103 a new work item on BR BS RF requirement evolution for FR1/FR2 and testing has been approved in [1]. One of the objectives of the core part related to the expected EIRP mask for upper 6GHz is the following:

- Introduce the concept of “limits on expected EIRP for angles above horizon” into the RAN4 BS specifications and specify related BS requirement for 6425-7125 MHz based on the WRC-23 outcome.

In RAN4#111, a WF was agreed in [2]. In this paper we share our view on the expected work within RAN4 to define the EIRP mask to control emissions above the horizon for an NR BS.

# 2	Discussion

## 2.1	Expected EIRP calculation in discrete spatial sampling grid

In [2], the following formulation of the expected EIRP in discrete spatial sampling grid was agreed in the WF:

Issue 2-7 Expected EIRP calculation in discrete spatial sampling grid Agreement: - PθL , θH=π2MNsinθH-sinθLn=nLnHm=1MP1θn,φm cosθn , where P1 is measured as EIRP - M is the number of sampling points in the azimuth range; - [N is the number of sampling points within the θL , θH bounding range over upper hemisphere in the elevation range ] - nL is the lowest elevation sampling angle within the θL , θH bounding range - nH is the highest elevation sampling angle within the θL , θH bounding range - m=1MP1θn,φm is to produce the global sum over the whole azimuth ranges for EIRP above the horizon in a given bin. - Note: the equation is assuming the angles in radians and the elevation angle is from horizon rather than the coordinate system defined in 3GPP. Note: the above equation should be carefully reviewed by companies.

In this section, we verify our understanding and correction of the above characterization as follows.

The expected EIRP mask is an outcome of ITU-R studies that studied the compatibility of IMT services with the fixed satellite service uplink (FSS UL), with the outcomes captured in in Resolution COM4/7 (WRC-23). The first step of the evaluation framework is the computation of the average EIRP P1θ0,φ0 over beamforming directions (αn,βn) for a given vertical θ0and horizontal ϕ0reference angles with a given weighting vector over each beamforming direction, i.e., wn

P1θ0,φ0=n=1NwnPθ0,φ0 ; αn,βn    (1)

Afterwards, another averaging over the vertical and horizontal angles is carried out over, while noting that for the GCS, θ is defined such that  θ=0o represents the north pole while here θ=0o represents the equator and any θ>0o represents the vertical angles above the horizon. Considering the above along with the fact that the vertical integration of θ is from θH to θL, the average EIRP can be expressed as

EEIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ dφ dθ  (2)

Transitioning from the integration to discrete summation of equation (2), the following is considered

dθ=Δθ= θH-θLN

dϕ= Δϕ= 2πM

Where N and M represents the number of sampling points in the elevation and azimuth planes, then the expected EIRP can be approximated to

EEIRPθL , θH=12πsinθH-sinθLθLθH-ππEIRPθ,φ cosθ ∆φ ∆θ

Which can be finally expressed as

EEIRPθL , θH=(θH-θL)NMsinθH-sinθLn=nL nH m=1MEIRPθn,φm cosθn

Where, the numerator is given in radians and the following definitions apply

θn : 	Discrete elevation sampling angles between the elevation bins

φm: 	Discrete azimuth sampling angles ranging from -π to π

nL : 	Lowest elevation sampling angles within the θL , θH bounding range

nH : 	Highest elevation sampling angles within the θL , θH bounding range

M  : 	Number of sampling points in the azimuth range

N  : 	Number of sampling points within the θL , θH bounding range over the upper hemisphere in the elevation range

Proposal 1: RAN4 to consider the following characterization of the expected EIRP

EIRPθL , θH=(θH-θL)NMsinθH-sinθLn=nL nH m=1MEIRPθn,φm cosθn

Where, the numerator is given in radians and the following definitions apply

θn : 	Discrete elevation sampling angles between the elevation bins

φm: 	Discrete azimuth sampling angles ranging from -π to π

nL : 	Lowest elevation sampling angles within the θL , θH bounding range

nH : 	Highest elevation sampling angles within the θL , θH bounding range

M  : 	Number of sampling points in the azimuth range

N  : 	Number of sampling points within the θL , θH bounding range over the upper hemisphere in the elevation range

## 2.2	Multi-user beamforming aspects

In the LS from WP5D to RAN4 R4-2400333 regarding the RF parameters for WRC-27 studies, one of the questions of WP5D, independent of the frequency range, is whether considering multi-user spatial beamforming techniques at the BS AAS might lead to higher unwanted emissions above the horizon. Although those discussions are not directly related to upper 6GHz discussions in RAN4/ WP5D, it is important for RAN4 to agree on how to address the aspect of multi-user beamforming in the EIRP mask currently being standardized for the upper 6 GHz band in this ongoing Rel-19 WI.

Considering the system-level simulation work typically conducted in ITU-R WP 5D for sharing and compatibility studies using 3D modelling of antenna pattern, ITU-R WP 5D is seeking information on whether for implementation of IMT BS multiuser spatial beamforming techniques, such as zero-forcing (ZF) or minimum mean-square error (MMSE) based schemes, would be necessary to improve the assessment of interference and the accuracy of studies. If considered necessary, guidance on the process of deriving the necessary beamforming weights for the IMT AAS BS to compute its radiation pattern.

Proposal 2: RAN4 to discuss whether the agreed framework and core requirements for the upper 6GHz band is applicable to multi-user beamforming or not.

## 2.3	     Impact of measurement overhead

Based on the expected EIRP framework, current discussions on the conformance testing need to consider several parameters when designing the conformance tests. As an example, the number of beamforming directions over which the test is done, the weights associated with each beamforming direction in the expected EIRP evaluation, and the sampling grid sizes in both elevation and azimuth. To understand the overall testing/ measurement overhead, RAN4 need to investigate the inter-dependency between the above aspects and overall testing complexity.

Observation 1: A balance between testing/measurement overhead and accuracy needs to be considered when determining the number of beamforming directions, weights associated with each beamforming direction and sampling grid sizes.

## 2.4	Number of beamforming directions

Given a coverage angular region (CAR) in the azimuth and elevation planes for the gNB AAS, the most natural array geometry is expected to be rectangular. As part of the conformance test, test beamforming vectors are expected to steer the energy only within the CAR. To derive  meaningful steering angle, in the conformance specs the following should be considered. A large number of beamforming vectors can be used, with the natural measurement unit is the 3-dB beamwidth of a beam. The number of beamforming directions can be evaluated based on the concept of “over sampling of beams”, similar to channel state feedback codebooks in 38.214. An oversampling factor Oazm and Oelev can be considered for both azimuth and elevation planes, leading to a uniform grid of Kazim=OazimφCARφ3dB and Kelev=OelevθCARθ3dB   progressive phase shift beams with the total the number of beamforming vectors is K=Kazim Kelev. A visualization of a rectangular CAR with a set number of beams is given in Figure 1 below.

Figure 1 A rectangular CAR with a set of beams

Proposal 3: RAN4 to define a set of oversampling factors Oazm and Oelev associated with the coverage angular region leading to a uniform grid of Kazim=OazimφCARφ3dB and Kelev=OelevθCARθ3dB   progressive phase shift beams (in each dimension) for testing, where θ3dB and ϕ3dB are th3 3-dB beamwidth of the BS main beam. The beam steering directions are θn,φm where m=1,…, Kazim and n=1,…, Kelev. Thus, the number of beamforming vectors is K=Kazim Kelev.

For the weighting factors associated with each of the beamforming directions, it is viable to consider uniform weighting across all the beams in order to avoid increased testing/ measurement complexity.

Proposal 4: RAN4 to consider in the conformance specs that the weighting factors of the supported beamforming directions follow a uniform distribution.

## 2.5	Measurement grid size

Similar to the number of beamforming directions, we can utilize the same rationale to identify the sufficient grid sizes in the azimuth and elevation planes. Since we need to sample the entire 360o in azimuth, we can use the same step size of φ3dB with an oversampling factor of Lazimfor a measurement grid of M beams where M=Lazim2π φ3dB. For the elevation domain (region of interest being θL ≤θ≤θH), we can consider an oversampling factor of Lelev for a measurement grid of N beams where N=LelevθH- θL θ3dB

Proposal 5: RAN4 to consider step size of φ3dB with an oversampling factor of Lazimfor a measurement grid of M beams where M=Lazim2π φ3dB for the azimuth plane and a measurement grid of N beams where N=LelevθH- θL θ3dB for the elevation plane.

## 2.6	Measurement accuracy reporting

Recall that the expected EIRP framework is expressed as below

EEIRPθL , θH=1K(θH-θL)NMsinθH-sinθLk=1.. K n=nL nH m=1MEIRPθn,φm;wk cosθn,

the θnand ϕm implicitly considers the sampling grid size, so a framework to determine the estimation process in the above framework/ methodology is sought. Leveraging the introduced oversampling factors for both azimuth and elevation planes, one can define the estimation error in evaluating the expected  EIRP as the difference between the expected EIRP over the oversampling factors (Oazim, Oelev) and (Lazim, Lelev) relative to the largest oversampling factor allowed (where testing is done).

Proposal 6: RAN5 to consider defining the estimation/measurement accuracy error as the difference between the expected EIRP over the oversampling factors (Oazim, Oelev) and (Lazim, Lelev) relative to the largest oversampling factor allowed (where testing is done).

# 3	Summary

In this paper, we have shared our initial views on the expected EIRP mask development for upper 6GHz NR BS where we propose the following:

Proposal 1: RAN4 to consider the following characterization of the expected EIRP

EIRPθL , θH=(θH-θL)NMsinθH-sinθLn=nL nH m=1MEIRPθn,φm cosθn

Where, the numerator is given in radians and the following definitions apply

θn : 	Discrete elevation sampling angles between the elevation bins

φm: 	Discrete azimuth sampling angles ranging from -π to π

nL : 	Lowest elevation sampling angles within the θL , θH bounding range

nH : 	Highest elevation sampling angles within the θL , θH bounding range

M  : 	Number of sampling points in the azimuth range

N  : 	Number of sampling points within the θL , θH bounding range over the upper hemisphere

Proposal 2: RAN4 to discuss whether the agreed framework and core requirements for the upper 6GHz band is applicable to multi-user beamforming or not.

Observation 1: A balance between testing/measurement overhead and accuracy needs to be considered when determining the number of beamforming directions, weights associated with each beamforming direction and sampling grid sizes.

Observation 1: A balance between testing/measurement overhead and accuracy needs to be considered when determining the number of beamforming directions, weights associated with each beamforming direction and sampling grid sizes.

Proposal 3: RAN4 to define a set of oversampling factors Oazm and Oelev associated with the coverage angular region leading to a uniform grid of Kazim=OazimφCARφ3dB and Kelev=OelevθCARθ3dB   progressive phase shift beams (in each dimension) for testing, where θ3dB and ϕ3dB are th3 3-dB beamwidth of the BS main beam. The beam steering directions are θn,φm where m=1,…, Kazim and n=1,…, Kelev. Thus, the number of beamforming vectors is K=Kazim Kelev.

Proposal 4: RAN4 to consider in the conformance specs that the weighting factors of the supported beamforming directions follow a uniform distribution.

Proposal 5: RAN4 to consider step size of φ3dB with an oversampling factor of Lazimfor a measurement grid of M beams where M=Lazim2π φ3dB for the azimuth plane and a measurement grid of N beams where N=LelevθH- θL θ3dB for the elevation plane.

Proposal 6: RAN5 to consider defining the estimation/measurement accuracy error as the difference between the expected EIRP over the oversampling factors (Oazim, Oelev) and (Lazim, Lelev) relative to the largest oversampling factor allowed (where testing is done).

# 4	References

- RP-240829, “New WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, RNA4 chair Huawei, RAN#103.

- R4-2409955, “Way Forward for [111][311] NR_BS_RF, ZTE, RAN4#111, Japan.