3GPP TSG-RAN4 Meeting #112	R4-2411517

Maastricht, Netherlands, 19th–23rd August, 2024

Title:	Views on BS demodulation requirements for MMSE-IRC

Source:	Qualcomm Incorporated

Agenda item:	8.16.3

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on BS advanced receiver performance was agreed in [1]. One of the objectives of the WI is as follows:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

In this contribution, we provide our views on the envisaged use cases and deployment scenarios, and potential deltas that RAN4 is expected to consider compared to the work that was carried out in LTE for BS with advanced receiver.

# 2 	Discussion

## 2.1	Background

As captured in [1], the motivation behind Rel-19 WI on BS demodulation requirement with enhanced receiver is to consider more realistic scenarios for dense deployments where uplink interference from co-scheduled UEs in the neighbouring cells can be a limiting factor to the uplink performance. High power UEs are being extensively studied and standardized within 3GPP (e.g., high-power UE (PC 1.5) in n77 and n78 (RP-202912), high power UE for NR TDD intra-band Carrier Aggregation for FR1 (RP-221774), and high-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands (RP-221795)), with the system and link level benefits of high power UEs, the inter- and intra-cell interference dynamics can be different, especially for urban dense deployments. Accordingly, suppression of such interference based on advanced BS receiver can provide an uplink performance boost in terms of coverage.

Observation 1: As high power UEs becomes more common, advanced base station receivers play a crucial role in reducing inter-cell uplink interference in densely populated urban areas.

## 2.2	Interference modelling

In LTE study captured in [2], homogenous, with single tier macro-BS and heterogeneous, with two-tier (macro and co-channel micro) BSs, scenarios were considered to quantify the link gains from employing MMSE-IRC Rx at the BS. In order to determine the interference models to quantify the gains of MMSE-IRC, [2] adopted a statistical measure, namely, Dominant Interferer Proportion (DIP), to evaluate the interference received energy from the i-th strongest inter-cell interferer involved in a given requirement scenario, where i can be set to 1 or 2, as chosen appropriately. The different DIP sets are then used to derive the different performance requirements. Thus, it is important to first discuss within RAN4 whether the same methodology will be used in the Rel-19 WI and if yes, what would be the realistic deployment scenarios and their corresponding parameters to be considered. For example, Section 5 in [2] considers only 10MHz CHBW without beamforming capability at the BSs, power class 3 for UEs, and 2GHz carrier frequency.

Observation 2: LTE work in TR 36.884 considered passive antennas at the BS, PC3 UEs, and 2 GHz carrier frequency in its system level study to derive the different DIP levels. Such assumptions might need to be revisited for Rel-19 WI.

To be reflect the demanding uplink use cases in terms of density and interference mitigation, several of those assumptions might need to be reconsidered.

Proposal 1: RAN4 to discuss what are the different assumptions in the Rel-19 WI in terms of deployment scenarios (i.e., homogenous and heterogenous deployments) and system level assumptions compared to those adopted in TR 36.884.

In terms of the interference modelling and the adopted statistical framework in TR 36.884, as captured in [1], it is reasonable to adopt the same DIP-based methodology as a starting point to characterize the interference profiles that will be then used to derive the different BS demodulation requirements.

Proposal 2: RAN4 to adopt the DIP-based methodology as a starting point to characterize the interference profiles based on realistic system scenarios and assumptions.

During LTE study, the performance gain between baseline (i.e., MMSE) and advanced (i.e., MMSE IRC) was marginal, thus, it was agreed to consider only one explicit interferer for 2Rx cases. However, two explicit interferers are for 4Rx and 8Rx cases. For the Rel-19 WI, a further distinction can be made between FDD and TDD scenarios:

- For FDD scenarios, it is recommended that first RAN4 identify the gains of MMSE IRC receiver considering two interferences only 4Rx and 8Rx cases. The rationale is that 2Rx scenarios for NR gNBs might not be foreseeable.

- Similarly for TDD, given that UEs are required to have 4RX capability is several deployment bands, it seems unrealistic use case to consider 2RX gNBs, focus should be on 4RX and 8 RX cases

Proposal 3a: RAN4 to consider only 4RX and 8RX gNB for TDD and FDD scenarios.

Proposal 3b: RAN4 to consider two interferences for both 4Rx and 8Rx cases as an initial assumption.

In terms of multi-path fading propagation conditions, [2] considered ETU channel with high delay spread for the interferers and EPA5 and EVA70 were chosen for the serving channel while, ETU5 and ETU70 were considered for the interfering channel. For Rel-19, it is expected that RAN4 would choose an appropriate propagation conditions that is realistic compared to the envisioned use cases and scenarios.

Proposal 4: RAN4 to revisit the multi-path fading propagation conditions in TR 36.884 following the agreed scenarios for Rel-19 WI.

# Conclusion

In this contribution we provide our views on the impact of multi-user spatial beamforming techniques on the system level simulation. Our observations and proposals can be summarized for that frequency range as follows:

Observation 1: As high power UEs becomes more common, advanced base station receivers play a crucial role in reducing inter-cell uplink interference in densely populated urban areas.

Observation 2: LTE work in TR 36.884 considered passive antennas at the BS, PC3 UEs, and 2 GHz carrier frequency in its system level study to derive the different DIP levels. Such assumptions might need to be revisited for Rel-19 WI.

Proposal 1: RAN4 to discuss what are the different assumptions in the Rel-19 WI in terms of deployment scenarios (i.e., homogenous and heterogenous deployments) and system level assumptions compared to those adopted in TR 36.884.

Proposal 2: RAN4 to adopt the DIP-based methodology as a starting point to characterize the interference profiles based on realistic system scenarios and assumptions.

Proposal 3a: RAN4 to consider only 4RX and 8RX gNB for TDD and FDD scenarios.

Proposal 3b: RAN4 to consider two interferences for both 4Rx and 8Rx cases as an initial assumption.

Proposal 4: RAN4 to revisit the multi-path fading propagation conditions in TR 36.884 following the agreed scenarios for Rel-19 WI.

# Reference

[1]		RP-240831, “New WID: WI resulting from discussion on Demodulation performance requirements enhancements”, Vivo, RAN#103, Maastricht, Netherlands.

[2] 	TR 36.884, “Performance requirements of MMSE-IRC receiver for LTE BS” 3GPP V13.1.0