3GPP TSG-RAN4 Meeting #112	R4-2411518

Maastricht, Netherlands, 21st – 25th August, 2024

Title:	Views on IMT parameters for 7125 - 8400 MHz

Source:	Qualcomm Incorporated

Agenda item:	8.2.3

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [1]. In RAN4#111, a WF was agreed in [2] where several agreements were captured for the 7125 – 8400 MHz and 14800 – 15350 MHz frequency range. In this contribution, we provide our views on the different RF parameters for BS and UE items for the 7125 – 8400 MHz frequency range.

# 2 	Discussion

## 2.1	System parameters

### 2.1.1	Duplexing

During RAN4#111, discussion was held on the incorporation of SBFD as a duplex method for the 4400-4800MHz frequency range and it was agreed to include SBFD only in the TR while mention TDD in the LS to WP5D. The TP capturing information on SBFD was agreed in . Additionally, the following was captured in [2] regarding the 7125 – 8400 MHz frequency range:

Issue 2-1 Duplex Method - Do not include SBFD in the LS, but add information in the TR. Add a reference to the TR in the LS

The existing n104 band is TDD and it is likely that TDD will be deployed. On the other hand, 3GPP has been studying since Rel-18 the concept of allowing the simultaneous existence of downlink and uplink at the gNB side within a conventional TDD band (i.e., SBFD). The outcome of the Rel-18 study has been captured in TR 38.858. Currently 3GPP is specifying in Rel-19 SBFD operation at the gNB side within a TDD carrier, gNB-to-gNB CLI handling schemes and UE-to-UE CLI handling schemes for SBFD operation, and RF requirements for SBFD operation at gNB [4]. It is thus important to document in this SI the ongoing progress within 3GPP on duplex evolution, specifically, SBFD.

Observation 1: 3GPP has been studying flexible duplexing (i.e., SBFD) at the gNB in Rel-18 and Rel-19 and it is desired to document such progress in this SI TR (i.e., TR 38.922).

Since the core requirements for SBFD-capable gNB is still under development within Rel-19 package [4], it is important to progress the sharing studies in WP5D via timely providing WP5D with the requested RF parameters. Accordingly, it is proposed that RAN4 adopts TDD duplexing as a baseline for the 7125 - 8400 MHz and capture in the TR text reflecting the ongoing work on SBFD within 3GPP.

Proposal 1: RAN4 to agree on TDD as a baseline duplexing for 7125 – 8400 MHz frequency range and capture the following text in TR 38.922.

“There is no defined 3GPP band for the 7125 - 8400 MHz frequency range, however, it is adjacent to existing TDD band n104 (6425 – 7125 MHz). Similar to the 4400 – 4800 MHz frequency range, SBFD can be a candidate duplexing method for this frequency range. To provide timely response to WP5D regarding the requested RF parameters, RAN4 assumed TDD as a baseline duplexing for the 7125 – 8400 MHz frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [SBFD_WID].”

### 2.1.2	Channel bandwidth

In [2], the following was agreed:

- Mention 100MHz as typical bandwidth in the LS - FFS whether to mention wider bandwidths may be possible in future in the LS - Describe other possibilities for bandwidth in the TR, mentioning that additional specification effort needed for >100MHz

In general, wider channel bandwidths should be enabled with increasing frequency following same approach as FR1 (3 to 100 MHz) to FR2 (50 to 400 MHz). This comes with increasing SCS and increasing RB size. Since the 71425 – 8400 MHz is not in the FR1 frequency range, higher channel bandwidths compared to 100MHz should not be precluded at this stage. Such channel bandwidths can be achieved via higher sub-carrier spacing (SCS) or intra-band CA employment.  However, due to the time limitation within RAN4 to study higher channel bandwidths on the other RF parameters (e.g., OBUE, SEM, spurious, etc.), RAN4 agreed to adopt 100MHz as a candidate/typical channel bandwidth for this frequency range.

An important aspect to consider is the implications of higher channel bandwidth on the adjacent channel coexistence findings. To this end, we compare the throughput loss % as a function of ACIR for both downlink and uplink transmissions in UMa deployments for both 100 MHz and 200 MHz channel bandwidth, as shown in Figure 1. Note that all the network, BS, and UE parameters follow the adjacent channel coexistence conducted in TR 38.921. It can be observed that the ACIR required to meet the 5% throughput loss degradation target is nearly identical for the 100 MHz and 200 MHz channel bandwidths.

Observation 2: Based on adjacent channel coexistence simulations, ACIR requirements (i.e., BS/ UE ACLR and ACS) based on 100MHz channel bandwidth can be reused for higher channel bandwidth values (e.g., 200MHz).

Figure 1 TP loss for UMa scenario for CHBW 100MHz (black) and 200MHz dBm (magenta)

As a consequence of our analysis shown above, we propose to consider 100MHz while mentioning that other channel bandwidths are not precluded in the LS. Additionally, we include in [xxx] a TP capturing, among others, the channel bandwidth considerations for this frequency range and its implication on adjacent channel coexistence.

Proposal 2: RAN4 to reply in the LS to WP5D that 100MHz is a typical value and that higher channel bandwidths are not precluded for the 7125 – 8400 MHz frequency range. Additional information on the channel bandwidth consideration for this range should be captured in TR 38.922.

### 2.1.3	Signal bandwidth

In [2] the following was captured as a WF for the signal bandwidth:

Outcome of discussion (Not agreed): Option 1: 273 RB, 30k SCS as 38.104 Option 2: Quote formula of RBs * SCS without number of RBs Option 3: Directly quote the signal bandwidth in MHz

Signal bandwidth depends on the channel bandwidth as well as on the SCS. The transmission bandwidth configuration defined in RAN4 core spec for BS and UE can be reused to derive the necessary signal bandwidth, where the signal bandwidth is calculated based on the NR spectrum utilization for each SCS as follows:

Signal bandwidth = NRB x SCS x 12

For the targeted bandwidth, 30 and 60 KHz might be possible candidates for this frequency range. Since it will be difficult to agree within RAN4 on a given SCS and NRB for the 7GHz frequency range, it is proposed to mention in the LS the dependency of signal bandwidth on the SCS and number of RBs.

Observation 3: As signal bandwidth depends on CHBW and SCS, no need to specify a fixed signal bandwidth but rather mention its dependency on SCS and number of RBs.

Proposal 3: RAN4 to respond to WP5D on the signal bandwidth with the Quote formula of RBs * SCS without number of RBs.

## 2.2	BS parameters

### 2.2.1	Spectral mask

In [2], the following was captured:

- AAS BS: - Use n104 unwanted emissions mask - [Aim to use 100MHz for delta_obue, conclude feasibility until RAN4#112] - Non-AAS BS: - [n104 unwanted emissions mask for IMT response] Double check on whether there are any concerns towards sharing by RAN4#112 and in case concerns are identified, consider n96 if needed to resolve concerns.

In TR 38.921 (i.e., 7GHz and 10GHz Rel-17 SI), the derived BS SEM For both 6.425 - 7.125 GHz and 10.0 - 10.5 GHz band, considered that channel bandwidths less than 50MHz are not foreseeable, thus, the basic limits for the SEM were based on 50MHz. Such assumption holds true for the investigated 71425 – 8400 MHz frequency range and thus, it is recommended to adopt the n104 unwanted emission mask for both AAS and non-AAS BS.

Proposal 4: RAN4 to adopt the n104 unwanted emission mask for both the AAS and non-AAS BS.

### 2.2.	Sensitivity

It is challenging to determine the BS Rx sensitivity for the 71425 – 8400 MHz frequency range without first agreeing whether it has a radiated or a conducted sensitivity requirement. Additionally, to determine the Rx sensitivity requirement, several parameters need to be agreed (e.g., Antenna gain, Noise figure, implementation margin, BW), which have not all been fully discussed within RAN4.

Observation 4: To determine the BS Rx sensitivity, RAN4 need to agree on the requirement type (i.e., conducted or radiated) as well as other parameters (e.g., Antenna gain, Noise Figure, implementation margin).

As a compromise, it was agreed in [2] to capture only in the TR the sensitivity following the given n104 requirement. We propose to not respond with a given sensitivity value in the LS, since it is a fundamental parameter for the WP5D sharing studies.

Proposal 5: RAN4 to mention the BS Rx sensitivity only in TR 38.922 and mentioned “to be specified” in the LS to WP5D.

## 2.3	UE parameters

### 2.3.1	Maximum Output power

In [2], RAN4 reached the following tentative agreement:

Tentative agreement: - Capture 23dBm as typical maximum output power in the LS to ITU. - Other output powers are not precluded and refer to TR - Capture 20dBm, 23dBm, 26dBm, and higher power classes in TR

As most of the RF parameters communicated to WP5D for this frequency range is based n104, it is rational to consider both PC2 and PC3 as a response to WP5D. High power UEs are being extensively studied and standardized within 3GPP (e.g., high-power UE (PC 1.5) in n77 and n78 (RP-202912), high power UE for NR TDD intra-band Carrier Aggregation for FR1 (RP-221774), and high-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands (RP-221795)). To capture the trend of higher UE power classes, envisaged for new applications and deployment use cases, we can compromise with capturing 23 dBm (i.e., PC3) as a typical maximum output power in the LS to WP5D, however, in the TR, higher values (i.e., PC2) should be considered as a candidate option for this frequency range.

Proposal 6: RAN4 to consider PC3 (i.e., 23 dBm) as a typical output power and to capture higher output powers in TR 38.922.

In order to ensure that the adjacent channel coexistence (i.e., BS and UE ACLR/ACS) derived for PC3 is not impacted when considering higher UE maximum output power (e.g., PC2), we compare the throughput loss % as a function of ACIR for both downlink and uplink transmissions in UMa deployments for both PC3 and PC2, as shown in Figure 2. Note that all the network, BS, and UE parameters follow the adjacent channel coexistence conducted in TR 38.921. It can be observed that the ACIR required to meet the 5% throughput loss degradation mark is nearly the same for both PC3 and PC2. This is attributed to the fact that the system is dominated by the co-channel interference and thus, increasing the UE maximum output power from 23 dBm to 26 dBm will not impact the derived ACLR/ACS values from adjacent channel coexistence point of view.

Figure 2 TP loss for UL UMa scenario for 23 dBm (magenta) and 26 dBm (black)  UE maximum output power

Observation 5: Based on adjacent channel coexistence simulations, ACIR requirements (i.e., BS/ UE ACLR and ACS) based on 23 dBm UE maximum output power can be reused for 26 dBm UE maximum output power scenarios.

### 2.3.2	UE power dynamic range

To derive the power dynamic range, RAN4 needs to agree on the UE minimum and maximum output power. We propose to reuse FR1 UE minimum output power of -33 dBm while 23 dBm as maximum output power. Accordingly, we propose 59 dBm for the UE power dynamic range.

Proposal 7: RAN4 to agree on 56 dBm as UE power dynamic range.

# Conclusion

In this paper we have shared our views on the RF parameters for 7125 – 8400 Frequency range. Our observations and proposals can be summarized for that frequency range as follows:

Observation 1: 3GPP has been studying flexible duplexing (i.e., SBFD) at the gNB in Rel-18 and Rel-19 and it is desired to document such progress in this SI TR (i.e., TR 38.922).

Proposal 1: RAN4 to agree on TDD as a baseline duplexing for 7125 – 8400 MHz frequency range and capture the following text in TR 38.922.

“There is no defined 3GPP band for the 7125 - 8400 MHz frequency range, however, it is adjacent to existing TDD band n104 (6425 – 7125 MHz). Similar to the 4400 – 4800 MHz frequency range, SBFD can be a candidate duplexing method for this frequency range. To provide timely response to WP5D regarding the requested RF parameters, RAN4 assumed TDD as a baseline duplexing for the 7125 – 8400 MHz frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [SBFD_WID].”

Observation 2: Based on adjacent channel coexistence simulations, ACIR requirements (i.e., BS/ UE ACLR and ACS) based on 100MHz channel bandwidth can be reused for higher channel bandwidth values (e.g., 200MHz).

Proposal 2: RAN4 to reply in the LS to WP5D that 100MHz is a typical value and that higher channel bandwidths are not precluded for the 7125 – 8400 MHz frequency range. Additional information on the channel bandwidth consideration for this range should be captured in TR 38.922.

Observation 3: As signal bandwidth depends on CHBW and SCS, no need to specify a fixed signal bandwidth but rather mention its dependency on SCS and number of RBs.

Proposal 3: RAN4 to respond to WP5D on the signal bandwidth with the Quote formula of RBs * SCS without number of RBs.

Proposal 4: RAN4 to adopt the n104 unwanted emission mask for both the AAS and non-AAS BS.

Observation 4: To determine the BS Rx sensitivity, RAN4 need to agree on the requirement type (i.e., conducted or radiated) as well as other parameters (e.g., Antenna gain, Noise Figure, implementation margin).

Proposal 5: RAN4 to mention the BS Rx sensitivity only in TR 38.922 and mentioned “to be specified” in the LS to WP5D.

Proposal 6: RAN4 to consider PC3 (i.e., 23 dBm) as a typical output power and to capture higher output powers in TR 38.922.

Observation 5: Based on adjacent channel coexistence simulations, ACIR requirements (i.e., BS/ UE ACLR and ACS) based on 23 dBm UE maximum output power can be reused for 26 dBm UE maximum output power scenarios.

Proposal 7: RAN4 to agree on 56 dBm as UE power dynamic range.

# Reference

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson, RAN #103, Maastricht, Netherlands.

[2] R4-2410741, “WF on IMT parameter study”, Ericsson, RAN4#111, Fukuoka, Japan.

[3] R4-2410722, “TP for TR 38.922: Addition of technical background for 4400 to 4800 MHz in clause 4”, Ericsson, RAN4#111, Fukuoka, Japan.