3GPP TSG-RAN4 Meeting #111	R4-2411519

Maastricht, Netherlands, 18th – 23rd August, 2024

Title:	Text proposal on 7125 – 8400 MHz IMT parameters in TR 38.922

Source:	Qualcomm Incorporated

Agenda item:	8.2.510.3.1

Document for:	Approval

# Introduction

In this document we provide our text proposals for the duplexing mode for the 4400-4800 MHz and 7125-8400 MHz frequency ranges following WF captured in R4-2406615.

******************Begin changed sections******************

# 5	7125 - 8400 MHz frequency range

## 5.1	General parameters

### 5.1.1	Duplex mode

### There is no defined 3GPP band for the 7125 - 8400 MHz frequency range, however, it is adjacent to existing TDD band n104 (6425 – 7125 MHz). Similar to the 4400 – 4800 MHz frequency range, SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6]. To provide a timely response to WP5D regarding the requested RF parameters, RAN4 assumed TDD as a baseline duplexing for the 7125 – 8400 MHz frequency range.

### 5.1.2 Channel bandwidth

Wider channel bandwidths should be enabled with increasing frequency following same approach as FR1 (3 to 100 MHz) to FR2 (50 to 400 MHz). This comes with increasing SCS and increasing RB size. Since the 71425 – 8400 MHz is not in the FR1 frequency range, higher channel bandwidths compared to 100MHz should not be precluded at this stage. Such channel bandwidths can be achieved via higher SCS or intra-band CA employment. However, in order to provide a pragmatic, simple and non-ambiguous value for the channel bandwidth to ITU-R, RAN4 has agreed to consider 100 MHz as a typical channel bandwidth for the 7125 – 8400 MHz frequency range, while noting that higher channel bandwidth should not be precluded.

An important aspect to consider is the implications of higher channel bandwidth on the adjacent channel coexistence findings. Accordingly, simulation studies were conducted to compare the throughput loss% as a function of ACIR for both downlink and uplink transmissions in UMa deployments for both 100MHz and 200MHZ, as shown in Figure 1. Note that all the network, BS, and UE parameters follow the adjacent channel coexistence conducted in TR 38.921. It can be observed that the ACIR required to meet the 5% throughput loss degradation target is nearly identical for the 100 MHz and 200 MHz channel bandwidths.

Figure 1 TP loss for UMa scenario for CHBW 100MHz (black) and 200MHz dBm (magenta)

### 5.1.3 Signal Bandwidth

The signal bandwidth for given channel bandwidth signal is calculated based on the NR spectrum utilization for a given SCS following the below equation:

Signal bandwidth = NRB x SCS x 12

with NRB: Number of Resource block for that channel bandwidth. Values for SCS and NRB for different channel bandwidths can be found in TS 38.104, subclause 5.3.2 for both FR1, FR2-1, and FR2-2.

## 5.3	UE Parameters

### 5.3.1	Transmitter Characteristics

#### 5.3.1.1	Power dynamic range

### The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. For the 7125 – 8400 MHz, minimum output power of -33 dBm for the typical100 MHz channel bandwidth can be reused. The power dynamic range can be thus computed as the difference between the UE maximum output power and minimum output power. For the typical UE output power of 23 dBm, the power dynamic range equals 56 dBm.

#### 5.3.1.5	Maximum output power

Following TR 38.921, RAN4 agreed to adopt 23 dBm (i.e., power class 3) for the 7125 – 8400 MHz frequency range. To address the increasing demand on higher UE power classes, envisaged for new applications and deployment use cases, other UE power classes should not be precluded at this stage.

In order to ensure that the adjacent channel coexistence (i.e., BS and UE ACLR/ACS) derived for PC3 is not impacted when considering higher UE maximum output power (e.g., PC2), we compare the throughput loss % as a function of ACIR for both downlink and uplink transmissions in UMa deployments for both PC3 and PC2, as shown in Figure 2. Note that all the network, BS, and UE parameters follow the adjacent channel coexistence conducted in TR 38.921. It can be observed that the ACIR required to meet the 5% throughput loss degradation mark is nearly the same for both PC3 and PC2.

Figure 2 TP loss for UL UMa scenario for 23 dBm (magenta) and 26 dBm (black) UE maximum output power

******************End changed sections******************