3GPP TSG-RAN4 Meeting #112	R4-2411520

Maastricht, Netherlands, 19th–23rd August, 2024

Title:	Views on Additional AAS aspects

Source:	Qualcomm Incorporated

Agenda item:	8.2.1

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [1]. One of the objectives of the SI is as follows:

- Considering the system-level simulation work typically conducted in ITU-R WP 5D for sharing and compatibility studies using 3D modelling of antenna pattern, ITU-R WP 5D is seeking information on whether for implementation of IMT BS multiuser spatial beamforming techniques, such as zero-forcing (ZF) or minimum mean-square error (MMSE) based schemes, would be necessary to improve the assessment of interference and the accuracy of studies. If considered necessary, guidance on the process of deriving the necessary beamforming weights for the IMT AAS BS to compute its radiation pattern.

Since it is expected that RAN4 provide its technical view on the above matter, independent of the frequency range, it is important that RAN4 have enough time to converge on the technical response to be provided. In this contribution, we provide our views on the impact of multi-user spatial beamforming techniques on the system level simulation, and we propose a way forward to be shared with WP5D.

# 2 	Discussion

## 2.1	Background

In ITU’s WP5D LS sent to RAN4 [2], questions on the impact of IMT BS multi-user spatial beamforming such as ZF, MRT, and MMSE on the system level studies outcome were included, independent of the frequency range under investigation. Since RAN4#110, RAN4 has been working on drafting its response to WP5D LS [2] for the different frequency ranges. To ensure that RAN4 provides timely feedback on all the requested information from WP5D, it is recommended to start addressing the generic questions in [2] to finalize RAN4 view and its response by the November meeting.

Proposal 1: RAN4 to respond to WP5D on the generic questions (i.e., Frequency range agnostic) captured in WP5D LS by the RAN4#113 meeting.

Among those generic questions is the impact of spatial beamforming on the studies outcome. Specifically, WP5D is currently trying to converge whether it is needed or not to consider multi-user beamforming techniques impact on top of the AAS model (i.e., ITU-R M.2101). Some administrations are concerned that such techniques might lead to additional unwanted aggregated emissions above the horizon, whereas the legacy model might not sufficiently account for that (see Document 5D/158-E for more information). On the other hand, during the June WP5D meeting, there were other views from the industry that incorporating ZF beamforming techniques into the legacy M-2101 model will have a quite minimal impact on the aggregate emissions above the horizon (see Document 5D/197-E for more information).

Observation 1: WP5D is currently investigating whether it is needed or not to consider multi-user beamforming techniques impact on top of the AAS model (i.e., ITU-R M.2101).

In our contribution we investigate the impact of ZF beamforming at the IMT BS, serving UEs in the DL transmission, with specific focus on the aggregate unwanted emissions above the horizon for a given IMT BS. Our IMT BS AAS modelling follows closely the framework on IMT AAS radiation pattern characterization in Recommendation ITU-R M.2101.

Finally, RAN4 should investigate the impact of such techniques on system level performance, converge on a technical response that addresses the concerns of WP5D members, and document its findings in TR 38.922.

Observation 2: RAN4 is expected to provide its technical response on the impact of multi-UE beamforming techniques impact on system level studies to WP5D and capture its findings in TR 38.922.

## 2.2	Technical background on Zero Forcing

ZF beamforming techniques aim at increasing the transmitted beam pattern towards the scheduled UE direction while reducing the interference towards other scheduled UEs in the azimuth domain. Accordingly, the design of the beamforming matrix W goal is to eliminate the interference from other users except the user of interest, as illustrated in the below

HkWi= 1,  k=i0,  k≠i, ∀ i

where Hk is the estimated channel matrix for user k. The beamforming vectors (w) which forms the beamforming matrix are chosen to satisfy the above-mentioned zero interference condition. The solution to this problem, is to get the pseudo-inverse of the estimated channel matrix H between the BS and the set of scheduled UEs, which can be evaluated via the following equation

W=H†=HHHHH-1

where H† is the Moore-Penrose pseudoinverse of the channel matrix H. Although the complexity of the ZF precoder is lower compared to other spatial multiplexing techniques (e.g., Maximum Ratio Transmission), it has an important drawback, which is its noise enhancement impact that occur at high SNR values. Such an impact can be alleviated by considering, as an example, the MMSE beamforming design. However, for this contribution we focus on the ZF design and its implementation as close as possible to ITU-R Recommendation M.2101. For simplicity and to align with the simulation framework in ITU-R Recommendation M.2101, the channel matrix H accounts only for the angles of arrival from each UE, which reflects the direct path between the BS and UEs. Such assumption alleviates the need to simulate full channel model between each BS and UEs in the network, which will increase the complexity of the sharing studies.

Our objective, as stated before, is to investigate whether incorporating ZF technique in a system level framework would lead to additional unwanted aggregated emissions above the horizon for an IMT BS serving its scheduled UEs. If no impact is observed, then RAN4 should advise WP5D that adopting such spatial multiplexing techniques will not provide additional insights to the sharing studies, envisioned to be carried out in WP5D. If impact is observed, RAN4 should provide the relevant modelling aspects or factors to be considered by WP5D when conducting their sharing studies.

Proposal 2:  RAN4 should investigate whether incorporating ZF technique in a system level framework would lead to additional unwanted emissions above the horizon for an IMT BS serving its scheduled UEs.

To visualize the impact of ZF on the AAS BS radiation pattern, we plot the AAS gain in the azimuth plane as the function of the azimuth angle ϕ when there are two UEs being scheduled. We assume that UE#1 is located at the BS boresight while UE#2 is located at either 5, 10, and 20 from UE#1. Figure 1 shows the radiation patterns towards UE #1 of a 16x8 AAS array with spacing of 0.5 wavelengths in the vertical and horizontal planes. As observed, when there is sufficient angular (i.e., spatial) separation between the scheduled UEs (e.g., at 10 and 20 degrees from UE#1), the antenna gain is maximized at UE#1 and minimized at UE#2, compared without ZF case (figure on the left). However, when such angular separation decreases (e.g., at 5 degrees), degradation of performance at UE#1 is expected compared to the case without ZF. From the figures, we can see a gain degradation of 4.8, 0.7, and 0.2 dB for the 5-, 10-, and 20-degrees angular separation, respectively. For that, in the simulation in section 2.3 below we see that a 10 degrees angular separation is sufficient since the antenna gain reduction seems very small i.e. 0.7 dB.

Figure 1 Figure 1 Radiation pattern at UE#1 assuming UE2 at 5, 10 or 20 degrees from UE1 respectively.

Observation 3: Enough angular separation should be considered in the system level studies to avoid performance degradation due to limited spatial resolution of the BS AAS.

## 2.3	Simulation assumptions and Results

BS AAS parameters and configurations that are considered in our Monte-Carlo study are provided in Table 1 and Table 2 respectively. In terms of scenario/ deployment, we consider a single sector where each snapshot, 3 UEs are randomly deployed within the coverage region of that sector with a given minimum angular/spatial separation of 10 degrees between the 3 UEs is considered. The location of those UEs is assumed to be known at the BS and a dedicated beam is generated to serve each UE. In order to investigate the impact of unwanted aggregated emissions above the horizon, probing points in the upper hemisphere have been dropped as visualized in Figure 2. Black points represent the total probing points, where blue points represent the dropped 3 UEs each snapshot with taking into account the angular separation between the UEs i.e. the 10 degrees. The probing points are distributed in an equal distance on a hemisphere above the horizon with angular separation between them approximately 18 degrees. It is worth mentioning that the objective of our study to investigate the experienced emissions at the different probing points, since they represent possible incumbent services with IMT that will be victim to the unwanted IMT BS emissions above the horizon.

Table 1: IMT deployment-related parameters

|  | Urban macro |
| --- | --- |
| Base station |  |
| Carrier frequency | 6425 MHz |
| BS Antenna height | 18 m |
| Cell radius | 300 m |
| Sectorization | 1 sector |
| User terminal |  |
| UE height | 1.5 m |
| UE density for terminals that are transmitting simultaneously | 3 UEs per sector |

Table 2: IMT base station beamforming antenna characteristics for IMT

|  | Urban Macro |
| --- | --- |
| Antenna pattern | Refer to 5D/716 (Annex 4.4 - WRC23) |
| Element gain (incl. Ohmic loss) (dBi) | 5.5 |
| Horizontal/vertical 3 dB beamwidth of single element (degree) | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 (Note 1) |
| Horizontal/Vertical radiating sub-array spacing | 0.5 of wavelength for H 0.5 of wavelength for V |
| Array Ohmic loss (dB) | 2 |
| Base station maximum coverage angle in the horizontal plane (degrees) | ±60 |
| Base station vertical coverage range (degrees) | 90-120 |
| Mechanical downtilt (degrees) | 10 |
| Note 1: 16 × 8 means there are 16 vertical and 8 horizontal radiating elements. |  |

.

Figure 2 Considered UE deployment for a given sector

CDF curves of the IMT BS gain towards the sample points are shown in Figure 3, where it can be observed the performance with and without ZF incorporated is quite close. Such similarity shows that when considering large number of snapshots in a Monte-Carlo framework, the impact of the spatial beamforming above the horizon is negligible.

Figure 3 Composite AAS gain from IMT BS towards probing points

Observation 4: From Monte-Carlo studies, similar IMT BS gain above the horizon is observed when zero forcing is considered at the BS IMT when compared to the case without zero forcing beamforming.

In order to provide RAN4 feedback to WP5D on the impact of spatial beamforming in system level studies, it is proposed based on our observation to consider the legacy AAS model without ZF beamforming (or similar techniques), since the performance is quite comparable compared to the case where ZF beamforming is considered. Thus, with no impactful difference and additional complexity for the system level studies, legacy AAS model should be sufficient for WP5D sharing studies.

Proposal 3: RAN4 to reply to WP5D that it is sufficient to adopt the legacy AAS model based on ITU-R Recommendation M.2101 since the performance when ZF beamforming is considered is similar to the performance without ZF beamforming technique.

# Conclusion

In this contribution we provide our views on the impact of multi-user spatial beamforming techniques on the system level simulation. Our observations and proposals can be summarized for that frequency range as follows:

Proposal 1: RAN4 to respond to WP5D on the generic questions (i.e., Frequency range agnostic) captured in WP5D LS by the RAN4#113 meeting.

Observation 1: WP5D is currently investigating whether it is needed or not to consider multi-user beamforming techniques impact on top of the AAS model (i.e., ITU-R M.2101).

Observation 2: RAN4 is expected to provide its technical response on the impact of multi-UE beamforming techniques impact on system level studies to WP5D and capture its findings in TR 38.922.

Proposal 2:  RAN4 should investigate whether incorporating ZF technique in a system level framework would lead to additional unwanted emissions above the horizon for an IMT BS serving its scheduled UEs.

Observation 3: Enough angular separation should be considered in the system level studies to avoid performance degradation due to limited spatial resolution of the BS AAS.

Observation 4: From Monte-Carlo studies, similar IMT BS gain above the horizon is observed when zero forcing is considered at the BS IMT when compared to the case without zero forcing beamforming.

Proposal 3: RAN4 to reply to WP5D that it is sufficient to adopt the legacy AAS model based on ITU-R Recommendation M.2101 since the performance when ZF beamforming is considered is similar to the performance without ZF beamforming technique.

# Reference

[1]		RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson, RAN #103, Maastricht, Netherlands.

[2]	R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”, WP5D, RAN4 #110, Athens, Greece.

[2] 	R4-2406615, “WF on IMT parameters for other frequency ranges”, CATT, RAN4#110-bis, Changsha, China.

[3]	TR 38.921, “Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz”, 3GPP V17.1.0.