3GPP TSG-RAN4 Meeting #111	R4-2411521

Maastricht, Netherlands, 19th – 23rd  August, 2024

Title:	Views on Radio parameters for 14800 - 15350 MHz

Source:	Qualcomm Incorporated

Agenda item:	8.2.4.2

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [1]. In RAN4#110-bis, a WF was agreed in [2] where several agreements were captured for the 7125 – 8400 MHz and 14800 – 15350 MHz frequency range. In this contribution, we provide our views on the different RF parameters for BS and UE items for the 14800 – 15350 MHz frequency range.

# 2 	Discussion

## 2.1	System parameters

### 2.1.1	Duplexing

Since there is no 14800 – 15350 MHz frequency ranges, it is expected that the discussion regarding duplexing would be similar to the 7125 – 8400 MHz frequency range. In [2], the following was agreed for the 7125 - 8400 MHz:

Agreement: Do not include SBFD in the LS, but add information in the TR. Add a reference to the TR in the LS

Accordingly, RAN4 is expected to consider TDD as a candidate duplexing for the 14800 – 15350 MHz Frequency range in its response to WP5D, while capture additional information on SBFD in TR 38.922.

Proposal 1: RAN4 to agree on TDD as a baseline duplexing for 14800 – 15350 MHz frequency range and capture the following text in TR 38.922.

“There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Similar to the 4400 – 4800 MHz and 7125 – 8400 MHz frequency ranges, SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6]. To provide a timely response to WP5D regarding the requested RF parameters, RAN4 assumed TDD as a baseline duplexing for the 14800 – 15350 MHz frequency range.”

### 2.1.2	Channel bandwidth

In general, wider channel bandwidths should be enabled with increasing frequency following same approach as FR1 (3 to 100 MHz) to FR2 (50 to 400 MHz). This comes with increasing SCS and increasing RB width and availability of the spectrum. Since the 14800 - 15350 MHz frequency range is not in the FR1 frequency range, higher channel bandwidths compared to 100MHz should be the baseline. Such channel bandwidths can be achieved via higher SCS or intra-band CA employment. Thus, we propose to consider 200MHz as a baseline while mentioning that other channel bandwidths are not precluded.

Proposal 2: RAN4 to adopt 200MHz is a typical channel bandwidth. Higher channel bandwidths should not be precluded for this frequency range.

### 2.1.3	Signal bandwidth

Signal bandwidth depends on the channel bandwidth as well as on the sub-carrier spacing (SCS). The transmission bandwidth configuration defined in RAN4 core spec for BS and UE can be reused to derive the necessary signal bandwidth, where the signal bandwidth is calculated based on the NR spectrum utilization for each SCS as follows:

Signal bandwidth = NRB x SCS x 12

For the targeted bandwidth, 30 and 60 KHz might be possible candidates for this frequency range. Since it will be difficult to agree within RAN4 on a given SCS till RAN4#112 (i.e., RAN4 target deadline for 7GHz parameters), it is proposed to mention the dependency of signal bandwidth on the SCS and number of RBs. ,

Proposal 3: As signal bandwidth depends on CHBW and SCS, no need to specify a fixed signal bandwidth but rather mention its dependency on SCS and number of RBs.

## 2.2	BS parameters

### 2.2.1	Array size

For the BS AAS parameters, the number of elements considered for the 4400 - 4800 MHz is based on TR 38.803, subclause 5.2.3.2.4, where the total number of antenna elements for macro suburban and macro urban is set to be 96 elements (4x8 subarrays with 3 elements per subarray). Assuming the same coverage metrics for the 14800 - 15350 MHz frequency range as FR1 deployments apply, it is anticipated that the number of antenna elements scale four times with doubling the frequency. 192 elements are typically considered for FR1 deployments, while discussions in the 7125 – 8400 MHz frequency range are in the ballpark of 384 antenna elements. To maintain the same physical area of the antenna array in the 14800 – 15350 MHz frequency range, the number of elements is expected to be larger than 1k elements. Accordingly, RAN4 has agreed in [2] on >1k elements with different AAS configurations.

Proposal 4: RAN4 to consider at least 1k elements at the BS side for the 14800 – 15350 MHz Frequency range. Exact parameters to be decided after the finalization of the adjacent channel coexistence.

From a feasibility point of view and with the expectation to provide wide-area coverage for this frequency range, considering at least 1k elements at the BS side is a viable option. When considering large antenna elements at this frequency range, there has been some discussion within RAN4 on the near-field impacts. This is justified under the fact that in the context of large antenna arrays and short UE-gNB distances where the overall array dimensions are of the same order as the transmitter to receiver separation, such large panels may potentially test the limits of planar wavefront modelling and may necessitate spherical wavefront modelling where the exact distance between any pair of transmit and receive antennas needs to be taken into account. Spherical wavefronts occur naturally when the antennas are modelled as point sources. Such a phenomena is currently being under investigation as part of the Rel 19 SI on FR3 channel modelling (see RP-234018  for more details). The impact of modelling near-by users using spherical waves on system level evaluations is questionable and thus, it is recommended that RAN4 ignores the near-field aspects for this frequency range within its reply to WP5D until RAN1 concludes its study

Observation 1: The impact of modelling near-by users using spherical waves on system level evaluations is questionable and currently being studied in RAN1.

Proposal 5: RAN4 to not consider near-field impacts in this frequency range in its correspondence to WP5D.

### 2.2.2 Spectral mask

For the 10GHz derivation of the spectral mask In TR 38.921 (i.e., 7GHz and 10GHz Rel-17 SI), the derived BS SEM For 10.0 - 10.5 GHz band, considered that channel bandwidths less than 50MHz are not foreseeable, thus, the basic limits for the SEM were based on 50MHz. RAN4 could follow the same approach considering scaling the basic limits for OOB emission mask with an appropriate minimum channel bandwidth value.

Observation 2: To derive the BS SEM, RAN4 could follow the same approach as TR 38.921 with the appropriate minimum channel bandwidth.

### 2.2.3	BS ACLR/ACS

The BS ACLR and ACS should be derived based on the outcome of the adjacent channel coexistence.

Proposal 6: For BS ACLR and ACS, RAN4 to decide on the UE ACLR and ACS based on the outcome of the adjacent channel coexistence study.

## 2.3	UE parameters

### 2.3.1	Maximum Output power

PC3 was assumed during Rel-17 work on 7GHz and 10GHz as captured in [3]. On the other hand, n104 considers both PC2 and PC3. Additionally, 3GPP has studied high power UEs during Rel-17 covering high-power UE (power class 1.5) operation in NR bands n77 and n78 (RP-202912), high power UE for NR TDD intra-band Carrier Aggregation in frequency range FR1 (RP-221774), and high-power UE operation for fixed-wireless/vehicle-mounted use cases in LTE bands and NR bands (RP-221795). To capture the trend of higher UE power classes, envisaged for new applications and deployment use cases, we propose to communicate with WP5D PC2 (i.e., 26 dBm) as the UE maximum output power.

Proposal 7: RAN4 to agree on 26 dBm (i.e., PC2) as UE maximum output power as a baseline.

### 2.3.2	UE power dynamic range

To derive the power dynamic range, RAN4 needs to agree on the UE minimum and maximum output power. We propose to reuse FR1 UE minimum output power of -33 dBm while 26 dBm as maximum output power. Accordingly, we propose 59 dBm for the UE power dynamic range.

Proposal 8: RAN4 to agree on 59 dBm as UE power dynamic range.

### 2.3.3	UE ACLR/ACS

The UE ACLR and ACS should be derived based on the outcome of the adjacent channel coexistence.

Proposal 9: For UE ACLR and ACS, RAN4 to decide on the UE ACLR and ACS based on the outcome of the adjacent channel coexistence study.

### 2.3.4 	UE beamforming

In [2] the following was agreed:

- For simulations, consider both options. - FR1 like - RX diversity gain: [0 or 5] dB assuming 4RX - TX: 0dBi omnidirectional - Max Power: 23 dBm, 26dBm - FR2 like - Two panels (one in each direction) 2x2 antenna - 5dBi element gain. Array gain comes on top - Power: 23 dBm as max TRP

The “FR1 like” option has two flavours, firstly within no Rx diversity gain, which follows that of TR 38.921 where an isotropic omnidirectional antenna with 0 dBi. The second “FR1 like” option considers a 5dBi gain resulting from 6 dBi gain assuming 4Rx and 1 dB losses. We have some concerns with those two options that are clarified below. For the 0 dBi isotropic assumption with 0 dBi Rx diversity, such a configuration paints a quite pessimistic line on the UE antenna performance, where, some UE gain is expected to be observed.

Observation 3: UE beamforming assumption based on 0 dBi isotropic element with 0 dB Rx diversity gain paints a pessimistic view of the UE performance from a system level perspective, where in reality some UE gain is observed.

On the other hand, the assumption of having a 5dB Rx diversity gain considers such a gain as a constant addition on top of the UE antenna gain and thus, an added 5dB is considered in the minimum coupling loss analysis, which is considered for both the link of interest and the interfering link. Such assumption is not fully true since RAN4 adjacent channel coexistence does not consider a full channel model (i.e., only large-scale parameters are considered) and thus full channelization and Rx diversity combining techniques might not be straightforward.

Observation 4: UE beamforming assumption based on 0 dBi isotropic element with 5 dB Rx diversity gain considers a simplistic 5dB addition in the minimum coupling loss for both scheduled and interfering links.

To provide a realistic model for the “FR1-like” UE beamforming case, as a compromise between the two options captured in [2], it is viable to consider a notion of directionality at the UE. This can be realized by considering a single antenna element at the UE with random orientation in the azimuth domain, uniformly distributed between -90 and 90 degrees (i.e., similar to TR 38.803).

Proposal 10: As an alternative option for the “FR1-like UE BF”, RAN4 to consider UE with a single antenna element at the UE with random orientation in the azimuth domain, uniformly distributed between -90 and 90 degrees.

For the FR2 options, several companies have conveyed their concern during RAN4#111 on the feasibility of having 2x2 modules in the FR3 frequency range due to the antenna size. To address such concerns, we propose to revisit the “FR2-like” proposal to consider 1x2 array at each panel instead of 2x2 array. Such a compromise will address the array size concern will still keeps the BF capability at the UE side from a system-level perspective.

Proposal 11: As an alternative option for the “FR2-like UE BF”, RAN4 to consider two panels, with each having 1x2 array.

# Conclusion3

In this paper we have shared our views on the RF parameters for 14800 – 15350 MHz Frequency range. Our observations and proposals can be summarized for that frequency range as follows:

Proposal 1: RAN4 to agree on TDD as a baseline duplexing for 14800 – 15350 MHz frequency range and capture the following text in TR 38.922.

“There is no defined 3GPP band for the 14800 - 15350 MHz frequency range. Similar to the 4400 – 4800 MHz and 7125 – 8400 MHz frequency ranges, SBFD can be a candidate duplexing method for this frequency range. The core requirements for Rel-19 SBFD work item can be tracked through the list of impacted specs captured in [6]. To provide a timely response to WP5D regarding the requested RF parameters, RAN4 assumed TDD as a baseline duplexing for the 14800 – 15350 MHz frequency range.”

Proposal 2: RAN4 to adopt 200MHz is a typical channel bandwidth. Higher channel bandwidths should not be precluded for this frequency range.

Proposal 3: As signal bandwidth depends on CHBW and SCS, no need to specify a fixed signal bandwidth but rather mention its dependency on SCS and number of RBs.

Proposal 4: RAN4 to consider at least 1k elements at the BS side for the 14800 – 15350 MHz Frequency range. Exact parameters to be decided after the finalization of the adjacent channel coexistence.

Observation 1: The impact of modelling near-by users using spherical waves on system level evaluations is questionable and currently being studied in RAN1.

Proposal 5: RAN4 to not consider near-field impacts in this frequency range in its correspondence to WP5D.

Observation 2: To derive the BS SEM, RAN4 could follow the same approach as TR 38.921 with the appropriate minimum channel bandwidth.

Proposal 6: For BS ACLR and ACS, RAN4 to decide on the UE ACLR and ACS based on the outcome of the adjacent channel coexistence study.

Proposal 7: RAN4 to agree on 26 dBm (i.e., PC3) as UE maximum output power as a baseline.

Proposal 8: RAN4 to agree on 59 dBm as UE power dynamic range.

Proposal 9: For UE ACLR and ACS, RAN4 to decide on the UE ACLR and ACS based on the outcome of the adjacent channel coexistence study.

Observation 3: UE beamforming assumption based on 0 dBi isotropic element with 0 dB Rx diversity gain paints a pessimistic view of the UE performance from a system level perspective, where in reality some UE gain is observed.

Observation 4: UE beamforming assumption based on 0 dBi isotropic element with 5 dB Rx diversity gain considers a simplistic 5dB addition in the minimum coupling loss for both scheduled and interfering links.

Proposal 10: As an alternative option for the “FR1-like UE BF”, RAN4 to consider UE with a single antenna element at the UE with random orientation in the azimuth domain, uniformly distributed between -90 and 90 degrees.

Proposal 11: As an alternative option for the “FR2-like UE BF”, RAN4 to consider two panels, with each having 1x2 array.

# Reference

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson, RAN #103, Maastricht, Netherlands.

[2] R4-2410741, “WF on IMT parameter study”, Ericsson, RAN4#111, Fukuoka, Japan.

[3]	TR 38.921, “Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz”, 3GPP V17.1.0.

[4] RP-240789, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon,   3GPP TSG RAN#103.