3GPP TSG-RAN4 Meeting #112	R4-2411522

Maastricht, Netherlands, 21st – 25th August, 2024

Title:	Views on co-existence parameters for 14800 - 15350 MHz

Source:	Qualcomm Incorporated

Agenda item:	8.2.4.1

Document for:	Approval

# 1.	Introduction

During RAN#103 plenary a new study item on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [1]. In RAN4#111, a WF was agreed in [2] where several agreements were captured for the 7125 – 8400 MHz and 14800 – 15350 MHz frequency range. In this contribution, we provide our preliminary results and views on the different system aspects, RF parameters for BS and UE items for the 14800 – 15350  MHz frequency range.

# 2 	Discussion

## 2.1	System parameters

### 2.2.1	Deployment scenarios

In [2], the following related to initial deployment scenarios was agreed:

Issue 3-1: Simulation scenarios Agreement: - 1st priority urban macro - 2nd priority indoor - 3rd priority dense urban

For 6GHz and 10 GHz coexistence work carried out in [3], indoor and urban (ISD= 450m) were considered when deriving the ACLR/ACS, where it can be observed that the Urban macro ACIR results were the more stringent between the two scenarios. In our view, this band is targeted for UMa-like coverage and should not be limited to indoor or micro dense deployments. Given that RAN4 is targeting large antenna array deployments at the BS side, it is viable to assert that the DL and UL SINR will be good enough for urban macro deployments. Additionally, since UMa ACIR requirements are generally more stringent compared to the indoor scenarios, it is viable for RAN4 to prioritize urban macro deployment scenarios and consider Indoor scenario after all assumptions/ parameters for the urban macro deployment are agreed.

Observation 1: Urban macro deployments for the 14800 – 15350 MHz frequency range is feasible based the assumptions of large antenna array at the BS side.

Proposal 1: RAN4 to prioritize urban macro deployment scenarios and consider Indoor scenario after all assumptions/ parameters for the urban macro deployment are agreed.

Regarding coordination of deployment, the following was agreed in [2]:

Issue 3-3 Initial coordinated operation - Option 1: - Co-ordinated for indoor - Both co-ordinated and un-co-ordinated for outdoor

In our views, indoor deployment is typically assumed to be co-ordinated in most of the RAN4 adjacent channel coexistence studies (as an example see TR 38.803 and TR 38.921), thus, we propose to consider only co-ordinated deployment for indoor. For outdoor, having un-coordinated deployment has been the common assumption as well since fully coordinated deployment between different operators requires additional complexity and coordination. Also given the timeline to finalize the parameters for this frequency range, only un-coordinated deployment should be prioritized.

Proposal 2: RAN4 to consider as a first priority uncoordinated urban macro deployments in the adjacent channel coexistence framework, similar to TR 38.921.

### 2.2.2	Channel bandwidth

In general, wider channel bandwidths should be enabled with increasing frequency following same approach as FR1 (3 to 100 MHz) to FR2 (50 to 400 MHz). This comes with increasing SCS and increasing RB width and availability of the spectrum. Channel bandwidth of 100 MHz has been considered as a representative channel bandwidth in TR 38.921 for 6GHz and 10GHz. Accordingly, it is proposed that higher channel bandwidth (e.g., 200 MHz) should be considered as a baseline for this frequency range.

Proposal 3: RAN4 to consider 200 MHz as a baseline in the adjacent channel coexistence framework.

## 2.2	BS parameters

For the BS AAS parameters, it is expected that large number of arrays to be deployed at the gNB to compensate for the incurred propagation losses at this frequency range. For the 4400-4800MHz, 192 elements were assumed, while discussions in the 7125 – 8400 MHz frequency range are in the ballpark of 384 antenna elements. To maintain the same physical area of the antenna array in the 14800 – 15350 MHz frequency range, the number of elements is expected to be larger than 1k elements. Accordingly, RAN4 will need to study the feasibility of having such number of antenna elements in that frequency range.

Proposal 4: RAN4 to consider at least 1k elements at the BS side and study feasibility of deploying larger number of antenna elements in the BS compared to FR1 for the 15 GHz range.

Another important AAS aspect is the underlying AAS pattern model. For 1710 – 4990 MHz frequency range, an extended version of the AAS array antenna model is in TR 38.803, subclause 5.2.3.2.4 to support vertical sub-array geometries with fixed sub-array down-tilt. For the 7125 – 8400 MHz frequency range, discussions are ongoing on the optimal set of AAS parameters with subarrays. It is important that AAS parameters with subarrays are selected to facilitate the coexistence with other incumbent (3GPP and non-3GPP) networks and services.

Observation 2: BS AAS parameters should be selected to facilitate the coexistence with other incumbent services.

## 2.4 	UE parameters

### 2.4.1		UE beamforming

In [2], the following was agreed:

- For simulations, consider both options. - FR1 like - RX diversity gain: [0 or 5] dB assuming 4RX - TX: 0dBi omnidirectional - Max Power: 23 dBm, 26dBm - FR2 like - Two panels (one in each direction) 2x2 antenna - 5dBi element gain. Array gain comes on top - Power: 23 dBm as max TRP

It is expected that further down-scoping of the above options would follow in RAN4 meetings prior to the November meeting. Another element that needs to be considered is the impact of UE beamforming on the derivation of the ACLR/ACS requirements for BS and UE. With UE beamforming in place, coexistence between adjacent networks is more favourable since the UE added directivity components leads to higher received signal power. However, this comes with the cost of design and complexity at the UE side. Since discussion on UE beamforming is still ongoing, we propose to investigate in the adjacent channel coexistence framework the two cases: UE with and without beamforming.

The “FR1 like” option agreed in [2] has two flavours, firstly within no Rx diversity gain, which follows that of TR 38.921 where an isotropic omnidirectional antenna with 0 dBi. The second “FR1 like” option considers a 5dBi gain resulting from 6 dBi gain assuming 4Rx and 1 dB losses. We have some concerns with those two options that are clarified below. For the 0 dBi isotropic assumption with 0 dBi Rx diversity, such a configuration paints a quite pessimistic line on the UE antenna performance, where, some UE gain is expected to be observed. Moreover, UEs rarely have a single antenna element in a band. To provide a realistic model for the “FR1-like” UE beamforming case, as a compromise between the two options captured in [2], it is viable to consider a notion of directionality at the UE as well as selection diversity between two single elements rather the receiver diversity option that was considered in RAN4#111. This can be realized by considering a single or multiple spaced antenna elements at the UE with random orientation in the azimuth domain, uniformly distributed between -90 and 90 degrees (i.e., similar to TR 38.803).

Proposal 5: As an alternative option for the “FR1-like UE BF”, RAN4 to consider UE with a single antenna element at the UE with random orientation in the azimuth domain, uniformly distributed between -90 and 90 degrees.

For the FR2 options, several companies have conveyed their concern during RAN4#111 on the feasibility of having 2x2 modules in the FR3 frequency range due to the antenna size. To address such concerns, we propose to revisit the “FR2-like” proposal to consider 1x2 array at each panel instead of 2x2 array. A 1x2 array can be designed to fit into the same outline as existing FR2-1 antenna modules. Such a compromise will address the array size concern will still keeps the BF capability at the UE side from a system-level perspective.

Proposal 6: As an alternative option for the “FR2-like UE BF”, RAN4 to consider two panels, with each having 1x2 array.

## 2.5 Preliminary results

In this contribution we present our preliminary results based on the set of parameters given in Table 1 below. In addition, we follow the simulation methodology described in Section 4 in [3], where the RF parameters (i.e., ACLR/ACS) are determined based on the degradation cause by adjacent channel interference (ACI).

Table 1 Adjacent channel coexistence for 14Ghz

| ## System Parameters | ## Parameter | ## Value |
| --- | --- | --- |
|  | ## Carrier frequency | ## 14 GHz |
|  | ## Deployment | ## Urban Macro |
|  | ## Channel BW | ## 200 MHz |
|  | ## Number of active UE per TTI | ## 1 |
|  | ## ISD | ## 450 meters |
|  | ## Grid shift | ## Un-coordinated scenario |
|  | ## UE to BS minimum distance | ## 35 meters |
| ## BS | ## BS Antenna Configuration ## (Mg, Ng, M, N, P) | ## (1,1,32, 32,2) |
|  | ## (dv, dh) | ## (0.5λ, 0.5λ) |
|  | ## Antenna element gain | ## 5.5 dBi |
|  | ## Maximum array gain | ## 36 dBi |
|  | ## Antenna element radiation pattern | ## ITU-R M.2101 AAS model |
|  | ## Max Tx conducted power (per polarization) | ## 43 dBm |
|  | ## EIRP | ## ~81.5 dBm |
|  | ## Noise figure | ## 8 dB |
| ## UE | ## UE antenna characteristics | Several UE beamforming settings considered (See Figure 2). |
|  | ## Antenna element gain | ## 5 dBi |
|  | ## Max conducted power | ## 23 and 26 dBm |
|  | ## Noise figure | ## 11 dB |
|  | ## Target uplink SNR | ## 15 dB |
|  | ## Uplink power control | ## Adjusted to meet UL SNR target |

### 2.5.1	Impact of UE Beamforming

In this section we investigate the impact of different UE beamforming setting on the experienced throughput loss for the cell-edge and mean user performance in UMa scenarios. Different UE beamforming settings are investigated in this contribution as shown in Figure 1, which are further explained below. Figures with markers represent the new proposed models that were not captured in RAN4#111 WF [2].

- FR1 (option a): This represents the FR1 case agreed in the WF with 0 dBi and no Rx diversity [2].

- FR2 (option a): This represents the FR2 case agreed in the WF [2].

- FR1 (option b): This represents a proposed compromise to the agreed FR1 like option in [2].

- FR2 (option b): This represents a proposed compromise to the agreed FR2 like option in [2].

Figure 2 Different UE beamforming settings considered (solid for mean, while dashed for cell edge).

Figure 3 shows the perceived UE antenna gain considering the four UE beamforming options shown in Figure 2. It can be observed the dominance of the FR2 option a since it has the higher BF gain while retaining equivalent spatial coverage. In addition, we can see that the two options listed in RAN4#111 WF [2] paint a rather pessimistic view of the FR1 performance due to the simplistic isotropic 0 dBi assumption that it encompasses. Accordingly, we believe that the two proposed models (i.e., blue and red) for FR1 and FR2 provide a better compromise between feasibility, performance, and complexity.

Proposal 7: RAN4 to consider FR1-like UE beamforming as two discrete antennas  while for FR2-like as two panels with 1x2 array at each.

In Figure 3 we plot the throughput loss performance as a function of the ACIR for both downlink and uplink scenarios for the different UE beamforming settings, shown in Figure 2. As observed, the performance of FR1 (option a) results in the worst performance due to its simplistic way to model the UE antenna gains experienced in both downlink and uplink. The delta compared to the variant option b is around 0.5 dB in the downlink and 3 dB in the uplink. Such performance enhancement is a result from the added directionality at the UE side. We also can observe the impact of the beamforming capability at the UE side with the variants of FR2 options. The performance gap between option a and option b is around 1 dB for both downlink and uplink, which does not justify having 2x2 arrays, when close performance can be realized with 1x2 arrays, but with lower antenna design complexity and cost.

Observation 3: There is an observed performance enhancement when considering directionality at the UE via single elements compared to the isotropic 0 dBi assumption.

Observation 4: TP loss performance is marginal between 1x2 and 2x2 array at the UE side.

Figure 3 TP loss as function of ACIR with solid (dashed) represent mean (cell edge) performance.

It is worth noting that the lower ACIR values are a byproduct of the enhanced performance resulting from large antenna arrays deployed at the BS. In comparison to Rel-17 work done on 10GHz, number of antennas at the BS side was assumed to be 16x8, whereas in [2], at least 1k element was considered, resulting in around 9 dBi difference in the directivity.

Observation 5: Large antenna arrays at the BS enables feasible adjacent channel coexistence via directing the wanted signal to the target destination and reducing the adjacent channel interference.

Focusing on the provided ACIR performance for the uplink scenario given in Figure 3, the worst-case performance provides around ACIR = 17 dB, which represents the combined effects of the UE ACLR and the gNB ACS. Additionally, UEs in the 14800 – 15350 MHz frequency range will anyway need to comply with the regulatory occupied bandwidth requirement, that specifies the maximum bandwidth within which emissions do not exceed a certain percentage of the total emissions. For NR, it is equal to the channel bandwidth with a maximum of 1% of emissions allowed outside of it (0.5% on each side), which result in around 24 dBc UE ACLR requirement.

Observation 6: Due to the regulatory occupied bandwidth requirement, required UE ACLR is technically equal to 24 dBc.

Considering the 17 dB ACIR observed from Figure 3, this implies that the gNB ACS will be around 18 dBc which motivates the case of considering higher power classes (i.e., higher maximum UE output power).

Observation 7: Exploring higher power classes is feasible for this frequency range given the observed ACIR values.

### 2.5.2	Impact of UE maximum output power

In [2], two options regarding the UE maximum output power were considered, namely 23 dBm (shown in black) and 26 dBm (shown in magenta). Figure 4 presents the results for the two power classes, where it can be observed that with higher power class, ACIR values are approximately 2 dB higher. With the low ACIR values observed, it is proposed to consider UE maximum

Observation 8: ACIR values for are observed to be approximately 2 dB higher when considering UE maximum output power = 26 dBm compared to 23 dBm.

Proposal 8: RAN4 to consider 26 dBm as the UE maximum output power in the 14800-15350 MHz frequency range adjacent coexistence study.

Figure 4 TP loss for Uplink scenario for black (magenta) 23 (26) dBm UE maximum output power.

# Conclusion

In this paper we have shared our views on the RF parameters for 14800 - 15350 MHz frequency range. Our observations and proposals can be summarized for that frequency range as follows:

Observation 1: Urban macro deployments for the 14800 – 15350 MHz frequency range is feasible based the assumptions of large antenna array at the BS side.

Proposal 1: RAN4 to prioritize urban macro deployment scenarios and consider Indoor scenario after all assumptions/ parameters for the urban macro deployment are agreed.

Proposal 2: RAN4 to consider as a first priority uncoordinated urban macro deployments in the adjacent channel coexistence framework, similar to TR 38.921.

Proposal 3: RAN4 to consider 200 MHz as a baseline in the adjacent channel coexistence framework.

Proposal 4: RAN4 to consider at least 1k elements at the BS side and study feasibility of deploying larger number of antenna elements in the BS compared to FR1 for the 15 GHz range.

Observation 2: BS AAS parameters should be selected to facilitate the coexistence with other incumbent services.

Proposal 5: As an alternative option for the “FR1-like UE BF”, RAN4 to consider UE with a single antenna element at the UE with random orientation in the azimuth domain, uniformly distributed between -90 and 90 degrees.

Proposal 6: As an alternative option for the “FR2-like UE BF”, RAN4 to consider two panels, with each having 1x2 array.

Proposal 7: RAN4 to consider FR1-like UE beamforming as two panels with single element at each while for FR2-like as two panels with 1x2 array at each.

Observation 3: There is an observed performance enhancement when considering directionality at the UE via single elements compared to the isotropic 0 dBi assumption.

Observation 4: TP loss performance is marginal between 1x2 and 2x2 array at the UE side.

Observation 5: Large antenna arrays at the BS enables feasible adjacent channel coexistence via directing the wanted signal to the target destination and reducing the adjacent channel interference.

Observation 6: Due to the regulatory occupied bandwidth requirement, required UE ACLR is technically equal to 24 dBc.

Observation 7: Exploring higher power classes is feasible for this frequency range given the observed ACIR values.

Observation 8: ACIR values for are observed to be approximately 2 dB higher when considering UE maximum output power = 26 dBm compared to 23 dBm.

Proposal 8: RAN4 to consider 26 dBm as the UE maximum output power in the 14800-15350 MHz frequency range adjacent coexistence study.

# Reference

[1]		RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson, RAN #103, Maastricht, Netherlands.

[2] R4-2410741, “WF on IMT parameter study”, Ericsson, RAN4#111, Fukuoka, Japan.

[3]	TR 38.921, “Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz”, 3GPP V17.1.0.