3GPP TSG-RAN WG4 Meeting #112	R4-2411525

Maastricht, Netherlands, Aug 19 – Aug 23, 2024

Source:	Nokia

Title:	Discussion on MIMO layer evaluation for 6Rx UE

Agenda item:	8.1.1.3.2

Document for:	Discussion

Introduction

During RAN#104 the work item on UE RF enhancements in Rel-19 was revised [2].

The performance part of this work item has not commenced yet, however discussions during RAN4#110-bis and RAN4#111 on the number of MIMO layers to support 6Rx has led to an Agenda Item to be introduced during RAN4#112 to specifically discuss MIMO layers for this Work Item with regards to 6Rx.

Specifically looking at the following aspects of the WID:

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this document we present views on the open issues which were identified as part of the Way Forward from RAN4#111 [1].

Discussion

General considerations for MIMO layer evaluation for 6Rx UE

Tightening BS EVM requirement

In RAN4#111 it was agreed to further discuss if BS EVM requirements need to be tightened if the 6-layer MIMO for 6Rx UEs are decided to be feasible [1]:

Issue 3-1-1: Tightening BS EVM requirement Way Forward: RAN4 to further discuss if the BS EVM requirement needs to be tightened when discussing the feasibility of 6-Layer MIMO for 6Rx UEs.

It may be required to tighten the BS EVM requirement, and thus increase the SNR achievable under test in the case where the final requirement are above 21dB for max throughput, a limit which is imparted by the current EVM constraints.

It is important to further note, that while typically requirements are not defined above 21dB for testability reasoning; this is due an EVM constraint that is not observed in either TE equipment nor the deployed equipment, see discussions in [7] whereby this EVM aspect was discussed in detail.

Ultimately, as the final requirements will not defined until the later RAN4 performance discussion, we would prefer to keep the issue open and further discuss during the demodulation performance discussions.

The need to possible tighten the BS EVM requirements depends on the SNR of the final requirement definition, hence should be postponed to RAN4 performance discussion.

Defer the decision of BS EVM requirement tightening to the later RAN4 performance discussion.

6-Layer Performance Evaluation Assumptions

In RAN4#111 the topic of 6-layer performance evaluation assumptions was discussed, with the following described in the Way Forward [1] :

Issue 3-1-2: 6-Layer Performance Evaluation Assumptions Way Forward: RAN4 to further discuss the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios.

The proposed 6-layer MIMO configuration would utilize two codewords, a feasible approach that should be acknowledged. For performance evaluation, it is crucial to assess this configuration in a rank-deficient channel. This would allow for the observation of distinct performance differences among the MIMO layers, mirroring the conditions encountered in realistic channels.

In order to fully assess the performance of 6Rx a spatial channel model must be used.

Current most advanced demodulation performance channel models

The demodulation session of RAN4 typically uses the TDL channel model with a provision for modelling antenna correlation (in uni- and X-pol) to represent fading channels with multi-element antenna correlation. The currently used model for antenna correlation is a “continuous squared exponential antenna correlation decay” model which has been used to extend the initially 2x2 correlation matrix (2007, [4]) to 4x4 (in 2008, [5]) and later 8x8 (in 2017,[6]). 	
For 4Tx, such a (unipolar) antenna correlation model (medium=0.3) looks like:

10.31320.32320.3.1....1.0.3..1

Thus, the “continuous squared exponential correlation decay” name is self-explanatory. Both the initial 2x2 matrices [4] and squared exponential decay [5] were chosen adhoc and do not match measurements for >2 antenna systems.	
This antenna correlation model is applied to all taps of the TDL model equally.

The used method of extending the initial (pragmatically) chosen 2x2 correlation matrices has several deficiencies, with two important ones for this WI being:

- The continuous squared exponential correlation decay function, nearly 100% correlates antenna pair (1 and 0.3152for 6 antennas) and is based on adhoc decision and on measurements of correlation across antenna elements. It models a LOS environment with very low diversity in receive direction (small angular spread).

- The symmetric property of the correlation matrix determines a broadside only receive/transmit direction, which is applied to every reflection, and an ULA-like antenna setup.

Therefore, the RAN4 TDL + antenna correlation model severely overestimates the rank deficiency, or rather ill-conditioning, of the channel matrix and this model shall not be used to evaluate the feasibility of full rank transmission.

Note: The X-pol extension of the TDL + antenna correlation model for 6Rx would apply the antenna correlation coefficient scaling only on a 3-antenna model (but twice), where the overestimation of rank deficiency is less pronounced. However, there are too many adhoc choices in this correlation model for it to be used in good conscience.

The RAN4 TDL + antenna correlation model severely overestimates the rank deficiency, or rather ill-conditioning, of the channel matrix and this model shall not be used to evaluate the feasibility of full rank transmission

Feasibility evaluation

Since the current demodulation antenna correlation model is not directly applicable for 6 Rx as there is no correlation matrix in TS 38.101-4, we propose to check feasibility (but not performance at this stage) by using the TR 38.827 CDL channel model, which models the antenna array as ULA X-pol with lambda/4 antenna distance.

Following this evaluation, we see the TPUT to reach 100%, i.e., full rank 6 Rx is practically feasible, as can be seen in Figure 1 below.

Figure 1: Relative TPUT vs. pre-EQ SNR, 8T6R rank 6 MCS17, SU-MMSE receiver, random precoding, TS 38.827 based CDLC UMa channel model with 3x1 ULA X-pol receiver antenna assumption (AE distance = lambda/4).

Since feasibility is given even for lambda/4, but the considered channel model is not yet commonly used, RAN4 shall come back to the final performance evaluation methodology in the performance part of this work item.

Demodulation of 6 Rx full rank transmission is feasible for handheld devices.

Way forward

Since the performance aspect of this work item has not yet commenced, it is advisable to defer this discussion until the performance evaluation phase begins. The output of the 'Spatial Channel Model' study item can then provide valuable insights into establishing appropriate performance requirements.

The performance part of this work item has not yet commenced

There is an ongoing study item on introduction of a spatial channel model for RAN4 performance requirements.

RAN4 shall discuss the performance requirements for 6Rx once the performance part commences.

Whether to use same requirement for handheld UE and FWA

The decision to use the same requirements for handheld UE and FWA in RAN4#111 is currently deferred to after the evaluation of 6 MIMO layer performance, in accordance with the Way Forward [1]:

Issue 3-1-3: Whether to use same requirement for handheld UE and FWA Way Forward: RAN4 to defer decision after evaluation of 6 MIMO layer performance for handheld and FWA devices.

Initial evaluation of 6 MIMO layer performance is not enough to make a decision if the same requirements can be used for both handheld UE and FWA, hence the decision should be deferred for now and fully handled in the performance part.

Reuse of handheld for FWA requirements cannot be decided on initial evaluation of 6 MIMO layer performance as results might change during the performance part in-depth analysis.

Defer decision to use same requirements for handheld UE and FWA to the performance part.

We further would note that requirements would not be defined per FWA or handheld UEs, but link level parameters could vary depending on the UE deployment used.

RAN4 demodulation performance requirements are not defined per UE deployments.

6-layer Support

6-layer Support as optional feature

In RAN4#111 it was discussed if 6-layer shall be supported as an optional feature, with the following described in the Way Forward [1] :

Issue 3-2-2: 6-layer Support as optional feature Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature.

As 6-layer support would require a band that already supports more than 4 layers, as follows for the “maxNumberMIMO-LayersPDSCH” in TS 38.822, as well as requiring signaling between the BS and UE to indicate support we fully believe that it should be an optional feature

6Rx will require a band that supports deployments with “maxNumberMIMO-LayersPDSCH” above 4.

6Rx will require signaling to indicate UE support.

RAN4 shall consider 6-layer support as optional feature.

Release independence of 6-layer Support

In RAN4#111, the following open issue regarding release independence was left open in the Way Forward [1].

Issue 3-2-3: Release independence of 6-layer Support Way Forward: Follow RAN4 Chair guidance and defer any release independence discussions until the requirements are stable.

Release independence refers to the ability of a specific feature or component to be implemented and deployed in a different release cycle than other parts of the system. While 8Rx requirements have been determined to be release independent, it is premature to assume the same would be true for 6Rx.

8Rx performance requirements are released independent, but it is premature to assume that it will be the same for 6Rx

A thorough analysis of the specific requirements and their impact on the overall system is necessary before determining release independence for 6Rx.

RAN4 shall defer the decision on release independence until the performance part of the work item commences.

Conclusion

The paper discusses the topic of 6 Layer MIMO evaluation in RAN4, and presents some of the Nokia views on this topic for discussion during RAN4#112.

The following Observations and Proposals were made:

Observation 1: The need to possible tighten the BS EVM requirements depends on the SNR of the final requirement definition, hence should be postponed to RAN4 performance discussion.

Proposal 1: Defer the decision of BS EVM requirement tightening to the later RAN4 performance discussion.

Observation 2: In order to fully assess the performance of 6Rx a spatial channel model must be used.

Observation 3: The RAN4 TDL + antenna correlation model severely overestimates the rank deficiency, or rather ill-conditioning, of the channel matrix and this model shall not be used to evaluate the feasibility of full rank transmission

Proposal 2: Demodulation of 6 Rx full rank transmission is feasible for handheld devices.

Observation 4: The performance part of this work item has not yet commenced

Observation 5: There is an ongoing study item on introduction of a spatial channel model for RAN4 performance requirements.

Proposal 3: RAN4 shall discuss the performance requirements for 6Rx once the performance part commences.

Observation 6: Reuse of handheld for FWA requirements cannot be decided on initial evaluation of 6 MIMO layer performance as results might change during the performance part in-depth analysis.

Proposal 4: Defer decision to use same requirements for handheld UE and FWA to the performance part.

Observation 7: RAN4 demodulation performance requirements are not defined per UE deployments.

Observation 8: 6Rx will require a band that supports deployments with “maxNumberMIMO-LayersPDSCH” above 4.

Observation 9: 6Rx will require signaling to indicate UE support.

Proposal 5: RAN4 shall consider 6-layer support as optional feature.

Observation 10: 8Rx performance requirements are released independent, but it is premature to assume that it will be the same for 6Rx

Proposal 6: RAN4 shall defer the decision on release independence until the performance part of the work item commences.

- References

- R4-2410751, “WF on requirements for 6Rx”, AT&T, RAN4#111, Fukuoka, Japan, June 2024

- RP-241656,  “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, RAN#104 Shanghai, China, June 2024

- TS 38.822 v17.1.0 “NR; User Equipment (UE) feature list”, June 2023

- R4-072223, “TP for 36.803 for MIMO correlation matrices for LTE”, Motorola, Ericsson, Nokia, NEC, NXP, InterDigitial, Agilent, RAN4#45, Jeju, Korea, November 2007

- R4-080620, “High Dimension E-UTRA MIMO Correlation matrixes”, Ericsson, RAN4#46-bis, Shenzhen, China, April 2008

- R4-1708533, “Discussion on SI for 8Rx”, Huawei, RAN4#84, Berlin, Germany, Aug 2017

- R4-2311072, “Discussion on PDSCH Demodulation Requirements for 8Rx”, Nokia, Nokia Shanghai Bell, RAN4#108, Aug 2023