3GPP TSG RAN WG4 Meeting #112	R4-2411530

Maastricht, Netherlands, 19th	 – 23rd August 2024

Source:		Rohde & Schwarz

Title:	(NR_newRAT-Perf) Discussion on PTRS configuration for FR2 PDSCH testing

Agenda Item:	4.6

Document for:	Approval

# Introduction

During the last RAN4 meeting an issue with respect to the PTRS configuration for FR2 PDSCH demod testing was discussed in [1], but no conclusion was reached. In this paper we share our view on the issue and propose a solution.

# Discussion

## Background

In [1] the PTRS configuration for FR2 PDSCH demod testing was discussed and a potential mismatch between the PTRS configuration in table 7.2-1 of TS 38.101-4 [2] and the related messaging in TS 38.508-1 [3] was highlighted.

As shown below table 7.2-1 in TS 38.101-4 [2] requires the following setting for the PTRS configuration:

In the section 5.4.2.0 in TS 38.508-1 the 2 parameters “frequencyDensity” and “timeDensity” are not set:

In combination with the requirements from TS 38.214 (see below) the configuration in TS 38.508-1 would lead to the fact that for certain MCS values no PTRS are configured.

## Proposed solution

In our understanding, the core requirements specified in TS 38.101-4 need to be followed in the test specification (here TS 38.508-1), so if a mismatch is observed, this requires a correction of TS 38.508-1.

Observation 1: If a mismatch between core and test specification is observed, the test specification should be adapted to follow the core requirements.

From performing further investigations into the Rel-15 agreements that lead to the definition of the requirements in TS38.101-4, it can be observed that the presence of PTRS for FR2 demod testing was intended according to [5], [6] and [7].

  - PTRS

    - PTRS is configured when introducing performance requirements in FR2

Observation 2: Table 7.2-1 from TS 38.101-4 follows the Rel-15 agreements for PTRS in FR2 demod testing.

Therefor we think that it is not necessary to make any adaptions to table 7.2-1 in TS 38.101-4 with respect to PTRS, but rather TS 38.508-1 needs to be adapted, so that the PTRS are always present. A corresponding CR [8] has been submitted to RAN5#104 to make the necessary changes to TS 38.508-1.

Proposal 1: RAN4 to conclude that TS 38.508-1 shall be adapted to resolve the mismatch.

As customary in RAN5, the CR has been marked as depending on the outcome of this discussion, so the CR could be agreed in RAN5 once the discussion in RAN4 has concluded. From our point of view no LS would be necessary.

# Summary

In this contribution, we discussed the issue of the PTRS configuration for FR2 demod requirements and propose the following.

Observation 1: If a mismatch between core and test specification is observed, the test specification should be adapted to follow the core requirements.

Observation 2: Table 7.2-1 from TS 38.101-4 follows the Rel-15 agreements for PTRS in FR2 demod testing.

Proposal 1: RAN4 to conclude that TS 38.508-1 shall be adapted to resolve the mismatch.

# References

- R4-2408987; “[NR_newRAT-Perf] Discussion on PTRS configuration for UE demodulation requirements”; Huawei, HiSilicon; RAN4#111 May 2024

- TS 38.101-4

- TS 38.508-1

- TS 38.214

- R4-2409392; “Way forward on NR UE PDSCH demodulation requirements”; NTT Docomo, Intel Corporation; RAN4 AH 1807 July 2018

- R4-2409393; “Simulation assumptions for NR PDSCH demodulation performance requirements”; Intel Corporation; RAN4 AH 1807 July 2018

- R4-2408987; “[NR_newRAT-Perf] Discussion on PTRS configuration for UE demodulation requirements”; Huawei, HiSilicon; RAN4#111 May 2014

- R5-244858; “ Correction to PTRS configuration for FR2 PDSCH testing”; Rohde & Schwarz; RAN5#104 August 2024