3GPP TSG RAN WG4 Meeting #112	R4-2411531

Maastricht, Netherlands, 19th	 – 23rd August 2024

Source:		Rohde & Schwarz

Title:	Discussion on STxMP measurement limitations

Agenda Item:		8.13.2

Document for:	Approval

# Introduction

In this paper we discuss a brief analysis of possible restrictions to signal level conditions when it comes to receiving the signals from a STxMP transmission on a single antenna based on the agreements in [1].

# Discussion

During last RAN4 meeting as part of the Way forward [1] the following was agreed:

Issue 1-1-1: Signal level condition for measuring/distinguishing EIRP per TCI

- Agreement:

  - RAN4 assume TE has the capability of distinguishing EIRP per TCI with the configuration of rank 2 PUSCH transmission in EIRP measurement of STxMP. under SDM scheme to further study the test methodology

  - Signal level conditions (e.g., SNR, power imbalance) and test procedure need to be further checked.

  - RAN4 to conclude the feasibility issue in next meeting.

To investigate that issue we emulated two separate UL signals with the help of a signal generator and measured the signal as it would happen during the TC for STxMP.

The power of one of the UL signals was then reduced in 0.5 dB steps to simulate different signal conditions at the receiving TE antenna, e.g. for cases where a different main/side lobes of the transmitted signal from the UE overlap.

Figure 1 shows EVM measurements for the 2 transmitted signal plotted versus the difference in power level of the two transmitted signals. The EVM measurement has been chosen as the metric, to detect potential synchronization issues in the measurement.

From the graph it can be seen that the EVM of the fixed level high power signal (blue) is constant, while the curve of the constantly lowered signal (green) increases the lower the power of the signal becomes. This is mainly driven by the fact that the reference level of the receiver in the test equipment needs to be set according to the high power signal, so not to overdrive and damage the TE. However, even up to a difference of 40 dB between the signals no synch errors or similar problems are observed.

Figure 1 EVM measurement of received signal 1 & 2

Observation 1: No synchronisation errors are observed for up to 40 dB difference in signal power.

Proposal 1: RAN4 shall conclude that the test of the STxMP is feasible up to a power difference of 40 dB.

One other issue that might come up during the testing is related to the fact that when the power difference becomes large, the SNR of the low power signal for the low power signal will become very small, leading to an increase in the measurement uncertainty. In this scenario however, the TE can also still perform the channel estimation based on the DMRS of the signal under test (as shown above no synch errors are observed), this way the power of the signal can be measured as well. However also in this case the MU needs further investigation, since the channel estimation will only be performed on the DMRS.

Observation 2: MU needs further investigation especially for in case of a large power imbalance.

Observation 3: Channel estimation of the TE based on the DMRS of the signal under test can be used to measure the power of the signal under test.

Since the exact value of the measurement uncertainty will also depend on the exact Test Case parameters defined by RAN5, this work should be left to them and can be performed once the work on the actual Test Case starts.

Proposal 2: Detailed MU analysis can be left to RAN5 to take into account the exact Test Case parameters.

# Proposals

In this contribution, we discussed the feasibility of testing the STxMP feature and the potential influence on measurement uncertainty.

Observation 1: No synchronisation errors are observed for up to 40 dB difference in signal power.

Proposal 1: RAN4 shall conclude that the test of the STxMP is feasible up to a power difference of 40 dB.

Observation 2: MU needs further investigation especially for in case of a large power imbalance.

Observation 3: Channel estimation of the TE based on the DMRS of the signal under test can be used to measure the power of the signal under test.

Proposal 2: Detailed MU analysis can be left to RAN5 to take into account the exact Test Case parameters.

# References

- R4-2409921, “Way Forward for [111][339] NR_FR2_OTA”, Qualcomm Incorporated, RAN4#111 May 2024