3GPP TSG RAN WG4 Meeting #112	R4-2411534

Maastricht, Netherlands, 19th	 – 23rd August 2024

Source:		Rohde & Schwarz

Title:	Discussion on test decoder for CSI compression

Agenda Item:	8.17.4

Document for:	Approval

# Introduction

In this contribution, we analyze the CNN-based model proposed for the Option 3 feasibility study for the CSI compression use case in the recent e-mail discussion. In particular, we investigate model performance (i.e., reconstruction accuracy) and computational complexity (i.e., number of floating-point operations required for decoder inference). We compare both KPIs with results obtained for an MLP-based model that has previously been proposed in RAN4. We conclude that the potential for model complexity reductions should be considered in the future. Additionally, model complexity is a KPI that should be considered during further investigations.

# Discussion

In the following, we investigate the trade-off between reconstruction accuracy and model complexity based on autoencoder architectures that were contributed to RAN4 in the past.

## Model Architecture

Our investigation considers both a CNN-based and an MLP-based autoencoder model. The following sections provide a brief introduction of the investigated models.

### CNN-based (Resnet) Model

Following the 3GPP RAN4#111 meeting, after two rounds of e-mail discussion the following CNN-based model was proposed for the Option 3 feasibility study:

| Figure 1 CNN-based (Resnet) Encoder | Figure 2 CNN-based (Resnet) Decoder |
| --- | --- |

The model used for our performance and complexity analysis differs from the originally proposed model in the following aspects:

- We use a fixed uniform scalar quantization (the quantizer and dequantizer do not contain trainable neurons)

- We use a sigmoid activation function after the last fully-connected (FC) layer of the encoder

### MLP-based Model

The investigated autoencoder model is based on [1]:

Figure 3 MLP-based Encoder and Decoder (Source: R4-2407334)

The model used for our performance and complexity analysis differs from the model described in [1] in the following aspects:

- We use a fixed uniform scalar quantization (the quantizer and dequantizer do not contain trainable neurons)

- We use a latent space dimension of 32 to make the model comparable to the CNN-based model. Each encoder output node is uniformly quantized with 2 bits

- We use a tanh activation function after the last fully-connected (FC) layer of the encoder

## Simulation parameters

Our evaluation is based on the following simulation parameters:

| Parameter | Value |
| --- | --- |
| Scenario | Urban Macro (UMa) |
| Inter-BS distance | 200 m |
| Channel model | According to TR 38.901 |
| Antenna Setup gNB | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna Setup UE | 4 RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ |
| BS Antenna Height | 25 m |
| UE Antenna Height | 1.5 m |
| UE Distribution | 80% indoor, 20% outdoor |
| UE Velocity | 0 km/h |
| Carrier Frequency | 2 GHz |
| Subcarrier Spacing | 15 kHz |
| Bandwidth | 10 MHz |
| No. of PRBs | 52 |
| No. of Subbands | 13 |
| Channel Estimation | Ideal |
| MIMO Layers | 1 |

Table 1 Simulation parameters

## Dataset Size

The training/validation/test data sets generated according to the simulation parameters listed in Section 2.2 contain the following number of samples:

- Training Set: 614,440 samples

- Validation Set: 81,920 samples

- Test Set: 81,920 samples

The same data sets are used to train and test both the CNN-based and MLP-based models.

## Training Parameters

The following table lists the training parameters:

| Optimizer | Adam |
| --- | --- |
| Learning Rate | CNN-based model: 1e-4 MLP-based model: 1e-3 |
| Learning Rate Adaptation | Reduce on plateau by factor 0.5 with patience of 50 epochs |
| Loss Function | SGCS |
| Batch Size | 512 |
| No. Epochs | 200 |

Table 2 Training Parameters

## Simulation Results

In the following sections, we present our simulation results (reconstruction accuracy, number of floating-point operations) for the CNN-based and MLP-based decoder models.

### CNN-based (Resnet) Model

The CNN-based test decoder model proposed for feasibility study of Option 3 in the recent e-mail discussion contains 6 Resnet blocks and 128 filters per convolutional layer. Due to the large model size, the decoder inference comes with very high computational complexity (i.e., high number of floating-point operations). In our analysis, we are investigating complexity reductions by decreasing the number of Resnet blocks and/or filters and evaluate the impact on reconstruction accuracy. Since an evaluation of the test decoder model is the primary interest of our investigation, we keep the encoder model fixed (i.e., 6 Resnet blocks, 32 filters per convolutional layer).

Our simulations reveal the following results:

| No. of conv filters in decoder | No. of Resnet blocks in decoder | SGCS | MFLOPs | Remarks |
| --- | --- | --- | --- | --- |
| 128 | 6 | 0.7650 | 1,475.4 | Original model proposed in e-mail discussion |
|  | 4 | 0.7643 | 984.3 |  |
|  | 2 | 0.7543 | 493.3 |  |
|  | 1 | 0.7391 | 247.8 |  |
| 64 | 6 | 0.7649 | 369.7 |  |
|  | 1 | 0.7381 | 62.6 |  |
| 32 | 6 | 0.7617 | 92.8 |  |
|  | 1 | 0.7353 | 16.0 |  |
| 16 | 6 | 0.7570 | 23.4 |  |
|  | 1 | 0.7389 | 4.2 |  |
| 8 | 6 | 0.7550 | 6.0 |  |
|  | 1 | 0.7267 | 1.2 |  |
| 4 | 6 | 0.7154 | 1.6 |  |
|  | 1 | 0.7165 | 0.4 |  |

Table 3 Simulation results for CNN-based (Resnet) model

Our evaluations show that the computational complexity of the CNN-based (Resnet) test decoder model proposed in the e-mail discussion following the RAN4#111 meeting appears to be significantly higher than originally reported. In contrast to the reported 740 MFLOPs, our investigations show that approx. 1,475 MFLOPs are required for a single model inference. Additionally, for the given scenario and latent space size, we observe that there is potential for complexity reductions that only moderately impact reconstruction accuracy.

Observation 1: The computational complexity of the CNN-based (Resnet) test decoder model selected in the e-mail discussion following the RAN4#111 meeting is higher than originally reported (1,475 MFLOPs instead of 740 MFLOPs).

Observation 2: For the given scenario and latent space size, the computational complexity of the CNN-based (Resnet) test decoder can be significantly decreased by reducing the number of convolutional filters and/or Resnet blocks while suffering only moderate performance degradation.

MLP-based Model

For the MLP-based model implementation described in Section 2.1.2, we observe the following reconstruction accuracy and computational complexity:

| SGCS | MFLOPs |
| --- | --- |
| 0.7589 | 0.9 |

Table 4 Simulation results for MLP-based model

Even though the computational complexity of the MLP-based model is significantly lower than the largest CNN-based (Reset) model that was investigated, we observe a similar construction accuracy for the given scenario and latent space size.

Observation 3: For the given scenario and latent space size, both the CNN-based (Resnet) and MLP-based autoencoder models yield similar performance. The computational complexity of the MLP-based test decoder model is significantly lower than the CNN-based (Resnet) model proposed in the recent e-mail discussion.

Based on our findings, we are contributing the following proposals:

Proposal 1: The computational complexity of the test decoder should be considered as an important KPI during the feasibility study and a potential future standardization.

Proposal 2: The potential for test decoder model complexity reductions should be considered during the feasibility study and a potential future standardization.

# Summary

In this contribution, we discussed the trade-off between reconstruction accuracy and computational complexity of autoencoder models that were previously contributed to RAN4.

Observation 1: The computational complexity of the CNN-based (Resnet) test decoder model selected in the e-mail discussion following the RAN4#111 meeting is higher than originally reported (1,475 MFLOPs instead of 740 MFLOPs).

Observation 2: For the given scenario and latent space size, the computational complexity of the CNN-based (Resnet) test decoder can be significantly decreased by reducing the number of convolutional filters and/or Resnet blocks while suffering only moderate performance degradation.

Observation 3: For the given scenario and latent space size, both the CNN-based (Resnet) and MLP-based autoencoder models yield similar performance. The computational complexity of the MLP-based test decoder model is significantly lower than the CNN-based (Resnet) model proposed in the recent e-mail discussion.

Proposal 1: The computational complexity of the test decoder should be considered as an important KPI during the feasibility study and a potential future standardization.

Proposal 2: The potential for test decoder model complexity reductions should be considered during the feasibility study and a potential future standardization.

# References

- R4-2407334