# 3GPP TSG-RAN WG4 Meeting #112			R4-2411535

# Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Sony

Title:	Further Views on MPR Reduction

Agenda item:	8.1.1.2.1

Document for:	Approval

# Background

In RAN4#111, the enhancement of the power domain via relaxed ACLR has been further discussed [1], and a WF [2] is agreed which mainly focuses on the approach that converts full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW.

In this contribution, we provide our further views on the MPR reduction via relaxed ACLR/SEM/spurious emission modification.

# Mechanism for MPR reduction

## Relaxation on Spurious emission

In RAN4#111, the following agreement has been made regarding the spurious emission:

- No relaxation of level of spurious emission requirements

  - FFS on whether to change the spurious emission boundary

As analyzed in [3], the spurious emission requirements defined in 3GPP originated from ITU-R to ensure co-existence while allowing for global circulation of terminals. From this perspective, allowing any relaxation on spurious emission requirements is risky, including both emission level and emission boundary.

In addition, the requirements for spurious emissions for UE co-existence that is defined in 38.101-1 in the per band is usually place more strict limitations than the general spurious emission when it comes to UE implementations, which is more likely to limit the UE transmitted power after the SEM and ACLR is lifted rather than the general spurious emission requirements. Therefore, it is unclear if there is any benefit to relaxing the spurious emission boundary or level.

Observation 1: The spurious emission requirement in 3GPP is to ensure co-existence when UE operates globally.

Observation 2: The benefit to changing the boundary of general spurious emissions is unclear, especially considering the requirements of spurious emissions for UE co-existence.

Proposal 1: Do not consider any change to the level and boundary of the spurious emission.

## Converting full RB allocation in UE CBW to “inner RB allocation”

During the RAN4#111 meeting, the scenario of converting full RB allocation in UE CBW to “inner RB allocation” with an extended UE BW has been focused on in the agreed WF [2]. The corresponding scenario is illustrated in Fig. 1. The MPR reduction can be achieved by applying the MPR values according to the RB allocation with respect to the extended UE CBW rather than the original UE CBW, e.g., the edge RB or outer RB can be transferred to inner RB allocation.

In addition, there are a few open issues related to this scenario that require further clarify:

- FFS feasibility of case where extended UE CBW edge exceeds the BS CBW edge, i.e. gap between edges of UE CBW and BS CBW < 1/2 UE CBW

- FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW

- FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB

- FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW

In this section, we provide our analysis of the issue listed above.

Figure. 1. The illustration of converting full RB allocation in UE CBW to “inner RB allocation” [2]

For the case that the extended UE CBW edge exceeds the BS CBW edge, i.e., gap between edges of UE CBW and BS CBW < 1/2 UE CBW. Considering the corresponding UE RF requirement may be adjusted based on the “extended UE CBW”, e.g., the boundary of ACLR and SEM, it may cause potential issues to allow the “extended UE CBW” to go beyond the BS CBW, where the out of band protection can not be guaranteed. Therefore, it is proposed that the extended UE CBW should always be within the BS CBW if some new signaling would be defined in this case, as illustrated in Fig. 2

Observation 3: Allowing extended UE CBW go beyond the BS CBW may also shift the boundary of ACLR and SEM, which will cause risk for out-of-band protection.

Proposal 2: The extended UE CBW should not exceed the BS CBW.

Based on this understanding, for certain scenarios, depending on the UE power class, modulation scheme, and the exact RB allocation, the UE MPR may still be improved from edge RB to the outer RB case, which is also illustrated in Fig. 2. However, it is worthy noting that the gain of such an operation will be limited especially for PC3, where only Pi/2 BPSK scheme may obtain a fractional of dB benefit.

Figure. 2. The illustration on the MPR reduction when the gap between edges of UE CBW and BS CBW < 1/2 UE CBW

Observation 4: It is possible to obtain some MPR reduction via transferring the edge RB allocation to the inner outer RB allocation, although the gain will be limited.

Regarding the rest of issues

- FFS whether IBE is used between edges of UE CBW and BS CBW or between edges of UE CBW and extended UE CBW

- FFS ACLR and SEM are applicable from the edge of extended UE CBW or edge of BS CBW, i.e. the start point of ΔfOOB

- FFS integral region and boundary of OOBE is based on extended UE CBW or UE CBW

Those are all related to which CBW shall be taken as a reference CBW to define the in-band and out-of-band requirements with reduced MPR. We believe all the in-band and out-of-band emission limits shall be scaled based on the extended UE CBW to avoid ambiguity or confusion.

Proposal 3: The corresponding in-band and out-of-band emission requirements shall be adjusted according to the extend UE BW.

With this understanding, the IBE requirements only need to apply within the extended UE CBW but not between the edges of extended UE CBW and BS CBW. In addition, the ACLR, SEM, and OOBE should also be shifted or scaled based on the extended UE CBW rather the original UE CBW or BS CBW.

Proposal 4: The IBE requirements only be applied within the extended UE CBW.

Proposal 5: ACLR and SEM are applicable from the edge of extended UE CBW.

Proposal 6: The integral region and boundary of OOBE is based on extended UE CBW.

From the UE perspective, indicate the extended UE channel BW can be totally transparent as long as if this extended UE CBW is one of the CBW that the UE can support. The network can always configure the UE with a larger UE channel BW but with limited RB allocations, and it will be equivalent to the case illustrated as the extended UE CBW. In this case, no additional simulation would be needed either, and the MPR reduction can be realized by applying the MPR according to the RB position in this extended UE channel BW rather than the original UE CBW. RAN4 can further discuss how to deal with the case that the extended channel BW is larger than the maximum UE channel BW.

One more issue related to this approach is which channel bandwidth should be taken as a reference BW here, where it is possible to adopt the BS CBW, cell CBW, and operator spectrum block bandwidth. RAN4 shall decide which channel BW will be used. The cell BW is indicated in SIB1 as carrierBandwidth. This bandwidth can correspond to the transmission bandwidth configuration of a channel bandwidth. On the other hand, the operator spectrum block size is not signaled (can be several cells), except when the UE is configured with CA, and then the carrierBandwidth of all cells (and their frequency location w.r.t. point A) are sent to the UE in dedicated signaling. Therefore, from a signaling perspective, it is most straightforward for the UE to adopt the cell bandwidth in SIB1 as reference channel BW to perform the MRP reduction.

Observation 5: UE can obtain the cell BW in SIB1 and used it as the extended UE channel BW.

Observation 6: From a signaling perspective, it is most straightforward for the UE to adopt the cell BW bandwidth as reference channel BW to perform the MRP reduction.

# Performance gain

In addition to the methodology study, the actual performance gain of supporting this reduced MPR also needs to be evaluated. Focus on the scenario that a narrower UE channel BW within wider BS bandwidth, allocating a device with narrow bandwidth, e.g., Redcap and eRedcap, towards the inner location within an operator's spectrum block may not be a common scenario in real life to avoid spectrum fragmentation from the network aspect. In addition, since such an operation scenario requires the operator spectrum block to be wide enough, it may not be feasible for most FDD bands, especially those at sub-1GHz.

Observation 7: From the network aspect, allocating a device with narrow bandwidth towards the inner location within an operator's spectrum block may not be a common scenario in real life since this may create spectrum fragmentation.

Observation 8: The frequency bands are usually small in FDD bands in sub 1 GHz, which makes it  not being feasible to move the UE away from the edge of the band.

Proposal 7: RAN4 shall examine if the proposed MPR reduction scheme can be enabled for all types of UEs, including TDD and FDD, as well as normal UE, Redcap, and eRedcap UEs.

Figure. 3. The illustration of UE spectrum emission requirements in NR

From the UE implementation aspect, all the out-of-band emission limits, including ACLR, SEM, spurious emission, and spurious emission for co-existence, need to be examined to determine the appropriate MPR reduction. As background information, the UE out-of-band emission limits are illustrated in Fig. 3. Depending on the UE RF front-end implementations, the required MPR to meet each out-of-band emission is different with different RB allocations and frequency bands due to the spurious emission limit for the co-existence.

Observation 9: Depending on the UE RF front implementations and UE bandwidth, the required MPR to meet each out-of-band emission is different with different RB allocations. In addition, it is also different at different frequency bands due to the spurious emission limit for the co-existence.

Proposal 8: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per band capability.

# Conclusion

In this contribution, we provide our views on the MPR reduction via modified out-of-band emission limits, where the following observations and proposals are given:

Observation 1: The spurious emission requirement in 3GPP is to ensure co-existence when UE operates globally.

Observation 2: The benefit to changing the boundary of general spurious emissions is unclear, especially considering the requirements of spurious emissions for UE co-existence.

Observation 3: Allowing extended UE CBW go beyond the BS CBW may also shift the boundary of ACLR and SEM, which will cause risk for out-of-band protection.

Observation 4: It is possible to obtain some MPR reduction via transferring the edge RB allocation to the inner outer RB allocation, although the gain will be limited.

Observation 5: UE can obtain the cell BW in SIB1 and used it as the extended UE channel BW.

Observation 6: From a signaling perspective, it is most straightforward for the UE to adopt the cell BW bandwidth as reference channel BW to perform the MRP reduction.

Observation 7: From the network aspect, allocating a device with narrow bandwidth towards the inner location within an operator's spectrum block may not be a common scenario in real life since this may create spectrum fragmentation.

Observation 8: The frequency bands are usually small in FDD bands in sub 1 GHz, which makes it  not being feasible to move the UE away from the edge of the band.

Observation 9: Depending on the UE RF front implementations and UE bandwidth, the required MPR to meet each out-of-band emission is different with different RB allocations. In addition, it is also different at different frequency bands due to the spurious emission limit for the co-existence.

Proposal 1: Do not consider any change to the level and boundary of the spurious emission.

Proposal 2: The extended UE CBW should not exceed the BS CBW.

Proposal 3: The corresponding in-band and out-of-band emission requirements shall be adjusted according to the extend UE BW.

Proposal 4: The IBE requirements only be applied within the extended UE CBW.

Proposal 5: ACLR and SEM are applicable from the edge of extended UE CBW.

Proposal 6: The integral region and boundary of OOBE is based on extended UE CBW.

Proposal 7: RAN4 shall examine if the proposed MPR reduction scheme can be enabled for all types of UEs, including TDD and FDD, as well as normal UE, Redcap, and eRedcap UEs.

Proposal 8: If any reduction of MPR would be specified in the end, it should be an optional feature for UE with per band capability.

# References

- R4-2408935	Topic summary for [111][124] NR_ENDC_RF_Ph4_part1	Moderator(Huawei)

- R4-2410589	WF on power domain enhancement		Huawei

- R4-2408794 	Discussion on power domain enhancements for single carrier	ZTE Corporation, Sanechips

- RP-233279	MediaTek Views on RAN4 Rel-19: RF and OTA	MediaTek Inc.