3GPP TSG-RAN WG4 Meeting #112		                 R4-2411536

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Sony

Title:	AIoT deployment scenario for D1T1

Agenda item:	8.20.2.1

Document for:	Approval

# Background

The deployment scenario for D1T1 was further further discussed in RAN4#111bis [1], where the following agreement was made [2]:

Issue 2-1-1: deployment scenarios for D1T1

Agreement in RAN4#110bis: Issue 2-1-1: deployment scenarios for D1T1 Option 1-1: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is only allowed outdoors. Option 1-2: Legacy NR gNB are outdoor macro gNB while AIoT reader/CW/devices are all indoors. Legacy NR UE is indoor accessing to outdoor NR marco gNB Option 2-1: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy gNB share same hardware Option 2-2: Legacy NR gNB are co-located with AIoT reader and CW. All of NR and AIoT BS/UE/Reader/Device/CW are indoors. AIoT reader /CW and Legacy NR gNB do not share same hardware. (less limitation on the power boosting)

Agreement in RAN4#111:

- Consider only adjacent RB/channel co-existence evaluation for in-band deployment scenario for NR and AIOT

- Encourage companies to provide the simulation results for option 1-1 and 1-2

  - FFS on co-site scenario (option 2-1 and 2-2)

In this contribution, we further discuss in the feasibility for option 2-1 and option 2-2

# Discussion

In scenario option 2-1 and 2-2, the NR gNB will be co-located with AIoT reader and CW, and it can be expected that AIoT devices have to face a significant interference level due to the closely located NR gNB and UEs compared to the non-collocated scenario (option 1-1 and 1-2). Suppose that the AIoT system is going to be deployed within the NR band and share the spectrum with NR systems (e.g., in-band deployment), due to the lack of frequency selectivity on RF-ED receiver. In that case, it will be very challenging for the R2D link to work, which means that device type 1 and 2a may not be able to work under such a condition.

Observation 1: The RF-ED receiver in devices types 1 and 2a has no frequency selectivity, making it very challenging to receive the R2D link if a strong NR signal appears on the adjacent spectrum.

On the other hand, in device 2b, especially if IF-ED architecture can be used, the BPF in the IF can improve the suppression of the interference signals on the adjacent spectrum, which may make it more feasible for the indoor co-site deployment scenario.

Observation 2: device 2b with IF-ED receiver in R2D link is more robust when there is a strong interference on the adjunct spectrum, and it is  more feasible for indoor collocated deployment scenarios.

Proposal 1: RAN4 can further study whether the indoor collocated scenario can be feasible for all device types.

# References

- R4-2408946   Topic summary for [111][135] FS_Ambient_IoT_solutions_part2       Moderator(Huawei)

- R4-2410597   WF on impacts of A-IoT on RF requirements	  Huawei