3GPP TSG-RAN WG4 Meeting #112		                 R4-2411537

Maastricht, Netherlands, 19th – 23rd August, 2024

Source:	Sony

Title:	Further considerations on the ambient IoT device implementation and RF aspect

Agenda item:	8.20.3.2

Document for:	Approval

# Background

The discussion on the RF requirement of ambient IoT (AIoT) devices was started in RAN4#111 [1], and some initial requirement framework was proposed to be a starting point for further discussion [2]. In addition, multiple implementation aspects, which include but are not limited to the feasibility of RF bandpass filter (RF BPF), ADC assumption, and frequency error, have also been brought up.

In this contribution, we further analyze selected implementation issues of AIoT devices and their impact on RF performance.

# Ambient IoT device implementation assumptions

Three types of Ambient IoT devices, device 1, device 2a, and device 2b, have been discussed so far (see below) where device type 1 and device type 2a operate with the backscattering mechanism while device type 2b generates the transmission actively by itself.

- Device 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device's UL transmission is backscattered on a carrier wave provided externally.

- Device 2a: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device's UL transmission is backscattered on a carrier wave provided externally.

- Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device. The device's UL transmission is generated internally by the device.

In addition, the following architectures for the AIoT devices, including one architecture for device 1, one architecture for device 2a, and three architectures for device 2b, have also been agreed to be studied:

- (b)

(c)

Figure 1: The architecture of device 1, device 2a and device 2b that are to be studied in RAN1 .

Generally speaking, the RF architecture of AIoT devices can be separated into three parts:

1) Energy harvesting and storage part: collect and store the energy used to power the reader-to-device (R2D) and device-to-reader (D2R) operations at the AIoT device.

2) reception part: To receive the interrogation signal from the reader (R2D).

3) transmission part: To transmit the signal to the reader (D2R) either via backscattering (devices 1 and 2a) or generated by the AIoT device itself (device 2b).

In the following sections, we further analyze selected implementation issues of AIoT devices and their impact on RF performance.

## Reception part (R2D)

For the reception part on the R2D link, the RF envelope detector (RF-ED) receiver will likely be used for devices type 1 and 2a due to the constrained power budget. In addition, the RF-ED, IF, and zero-IF architecture have been proposed for device 2b.The AIoT receiver types are similar to receivers that are being discussed for low-power wake-up signal LP-WUS. Therefore, it can be expected that the reception part on the R2D link in the AIoT device can be leveraged from the ED-based LP-WUS receiver.

Observation 1: From the RF perspective, the reception part of the R2D link in AIoT devices can be leveraged from the ED-based LP-WUS receiver.

### RF BPF

One particular issue observed during the LP-WUS SI in Rel-18 is that the RF-ED receiver lacks frequency selectivity [3]. Therefore, to suppress the interference from signals on adjacent spectrums, a high-Q RF BPF is usually required, where the bandwidth of BPF needs to be equal to the AIoT BW.

Observation 2: The RF-ED envelope detector lacks frequency selectivity.

However, if a multi-band operation needs to be supported in the R2D link, either no RF BPF at all or multiple RF BPFs that can cover all the frequency bands with all the possible RB allocations shall be used. Considering the complexity and power consumption limits for AIoT devices, the former choice (no RF BPF) may be more feasible, at least for device type 1.

In addition, even if there is only a single FDD band that the AIoT device needs to support, if the R2D link may happen both in the DL and UL spectrum due to the reader can be either BS or intermediate UE, it will also be preferred not to implement any RF BPFs.

Observation 3: If multiple bands or different RB allocations need to be supported in the R2D link, using RF BPF in AIoT devices may not be feasible due to the complexity and power consumption limits.

Observation 4: even under single band operation, if the R2D link may happen both in the DL and UL spectrum due to the reader can be either BS or intermediate UE, it will also be hard to implement any RF BPFs.

On the other hand, although the interference level would inevitably be increased after removing the RF BPF, some extent of interference mitigation can still be achieved by squaring the signal and passing through a BB LPF in the ED receiver. In addition, some guard RBs between R2D signal and the neighboring signals may be configured to reduce further the interference level. Eventually, considering the target coverage of AIoT (~ 50m) is significantly smaller than the normal NR system, this may allow the implementation of RF-ED without any RF BPF.

Observation 5: the performance of the R2D link may still be acceptable without any RF BPF, considering LP filter in the BB, the potential guard RB, and also the limited coverage target.

Proposal 1: Assuming no RF BPF filter for at least device type 1 when discussing the RF performance of AIoT devices type 1, FFS on device type 2b and 2a.

### BB LPF

For the BB LPF, a similar issue has also been studied for LP-WUS [3], where 3rd order and 5th order Butterworth filter were taken as baseline assumption. Same assumption was adopted in RAN1 LLS assumption for AIoT in [4]. In our view, considering the extreme constrained power consumption and complexity,  a possible starting point of BB LPF filter's could be the 3rd Butterworth filter.

Proposal 2: Considering the 3rd order Butterworth filter as a starting point for the BB LPF.

### ADC

Another critical aspect that affects R2D performance is the ADC. As we know, many ADC bits provide a higher dynamic range and SNR but also increase power consumption. Therefore, considering the extremely limited power budget for device type 1, a 1-bit ADC seems the only viable option. Meanwhile, more power consumption may be allocated to ADC for devices 2a and 2b, which allows 4-bit ADC to be considered. To our knowledge, this also aligns with RAN LLS assumption that agreed in RAN1#117 [4]

Proposal 3: Considering 1-bit ADC for device type 1 and 4-bit ADC for devices 2a and 2b.

### Summary of reception requirements for R2D

In Tab. I below, we summarize our views on the potential impact of different implementations on selected RF requirements:

Table I RF Requirement for AIoT device- RX part

| RX requirement | Reference sensitivity | Different sensitivity requirements may be considered for different AIoT device types due to the different assumptions on ADC, NF, insertion loss, etc. |
| --- | --- | --- |
|  | Maximum input power | The maximum input power is affected mostly by non-linear components, including LNA, the ADC, and the mixer. Therefore, this requirement should be specified by all types of AIoT devices. - However, different maximum input power levels can be specified for different types of devices. |
|  | ACS | The ACS and ASCS requirements shall be derived based on co-existence simulation, and guard band/guard RB may also be defined. - It should also be taken into account that no RF BPF would be used, at least for device type 1, so different sizes of guard band/RB may be considered for different device types. |
|  | ASCS |  |
|  | In-band blocking | It shall be based on the co-existence simulation while also considering the actual deployment scenario. |
|  | Out-of-band blocking |  |

## Transmission part (D2R)

### Output power

In the transmission direction (D2R), for device 2b, as it can generate the signal by itself, the device function can be more similar to legacy IoT devices than backscattering-type devices. Therefore, its maximum output power and the corresponding power class can also be specified similarly to legacy UEs, but the value is expected to be significantly lower than legacy IoT devices due to the power consumption limits.

However, due to the nature of energy harvesting, its transmission and reception duty cycle may be limited by the availability of the harvested energy. Moreover, the power constraint on the AIoT devices may further impact the choice of components, e.g., oscillator, amplifier, etc.; thus, a significantly lower performance/capability is expected.

Observation 6: For the D2R link, the AIoT device 2b might be similar to the UL of legacy IoT devices but with lower transmission power and reduced availability due to the energy harvesting and power consumption constraint.

Proposal 4: the maximum output power of device type 2b can be specified similarly to legacy UEs, and a new power class with a significantly lower power level than existing ones may need to be specified accordingly.

On the other hand, the backscattering type of devices, e.g., devices 1 and 2a, the transmission to the reader is via the reflection of a carrier wave (CW) from external nodes. Therefore, its transmission performance depends on both the design of AIoT devices and the property of the external CW and the emitter node.

Observation 7: For device types 1 and 2a, its transmission performance depends not only on the design of AIoT devices but is also highly impacted by the design of the CW and the corresponding transmission node.

Therefore, the maximum output power of device types 1 and 2a will be up to the device's reflection gain and the maximum input power level it can tolerate from the CW. Please note that this maximum input power level is different from the one in R2D, as this power level would be determined by the backscattering modulator and the reflection amplifier if applicable.

Proposal 5: The maximum output power and power class of device types 1 and 2a can be defined based on the reflection gain of the device and the maximum input power level it can tolerate from the CW.

For passive RFID tags, the type of maximum input power level that does not saturate the tag ranges from about -20 to -10 dBm. When it comes to the reflection amplifier, based on the literature review, a 10-20 dB amplification gain with a couple of hundred of uW power consumption can be considered as a starting point for the reflection performance. Therefore, defining the maximum output power and power class of device type 1 in the range of -20 to -10 dBm while about -10 dBm to 0 dBm for device 2a might be feasible. RAN4 shall further study what would be a reasonable assumption here.

Observation 8: The maximum out power of device type 1 could be in the range of -20 to -10 dBm, while about -10 dBm to 0 dBm for device 2a.

Proposal 6: RAN4 shall further study the maximum input power level of CW of device types 1 and 2a and the maximum gain if the reflection amplifier applies.

### Output power dynamic

As device type 2b, assuming the device is fully charged, it can be foreseen that it will work similarly to legacy IoT devices. Therefore, the corresponding output power dynamic requirements should be specified for type 2b.

Proposal 7: the output power dynamic requirements should also be specified for device type 2b.

However, it is unclear if all the output power dynamic requirements would be relevant for backscattering-type devices. The power dynamic would reflect the incoming CW signal, and the power dynamic mostly depends on the CW signal (assuming the reflection gain of the device is constant). Therefore, it is questionable whether the transmit time mask and power control requirement would be relevant for AIoT devices type 1 and type 2a.

Proposal 8: There might be no need to define power control requirements and the ON/OFF time masks for AIoT devices type 1 and type 2a.

However, it would still be critical to ensure the backscattering device does not emit any high-power level when there is no incoming CW signal, and thus, an off-power requirement would be needed. The same value as NR and LTE as -50 dBm can be re-used.

A minimum output power requirement would also be necessary to ensure the AIoT devices can operate in a controlled manner under a minimum incoming CW power level. -40 is the minimum output power level, which is the same as NR and LTE, and might also be re-used, and the corresponding CW power level for this minimum output power level can be further studied.

Proposal 9: Re-use the -50 dBm transmit off power and -40 dBm Minimum output power for all device types

Proposal 10: The transmit-off power of the backscattering devices should be defined as the emission level when no CW is illuminated on the devices. FFS on what the CW power level should be for minimum output power for devices type 1 and type 2a.

### Transmit signal quality and Output RF spectrum emissions

Transmit signal quality requirements and output RF spectrum emission requirements shall be applicable for all AIoT device types to ensure the nonlinearity components in the device can perform in an acceptable level, and the spectrm of CW signal will not grow beyond the limit after modulated and reflection. In particular, the ACLR requirements will be based on the outcome of the co-existence study.

Proposal 11: Transmit signal quality requirements and output RF spectrum emission requirements shall be specified for all AIoT device types.

In addition, when it comes to the frequency accuracy requirement for device type 2a, it is critical if a frequency shift function would be needed for the backscattering type of devices. A common approach for such a frequency shift can be carried out by using an oscillator to toggle the switch speed on the load impedance, as shown in Fig. 3(a). Generally speaking, there are three levels of frequency shift: 1) no explicit frequency shift where the backscattering signal is on the same frequency as the CW signal (note that any frequency shift caused by the coding scheme may still be included). 2) a small but explicit frequency shift where the backscattering signal is slightly shifted away from the CW signal (e.g., one AIoT channel away) so that the interference from the CW on the backscattering signal can be reduced. 3) a large explicit frequency shift where the backscattering signal can be shifted to another band than the CW signal, e.g., the CW can be transmitted in the DL bands where the backscattering signal is transmitted in the UL frequency bands. The three different frequency shift schemes are illustrated in Fig. 4, which are represented by backscattering signals 1, 2, and 3, respectively, in the plot.

Observation 9: A small frequency shift (a few MHz) can be used to separate the backscattering signal from the CW signal to improve the SINR on the reader, while a larger frequency shift (tens or hundreds MHz) can be used to separate the CW and the backscattering signals on different bands or the same FDD band but different UL/DL spectrum.

Generally speaking, an oscillator consumes more power at a higher frequency. For example, assuming a ring oscillator has a few uW when it oscillates at a few MHz but can go up to hundreds of uW or even mW if it needs to oscillate at hundred(s) of MHz (depending on the oscillator design, for example, the number of stages in a ring oscillator). Therefore, with the given power budget (≤ a few hundred µW peak power consumption), the feasibility of a larger frequency shift needs to be further studied. Based on the state-of-the-art works, generating a frequency shift for backscattering communication in the range of tens of MHz has been realized [5], while the feasibility of realizing a larger frequency shift remains to be further investigated.

Observation 10: The frequency translation range of the backscattering signal needs to be studied under the power consumption limitation.

(a)                                                                              (b)

Figure. 3. A possible method to perform (a) the frequency shift on backscattering signal and (b) reflection amplification.

Figure. 4. The possible frequency shift scheme for backscattering AIoT devices.

Maintaining good frequency accuracy when the frequency shift is larger will also be more challenging. Therefore, the frequency accuracy requirements may also differ depending on the necessity.

Proposal 12: RAN4 shall study the feasibility and necessity of frequency shift function for device type 2a and investigate its impact on frequency accuracy.

### Summary of transmission requirements for D2R

With the above mentioned analysis, our view regarding some selected transmission requirements for the D2R link is summarized in the tab. II:

Table II RF Requirement for AIoT device- TX part

| RF Requirement for AIoT device- TX part |  |  |  |
| --- | --- | --- | --- |
| Tx requirement | Transmit output power | Maximum output power | The maximum output power for the backscattering device can be specified based on the device's reflection gain and the maximum input power level it can tolerate from the CW. - We can consider -20 to -10 dBm dBm for device 1 while around -10 dbm to 0 dBm for device 2a as a starting point. For device type 2b, a similar approach as legacy UEs can be used to define its maximum output power. - A bit lower power level as device 2a (-10 dBm to -5 dBm dBm) might be a starting point, consider that the device 2b needs to generate the CW wave internally by itself, thus more limited output power if we assume the same power consumption budget would be applied on device 2a and 2b. |
|  | Output power dynamic | Transmit OFF power | For all device types, consider the same level as NR and LTE, e.g., -50 dBm. - For the backscattering type of the device, this power level can be defined as the emission level from the device when there is no incoming CW signal. |
|  |  | Transmit time mask | It may not be necessary for backscattering types of AIoT device (1 and 2a), but it is only specified for the CW node. Needs to be specified for device 2b. |
|  |  | Minimum output power | Consider the same level as NR and LTE for all AIoT device types, e.g., -40 dBm. - Further study should be conducted on the corresponding CW power level in this case. |
|  |  | Power control requirement | It may not be necessary for backscattering types of AIoT devices (1 and 2a) but only specified for CW nodes. Needs to be specified for device 2b. |
|  | Transmit signal quality | Frequency error | Transmit signal quality requirements shall be specified for all AIoT device types to ensure the device's nonlinearity component can operate properly. - The frequency shift function of device 2a needs to be studied, as it may affect the frequency accuracy of the performance. |
|  |  | EVM |  |
|  |  | In-band emissions |  |
|  |  | Carrier leakage |  |
|  | Output RF spectrum emissions | Occupied bandwidth | Output RF and spectrum emissions requirements shall be specified for all AIoT device types. - The ACLR level will depend on the co-existence. |
|  |  | SEM |  |
|  |  | ACLR |  |
|  |  | Spurious emissions |  |

## Testability

For AIoT devices, antenna connectors may not be feasible due to the purpose of integration in a thin and slim tag. In addition, it will also be very challenging to test the backscattering communication under a conductive test system. In theory, a circulator can be used where one port is connected to the CW signal input, and one port is used to measure the reflected signal. However, it is questioned if the isolation in a commercial circulator would be good enough for such a test. Therefore, it is proposed that the OTA test should be taken as the baseline for at least devices 1 and 2a.

In addition, the CW signals must be provided for Tx testing for devices 1 and 2a. How this CW signal should be provided in an OTA test system needs to be further investigated.

To be fully functional, the ambient IoT devices also require a dedicated energy source for harvesting to be provided during the test. Theoretically, different types of resources can be used for different devices, and the question of how the energy source can be provided during the OTA test chamber is a question.

Proposal 13: the OTA test should be taken as the baseline for at least ambient IoT devices 1 and 2a.

Proposal 14: RAN4 shall study how CW signal and energy source should be provided in the test system

# Conclusion

In this paper, we provide our further analysis of the AIoT device RF implementation and requirement concept, where the following observations and proposals have been given:

Observation 1: From the RF perspective, the reception part of the R2D link in AIoT devices can be leveraged from the ED-based LP-WUS receiver.

Observation 2: The RF-ED envelope detector lacks frequency selectivity.

Observation 3: If multiple bands or different RB allocations need to be supported in the R2D link, using RF BPF in AIoT devices may not be feasible due to the complexity and power consumption limits.

Observation 4: even under single band operation, if the R2D link may happen both in the DL and UL spectrum due to the reader can be either BS or intermediate UE, it will also be hard to implement any RF BPFs.

Observation 5: the performance of the R2D link may still be acceptable without any RF BPF, considering LP filter in the BB, the potential guard RB, and also the limited coverage target.

Observation 6: For the D2R link, the AIoT device 2b might be similar to the UL of legacy IoT devices but with lower transmission power and reduced availability due to the energy harvesting and power consumption constraint.

Observation 7: For device types 1 and 2a, its transmission performance depends not only on the design of AIoT devices but is also highly impacted by the design of the CW and the corresponding transmission node.

Observation 8: The maximum out power of device type 1 could be in the range of -20 to -10 dBm, while about -10 dBm to 0 dBm for device 2a.

Observation 9: A small frequency shift (a few MHz) can be used to separate the backscattering signal from the CW signal to improve the SINR on the reader, while a larger frequency shift (tens or hundreds MHz) can be used to separate the CW and the backscattering signals on different bands or the same FDD band but different UL/DL spectrum.

Observation 10: The frequency translation range of the backscattering signal needs to be studied under the power consumption limitation.

Proposal 1: Assuming no RF BPF filter for at least device type 1 when discussing the RF performance of AIoT devices type 1, FFS on device type 2b and 2a.

Proposal 2: Considering the 3rd order Butterworth filter as a starting point for the BB LPF.

Proposal 3: Considering 1-bit ADC for device type 1 and 4-bit ADC for devices 2a and 2b.

Proposal 4: the maximum output power of device type 2b can be specified similarly to legacy UEs, and a new power class with a significantly lower power level than existing ones may need to be specified accordingly.

Proposal 5: The maximum output power and power class of device types 1 and 2a can be defined based on the reflection gain of the device and the maximum input power level it can tolerate from the CW.

Proposal 6: RAN4 shall further study the maximum input power level of CW of device types 1 and 2a and the maximum gain if the reflection amplifier applies.

Proposal 7: the output power dynamic requirements should also be specified for device type 2b.

Proposal 8: There might be no need to define power control requirements and the ON/OFF time masks for AIoT devices type 1 and type 2a.

Proposal 9: Re-use the -50 dBm transmit off power and -40 dBm Minimum output power for all device types

Proposal 10: The transmit-off power of the backscattering devices should be defined as the emission level when no CW is illuminated on the devices. FFS on what the CW power level should be for minimum output power for devices type 1 and type 2a.

Proposal 11: Transmit signal quality requirements and output RF spectrum emission requirements shall be specified for all AIoT device types.

Proposal 12: RAN4 shall study the feasibility and necessity of frequency shift function for device type 2a and investigate its impact on frequency accuracy.

Proposal 13: the OTA test should be taken as the baseline for at least ambient IoT devices 1 and 2a.

Proposal 14: RAN4 shall study how CW signal and energy source should be provided in the test system

In addition, the impact on the RF requirements are summarized in the table below:

Table I RF Requirement for AIoT device- RX part

| RX requirement | Reference sensitivity | Different sensitivity requirements may be considered for different AIoT device types due to the different assumptions on ADC, NF, insertion loss, etc. |
| --- | --- | --- |
|  | Maximum input power | The maximum input power is affected mostly by non-linear components, including LNA, the ADC, and the mixer. Therefore, this requirement should be specified by all types of AIoT devices. - However, different maximum input power levels can be specified for different types of devices. |
|  | ACS | The ACS and ASCS requirements shall be derived based on co-existence simulation, and guard band/guard RB may also be defined. - It should also be taken into account that no RF BPF would be used, at least for device type 1, so different sizes of guard band/RB may be considered for different device types. |
|  | ASCS |  |
|  | In-band blocking | It shall be based on the co-existence simulation while also considering the actual deployment scenario. |

Table II RF Requirement for AIoT device- TX part

| RF Requirement for AIoT device- TX part |  |  |  |
| --- | --- | --- | --- |
| Tx requirement | Transmit output power | Maximum output power | The maximum output power for the backscattering device can be specified based on the device's reflection gain and the maximum input power level it can tolerate from the CW. - We can consider -20 to -10 dBm dBm for device 1 while around -10 dbm to 0 dBm for device 2a as a starting point. For device type 2b, a similar approach as legacy UEs can be used to define its maximum output power. - A bit lower power level as device 2a (-10 dBm to -5 dBm dBm) might be a starting point, consider that the device 2b needs to generate the CW wave internally by itself, thus more limited output power if we assume the same power consumption budget would be applied on device 2a and 2b. |
|  | Output power dynamic | Transmit OFF power | For all device types, consider the same level as NR and LTE, e.g., -50 dBm. - For the backscattering type of the device, this power level can be defined as the emission level from the device when there is no incoming CW signal. |
|  |  | Transmit time mask | It may not be necessary for backscattering types of AIoT device (1 and 2a), but it is only specified for the CW node. Needs to be specified for device 2b. |
|  |  | Minimum output power | Consider the same level as NR and LTE for all AIoT device types, e.g., -40 dBm. - Further study should be conducted on the corresponding CW power level in this case. |
|  |  | Power control requirement | It may not be necessary for backscattering types of AIoT devices (1 and 2a) but only specified for CW nodes. Needs to be specified for device 2b. |
|  | Transmit signal quality | Frequency error | Transmit signal quality requirements shall be specified for all AIoT device types to ensure the device's nonlinearity component can operate properly. - The frequency shift function of device 2a needs to be studied, as it may affect the frequency accuracy of the performance. |
|  |  | EVM |  |
|  |  | In-band emissions |  |
|  |  | Carrier leakage |  |
|  | Output RF spectrum emissions | Occupied bandwidth | Output RF and spectrum emissions requirements shall be specified for all AIoT device types. - The ACLR level will depend on the co-existence. |
|  |  | SEM |  |
|  |  | ACLR |  |
|  |  | Spurious emissions |  |

# References

- R4-2408946   Topic summary for [111][135] FS_Ambient_IoT_solutions_part2       Moderator(Huawei)

- R4-2410597   WF on impacts of A-IoT on RF requirements	  Huawei

- TR 38.869	     Study on low-power Wake-up Signal and Receiver for NR

- R1-2405772    Email discussion on Ambient IoT evaluation assumptions  Moderator (CMCC)

- P. -H. P. Wang, C. Zhang, H. Yang, M. Dunna, D. Bharadia and P. P. Mercier, "A Low-Power Backscatter Modulation System Communicating Across Tens of Meters With Standards-Compliant Wi-Fi Transceivers," in IEEE Journal of Solid-State Circuits, vol. 55, no. 11, pp. 2959-2969, Nov. 2020